SA WG2 Meeting #S2-151e(e-meeting)	S2-2203902

16 - 20 May 2022, Electronic Meeting	(revision of S2-220xxxx)

Source:	Ericsson

Title:	        Updates to the TR based on response SA4 LS

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This contribution provides updates to the TR based on the SA4 response LS in S4-220505.

# 1. Introduction

Based on the response LS in S4-220505 on LS Reply on QoS support with PDU Set granularity, following updates are proposed:

- Replacement of video slice definition by a reference to SA4 TR 26.926 Adding further clarification concerning handling of lost PDUs.

- Annex A1 – clarification of video frame type concepts.

- Annex A2 – clarification on periodicity&jitter and on frame importance.

# 2. Proposal

This paper proposes the following updates to TR 23.700-60 clause 6.

* Start of change *

**** First Change ****

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services, as used in TR 26.926). In some implementations all PDUs within the PDU Set are of same importance requirement at application layer. All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In other implementations, the application layer can use the data up to the first lost PDU in the PDU Set and apply error concealment to recover parts of the information unit.

Multi-modal Data: Multi-modal Data is defined to describe the input data from different kinds of devices/sensors or the output data to different kinds of destinations (e.g. one or more UEs) required for the same task or application. Multi-modal Data consists of more than one Single-modal Data, and there is strong dependency among each Single-modal Data. Single-modal Data can be seen as one type of data.

NOTE 1:	This definition was taken from TR 22.847 [6].

Tactile Internet: A network (or network of networks) for remotely accessing, perceiving, manipulating, or controlling real or virtual objects or processes in perceived real time by humans or machines.

NOTE 2:	This definition is based on IEEE SA P1918.1 [7].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

PSDB	PDU-Set Delay Budget

PSER	PDU-Set Error Rate

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

-	The architecture, framework and the QoS model as specified in TS 23.501 [2], TS 23.502 [3], and TS 23.503 [4] are regarded as the baseline for this study.

NOTE 1:	The study focuses on using NR as access technology. Specific enhancements to other access types are not required to be studied in this TR (although not prohibited).

-	The functional split in 5GS between UE, RAN and CN remains unchanged, i.e. packet classification of DL packets is performed in CN, and the packet classification of UL packets is performed in UE.

-	XRM services are assumed to use the IP PDU session types (however other PDU types are not excluded).

-	XRM services shall be able to coexist within a PLMN or SNPN with existing services simultaneously

-	XRM services can be between client-server (i.e. UE - application server) and/or peer-to-peer (i.e. between two UEs routed via the 5G CN).

-	Architecture enhancements should support XRM applications and its traffic characteristics. However, media codec mechanisms are not in the scope of this study. Interested readers may check TR 26.926 for related mechanisms.

-	XR and media application data may be encrypted by the client and/or server in some cases and unencrypted in other cases.

NOTE 2:	It is assumed that the some header information necessary for the identification of PDUs is not encrypted.

## 4.2	Architectural Requirements

The following architectural requirements are applicable to this study:

-	Solutions shall build on the 5G System architectural principles as in TS 23.501 [2], including:

-	flexibility and modularity for newly introduced functionalities.

-	existing methods for communicating the QoS profile (i.e. Standardized 5QI, Operator Pre-configured 5QI, Dynamically Signalled QoS Profile).

-	Any enhancements for this study shall not impact Emergency Services and other Priority Services (MPS, Mission Critical, etc) capabilities.

-	The existing specific 5GS services using existing QoS functions shall not require enhancement to co-exist with any QoS enhancement being specified for XRM services.

**** Next Change ****

Annex A:
Traffic characteristics of XR and media services

# A.1	Frame and GOP

A video consists of a sequence of consecutive video frames. Each video frame is a picture encoded/compressed using different codec mechanisms (e.g. H.264/H.265/H.266, AV1 or AVS) for efficient storage and transmission. During encoding, the video spatial and temporal redundancy can be removed using, e.g. inter-frame/intra-frame prediction, motion estimation/compensation. Typically, three major frame/picture types can be used during encoding, i.e. I-frame, P-frame and B-frame:

-	I-frame, as an intra-coded picture, is a complete picture and can be encoded and decoded independently, like a JPG image file.

-	P-frame, as a predicted picture, is not a complete frame and only contains the image changes compared to the previous frame. If the reference frame is lost, the P-frame cannot be decoded and displayed.

-	B-frame, as a bidirectional predicted picture, contains the changes between the previous and following reference frames. With more reference frames, the compression ratio can be higher. However, the B-frame can only be decoded when the previous and following reference frames are available.

A Group of Pictures (GOP) includes a collection of successive video frames. The first frame in a GOP is an I-frame. The following frames until the next GOP can be P-frames or B-frames.

The concept of I, P and B pictures and use of GOPs dates back to MPEG-2 video (first published year 2000) and broadcast distribution, and is not generally applicable to new codecs, in particular not for low-latency applications. In modern video codecs, complex prediction structures are used that account for application constraints, encoding complexity, latency, and dynamic decisions in the encoding. This may result in irregularities, for example based on sequence properties. In particular, for low-latency delivery with error resiliency, different flavors of encoding operations are in use such as Gradual Decoder Refresh (GDR), multi-hypothesis prediction, in-loop filters, or long-term reference including synchronized reference buffers between the encoder and decoder. For low-latency, as typically needed in XR, classical B pictures would not exist.

# A.2	Other traffic characteristics

1)	Periodic and jitter.

The set of packets of video frame are always periodic with variable frame size. For example, audio frame ususally have a periodicity of 20ms, while video stream can have different frame rate, e.g. 60/90/120 fps. Besides, pose/control packets are always periodic (e.g. 4ms) with fixed packet size. Due to the unpredictable jitter, the packet arrival time might not be exacly periodic, which leads to transmission delay and potentially packet loss. 5GS should take both periodicity and jitter into consideration for XR and media service.

Video is not necessarily always constant frame rate. It can be variable framerate, for example there are systems that grab a frame from the camera when the uplink buffer is cleared, a video frame is rendered based on a trigger, event-based cameras get more popular, etc. Frame periodicity may be an application configuration option, as would be a variable framerate configuration.

2)	Frame importance.

For XR and media service, different types of frame coexist and have different importance. For example, in Group of Picture (GOP), the first I-frame is the most important frame, other P-frames are encoded based on the I-frame. That is, the decoding of P-frames depends on the decoding of I-frame in the same GOP. Therefore, frame importance should be provided to enable 5GS to perform efficient packet dropping (when un-preventable), e.g. the non-transmitted part with low importance of a GOP will be discarded at the transmitter if the packets with high importance of the GOP have been lost or exceed the delay budget.

In some cases, there are no dependencies, in other cases dependencies exist. Referring to the examples, spatial and/or temporal prediction across different slices/frames in the case of video coding typically applies.  In motion-compensated predicted video decoding, some frames/slices refer to other frames (typically entire frames and not restricted to slices) based on the video encoding configuration but also based on dynamic operational decisions. As consequence, a PDU Set may “depend” on previously received PDU Sets. However, such dependencies do not necessarily result in discarding dependent information units, but the user experience may be degraded.

Due to its heavy-compression and spatial-temporal prediction, any packet losses in video generally result in degradation of the user-perceived quality of experience. Hence, video applications generally (i) benefit, (ii) are more efficient and (iii) can be simplified, if the network minimizes video packet losses. Nevertheless, a video decoder, in particular in a low-latency application, needs to include mechanisms to handle packet losses and delayed packets, such as frequent resynchronization and error concealment. In those cases, the operation of the receiver/network may vary. For example, the handling of dependent PDU Sets once a leading PDU Set is lost is not universally defined and depends on the operation of the application. However, typically, video applications prefer reducing the encoding bitrate in order to minimize congestion-related packet losses. If the application and the 5GS have agreed to a QoS flow establishment, then the network is obviously expected to support the delivery of PDUs according to the QoS requirements of the application.

3)	PDU set granularity.

XR applications impose requirements in terms of PDU sets (e.g. video/audio frame/tile, Application Data Units etc.), rather than in terms of single packets/PDUs. Packets of one PDU set need to be jointly processed for XR traffics. 5G system should be aware of the PDU set in order to improve transmission performance. Basically, the PDU set SN, the packet SN within the PDU set, the total packet number of the PDU set or the last packet indication should be provided to 5G system for each packet.

4)	Traffic correlation.

Some advanced XR or media services may include multiple traffic streams such as video, audio, haptic data, or sensor data etc., which are called multi-modality communication. Different traffic streams with different QoS parameters are always mapped to different QoS flows. Packets within the same traffic stream with different packet characteristics (e.g. packet importance, frame type, etc.) may also be classified and mapped into different QoS flows. These QoS flows belong to the same XR and media service have strong correlations, which should be coordinated for QoS requirement, packet processing and time synchronization, etc.

**** Last Change ****

* End of changes *