3GPP SA WG2 Meeting #151E	S2-2203903

16-20 May 2022, Electronic Meeting

Source:	Ericsson

Title:	KI#4, Sol #12: Update with Editor’s notes for aspects that are FFS

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This paper adds some Editor’s notes for the key issue “PDU Set integrated packet handling” for inclusion in TR 23.700-60.

# Discussion

The FS_XRM SID has a KI “PDU Set integrated packet handling” as agreed in S2-2201802. As per the KI description,

This key issue proposes to study PDU Set integrated packet handling in 5G network, in which the group of packets belongs to a same PDU Set will be handled in an integrated manner.

Based on the related solution #12, the following changes are proposed for TR 23.700-60.

# Proposal

It is proposed to add the following changes to TR 23.700-60.

*** First change ***

## 6.12	Solution #12: PDU Set integrated QoS handling

### 6.12.1	Key Issue mapping

This solution aims to the resolve the Key Issue #4, "PDU Set integrated packet handling".

### 6.12.2	Description

For interactive media services, e.g. Cloud XR, cloud gaming, real-time video based remote control, each frame/video slice is delivered via multiple PDUs in the 5GS. For example, an I-Frame of 4K video can be more than 1MB which means around 1000 IP packets are needed to deliver it. Considering the frame/video slice can only be decoded in case all packets (or most of the packets, if FEC is used during encoding) are successfully received. 5G system should be aware of the PDU set and handle packets of one PDU Set in an integrated manner. When one or some PDUs fail to be transmitted, the whole PDU Set can be dropped.

In this solution, the following aspects are included:

1.	Optionally, AF provides PCF with the PDU Set level QoS requirements and the Flow description for the target media service data flow.

2.	PCF may generate the PCC rule with PDU Set level QoS parameters and also the detection rules for the PDU Set.

3.	PCF sends the PCC rule to SMF. The SMF distributes these QoS parameters/policy to RAN/UPF and instructs the UPF to detect PDU Set during the PDU Session Establishment/Modification procedures.

4.	Once the application service started, the UPF identify the PDUs of each PDU Set and marks PDU Set info in the GTU-U header of DL packets to RAN. The details of how to identify the PDUs within a PDU Set is in clause 6.12.3.2.

5.	RAN identifies the PDUs of a PDU Set based on the PDU Set info in the GTP-U headers. RAN performs the PDU Set integrated packet processing to deliver the PDU Set to UE.

Editor's note: The kind of PDU Set detection rules that PCF generates is FFS. The relationship between these rules and the detection mechanisms in 6.12.3.2 (except for periodicity) needs to be clarified.

Editor's note: How PCF communicates PDU Set detection rules to the SMF is FFS.

Editor's note: The suitable source of PDU Set detection rules (AF, PCF or UPF) is FFS.

Editor's note: The identification and handling of multiple PDU Sets that depend on each other is FFS.

*** Second change ***

##### 6.12.3.2.1	Identification of PDU Set as a video frame.

If the PDU Set represents a video frame, the identification of the video frame can be realized via following options

Option#1 Identification based on RTP header

The format of RTP header is defined in RFC 3550 [9] as shown below:

| 0 |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 |
| V=2 |  | P | X | CC |  |  |  | M | PT |  |  |  |  |  |  | sequence number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| timestamp |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| synchronization source (SSRC) identifier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| contributing source (CSRC) identifier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| …. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 6.12.3.2-1: RTP header format defined in RFC 3550 [9]

In RTP headers, Marker (M) bit is set for the very last packet of the frame indicated by the RTP timestamp. Therefore, UPF can be based on the M bit to identify the start and end of a PDU Set/frame.

Option#2 Identification based on the RTP header extension

Based on the draft-ietf-avtext-framemarking [11], the extended RTP header format is shown as below. The "S" bit and "E" bit in the Frame Marking RTP header extension respectively represent the start and the end of a video frame. UPF can identify the start and end of a PDU Set/frame according to the "S" and "E" bits.

| 0 |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 |  | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 |
| V=2 |  | P | X | CC |  |  |  | M | PT |  |  |  |  |  |  |  | sequence number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| timestamp |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| synchronization source (SSRC) identifier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| contributing source (CSRC) identifier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| …. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| defined by profile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | length of header extension |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  |  | L |  |  |  | S | E | I | D | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 6.12.3.2-2: RTP header extension format

NOTE 1:	The identification of a video frame based on the "S" and "E" bits in the RTP header extension also applies for scalable video streams.

Editor's note: whether the usage of an RTP header extension is mandatory to ensure interoperability between UPF and multiple AFs is FFS.

Option#3 Identification based on the periodicity via UPF.

Alternatively, the UPF can also use the periodicity information as frame traffic pattern to identify the PDU Set/frame. The traffic pattern may also be detected by UPF. For example, the DL media traffic may be 60 PDU Sets per second, i.e. 60 FPS. Then the UPF can determine that it receives a PDU Set every 16.67 ms.

NOTE 2:	This assumes the service data flow only contains the video stream, or if both video and audio streams are included in the same service data flow, the UPF can use other ways to separate the video stream, e.g. PT field in the RTP header.

*** End of changes ***