SA WG2 Meeting #S2-151E	          S2-2203947

16 - 20 May, 2022, Electronic meeting

Source:	CableLabs, Charter

Title:	Access SNPN via 3GPP AN using same credentials and credential holder used to access N3GPP AN

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: Proposes a solution for access to SNPN via 3GPP AN using same credentials and credential holder used for access to N3GPP AN.

# 1	Background

Proposes a solution for inclusion in TR 23.700-08.

* * * * Start of Change * * * *

## 6.X	Solution X: Access SNPN via 3GPP AN using same credentials and credential holder used to access N3GPP AN

### 6.X.1	Introduction

This solution addresses KI#2. The solutions offer an extension to Solution#16 adding wireline access and N3GPP devices.

The solution defines how the same credentials from a credential holder external to the SNPN can be leveraged for devices accessing SNPN via NG-RAN and 5GC and non-3GPP access network (without connection to 5GC).

Many enterprise networks have existing deployments with non-3GPP network infrastructure (WLAN or Wireline access) using the AAA server to authenticate the end-devices. The addition of SNPN deployments (access via NG-RAN and 5GC) could leverage the already provisioned identities and credentials to authenticate devices accessing to non-3GPP access networks for SNPN access via 3GPP access network using the same credential holder (AAA server) external to the SNPN.

### 6.X.2	Functional Description

-    The same AAA Server that is used for authenticating the 3GPP device connecting to the non-3GPP network (e.g., WLAN or Wireline access network) is also used for primary authentication for accessing SNPN via NG-RAN and 5GC, re-uses the architecture defined for "Credentials Holder for primary authentication and authorization" as defined in clause 5.30.2.9.2 of TS 23.501 [1]. A N3GPP device accessing the SNPN via non-3GPP access network (e.g., WLAN or Wireline access network) can also leverage the same external credential holder for authentication.

-	The 3GPP device uses the same permanent identity and credentials for primary authentication in SNPN that are used for access to the non-3GPP network (e.g., WLAN or Wireline access network). The existing identity and credentials used for WLAN and Wireline access authentication can be re-used for SNPN using a SUPI in NAI format as defined in clause 28.7.2 of TS 23.003 [2].

-    The SNPN and the non-3GPP network (e.g., WLAN or Wireline access network) provides access to the same Data Network e.g., internet or enterprise network.

-    Seamless mobility between the SNPN and the non-3GPP network (e.g., WLAN or Wireline access network) is not supported by this architecture.

Figure 6.X.2.1: Access to SNPN via NG-RAN and 5GC and non-3GPP access network (without connection to 5GC) using the same credentials

### 6.X.3	Procedures

0)	If the SNPN supports the architecture as in Figure 6.X.2-1 and wants the 3GPP device to use the same credentials for access to the non-3GPP network (e.g., WLAN or Wireline access network) and to SNPN, then the 3GPP device is configured with the same permanent identity and credentials for primary authentication with both the SNPN and the non-3GPP network (e.g., WLAN or Wireline access network).

1) A connection is established between a 3GPP device and the non-3GPP network (e.g., WLAN or Wireline access network) that uses the external AAA server for primary authentication (outside of 3GPP scope).

2) A connection is established between a 3GPP device and the SNPN network via NG-RAN and 5GC as defined in clause 5.30.2 of TS 23.501 [1] which also uses the same external AAA server for primary authentication re-using the architecture defined for "Credentials Holder for primary authentication and authorization" as defined in clause 5.30.2.9.2 of TS 23.501 [1].

The 3GPP capable device may establish connection towards either or both the non-3GPP network (e.g., WLAN or Wireline access network) and SNPN via NG-RAN and 5GC simultaneously.

### 6.X.4	Impacts on services, entities, and interfaces

UE:

-	Support the use of the same permanent identity and credentials for primary authentication in SNPN via NG-RAN and 5GC, and WLAN/Wireline access network (without connection to 5GC).

No other impacts in the UE, NG-RAN, and 5GC are identified.

* * * * End of Change * * * *