SA WG2 Meeting #S2-151E	S2-2203967

16 -  20 May, 2022, E-Meeting

Source: 	Futurewei

Title: 	KI#4, New Solution: Enable UE to query localized services information from the hosting network for service discovery

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_ph2 / Rel-18

Abstract of the contribution: This paper proposes a solution to address one aspect in the Key Issue #4 regarding providing localized service information to UE for localized serviced discover.

# Discussion

# Proposal

Add the following solution to TR 23.700-08.

*** START CHANGES **

## 6.x	Solution #x: Enable UE to query localized services information from the hosting network for service discovery

### 6.x.1	Introduction

As illustrated in the Key #4, UE need to be able to discover the hosting network and the localized service being hosted by this network. One aspect of KI #4 is to identify the mechanism to provisioning UE with appropriate localized service information. Considering we need to support the cases which the UE may or may not have prior subscription or information about the hosting network and the localized service, this solution is to propose an information query/response mechanism by enhancing the registration procedure to allow UE to query the localized service information from the hosting network which UE may or may have subscription with.

### 6.x.2	Functional Description

This solution proposes to allow UE to use registration procedure to query the localized service information from the AMF of the hosting network. The local hosting network may configure a designated AMF to handle the UE access for localized service and store the localized service information. The localized service information stored in the designated AMF can be configured locally or provisioned by the localized service provider’s AF via NEF. A new localized service intention indicator is carried in the registration request message sent by UE to the AMF, to indicates UE’s intention of using the registration for the query of localized service information. The gNB of the hosting network can use this indicator to select the designated AMF and forward the registration to this AMF, if there is a designated AMF for localized services.  Upon receiving this indication, the designated AMF can provide UE with the localized service information during or after the registration procedure. The UE may not have the right subscription which allows UE to access the hosting network before UE queries the localized service information, therefore, UE may only use the registration procedure to query the localized service information instead of registering to the network. For the latter case, network will send registration reject to the UE but with cause value of localized information query response along with the localized service information.

There can be 3 different registration intention indicators which can be carried in the registration request:

  - Local service information Query only: This indicator indicates the UE only uses this registration request to query localized service information without actual camping and registering to the network. After the hosting network gNB receives the registration request message with this indicator, the gNB forwards the request message to the AMF which can provide the information, such as the designated AMF. The AMF skips the normal registration procedure and just responses UE with registration reject but with cause value of localized information query response along with the localized service information.

  - Registration with local service information query: This indicator indicates that the UE would like to collect local service information during service registration phase. After the hosting network gNB receives the registration request message with this indicator, the network forwards the request message to the AMF which can provide information, such as the designated AMF. The AMF can provide the localized service information by either adding the localized service information in the registration accept message or sending separate NAS message, such as UCU.

  - Registration only: This indicator indicates that the UE only wants to conduct normal service registration without collecting the localized service information. When the local service intention indicator is absent in the registration request message, it’s default to consider the intention indicator is registration only, in order to support backword compatibility or the network which doesn’t support localized services.

In order to enable UE to use registration message to query localized service information, the hosting network needs to broadcast its capability of supporting the localized service access. UE only send register request with localized service query to the hosting network which supports localized service access.

### 6.x.3	Procedure

#### 6.x.3.1 UE query local service information without registering with the hosting network

Step 1: RAN of hosting Network C broadcasts SIB message which includes indication for supporting localized service.

Step 2:  UE1 of Network A is interested in the localized service provided by local network C. Although UE1 may not have a subscription which allow it to access the network C, UE1 may still like to know local services information before making a decision to register to network C to access the local services which the User is interested. UE1 sends registration request message with an indicator “Local service information Query only” for querying local service information.

Step 3: After receiving this registeration request message with “local service information query only” indicator, RAN selects the designated AMF2 per the local configuration.

Step 4: RAN forwards the registration request message to AMF2.

Step 5: AMF2 sends a registration reject to UE1 with cause code of local service information response as well as the addition local service information as queried by the UE.

#### 6.x.3.2 UE local service discovery while registering with the hosting network

Step 1: RAN of hosting Network C broadcasts SIB message which includes indication for supporting localized service.

- Option a: if UE likes to receive local service information.

Step 2a:  UE1 of network A may have the subscription to allow it to access network C and decides to regiser to the Network C. UE1 sends a registration request message with an local service intention indicator “Registration with local service information query”.

Step 3a: After receiving this registeration request message with “Registration with local service information query” indicator, RAN selects the designated AMF2 per the local configuration.

Step 4a: RAN forwards the registration request message to AMF2.

Step 5a: AMF2 sends Registration Accept to UE1 with additional local service information as queried by the UE.

Or

Step 6a: After AMF2 sends reqistration accept to UE1, AMF2 may send additional NAS message to UE, such as UCU which contains the local service information as queried by UE.

- Option b:  UE already has the local service information, or UE is not interested to collect local service information at this monent

Step 2b: UE1 sends registration request without any local service intention indicator or with “ Registration only” .

Step 3b: RAN forwards the registration request to AMF1 which is not designated for handling local services.

Step 4b: AMF1 sends registration accept to UE1.

.

### 6.x.4	Impacts on services, entities and interfaces

Editor's note:	Further impacts are FFS.

*** END CHANGES ***