3GPP TSG-SA2 Meeting #151-e	S2-2203969

# Online, , 16th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0118 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to PCC aspects related to 5MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Incoming LS S2-2203646 ask for clarifications and corrections: - The parameters of the Nnef_MBSSession_Create service operation and the Npcf_MBSPolicyAuthorization_Create service operation are unclear - Although no Policy Control Request Trigger and Npcf_MBSPolicyControl_Update service operations are defined, some related text remains. - Policy information stored in the UDR is not fully defined - The status that is notified in step 20 in clause 7.1.1.3 is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The parameters of the Nnef_MBSSession_Create service operation and the Npcf_MBSPolicyAuthorization_Create service operation are clarified: Service requirements provided by the AF to the NEF include the service description provided as part of the policy Authorization service. It is possible to receive several media components, also with the same media type. The emergency indicator and Application service provider are removed since in 29.514 they relate to the "communication of 5GC IMS NF service consumer via Service Based Interfaces" and sponsored connectivity, respectively, which are not applicable for 5MBS. Similar to TS 29.514, there is the possibility to have an MBS session level "AF application identifier" and in addition media component level application identifier. - The remaining text related to Policy Control Request Trigger and Npcf_MBSPolicyControl_Update service operations is removed. - Policy information stored in the UDR is defined - step 20 is used to report that the MBS Policy control association has been created. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear issues related to PCC prevent stage 3 progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10, 7.1.1.2, 7.1.1.3, 9.1.3.1, 9.2.2.1, 9.2.3.2, 9.2.3.3, 9.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 6.10	Policy control for Multicast and Broadcast services

### 6.10.1	General

The policy and charging control framework as defined in TS 23.503 [7] applies to Multicast and Broadcast services in the following aspects:

-	MBS Session binding: MBS Session binding is the association of an AF Session information to one and only one MBS Session. The PCF shall perform the session binding based on the MBS Session ID, i.e. TMGI or source specific IP multicast address.

-	QoS Flow binding: For an MBS Session, QoS Flow binding is the association of a PCC rule to a QoS Flow within an MBS Session. The MB-SMF performs QoS Flow binding for an MBS Session in the same way as the SMF for a PDU Session.

-	PCC rules for MBS Session are used to provide policy for QoS flows: The following PCC rule parameters defined in Table 6.3.1 of TS 23.503 [7] are applicable for MBS:

-	Rule identifier.

-	Service data flow detection: Precedence, Service data flow template (only for IP PDU traffic).

-	Policy Control: 5G QoS Identifier (5QI), DL-maximum bitrate, DL-guaranteed bitrate, ARP, Priority Level, Averaging Window, Maximum Data Burst Volume.

-	Policy information can also be applicable for an entire MBS session. The following parameters defined for a PDU session in Table 6.4.1 of TS 23.503 [7] are applicable for an entire MBS session:

-	Authorized Session-AMBR.

-	Explicitly signalled QoS Characteristics.

### 6.10.2.	MBS Session policy control data in UDR

The policy control profile information provided by the UDR at MBS Session establishment, using Nudr service for Data Set "Policy Data" and Data Subset "MBS Session policy control data" with MBS session ID of Application ID as data key is described in Table 6.4.1-1.

Table 6.10.2-1: MBS Session policy control subscription information

| Information name | Description | Category |
| --- | --- | --- |
| 5QI | 5QI for default PCC of an MBS session | Optional |
| ARP | ARP for default PCC of an MBS session | Optional |
| AMBR | AMBR for an MBS session | Optional |
| GBR | Maximum aggregate bitrate that can be provided across all GBR QoS Flows for an MBS session. | Optional |
| media types | Allowed media types for an MBS session | Optional |

The policy subscription profile may be extended with operator-specific information.

2nd change

#### 7.1.1.2	MBS Session Creation without PCC

This procedure is used by the AF to start the MBS Session towards 5GC and consist of TMGI allocation, and MBS session creation, and they apply to both multicast and broadcast communications unless otherwise stated.

For multicast, MBS session establishment procedure triggered by UE join requests may follow the MBS session creation procedure to reserve resources towards NG-RAN. For broadcast, the MBS session start procedure to reserve resources towards NG-RAN is triggered by the MBS session creation procedure.

For both broadcast and multicast communication, the TMGI allocation may be separated from the MBS Session creation request.

For multicast communication, TMGI allocation procedure is applicable if TMGI is used as MBS Session ID.

Figure 7.1.1.2-1: MBS Session Creation without PCC

Steps 1 to 6 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1.	AF sends Nnef_TMGI_Allocate Request (TMGI number) message to NEF/MBSF to request allocation of a TMGI(s) to identify new MBS session(s).

NOTE 1:	Depending on the network deployment and use case, MB-SMF may receive requests from AF directly, or via NEF, or via MBSF, or via NEF and MBSF.

2.	NEF checks authorization of AF.

NOTE 2:	NEF is not required if AF is in trusted domain.

3.	NEF/MBSF discovers and selects an MB-SMF using NRF or based on local configuration,

4.	NEF/MBSF sends an Nmbsmf_TMGI_Allocate Request (TMGI number) message to the MB-SMF.

5.	MB-SMF allocates TMGI(s) and returns the TMGI(s) to the NEF/MBSF via the Nmbsmf_TMGI_Allocate response (TMGI(s), expiration time).

6.	The NEF or MBSF responds to the AF by sending an Nnef_TMGI_Allocate Response (TMGI(s), expiration time).

7.	The AF may perform a Service Announcement towards UEs. The AF informs UEs about MBS Session information with MBS Session ID, e.g., TMGI, SSM, and possibly other information e.g., MBS service area, session description information, etc.

The MBS service area information can be Cell ID list, TAI list, geographical area information or civic address information. Amongst them, Cell ID list and TAI list shall only be used by AFs who reside in trust domain, and when the AFs are aware of such information.

The UE needs to be aware if the service is broadcast or multicast to decide if JOIN is to be performed.

Editor's note:	How to do service announcements requires SA WG4 /WG6 coordination.

8.	AF of content provider may provide description for an MBS session (possibly providing information for a previously allocated TMGI to NEF via a Nnef_MBSSession_Create request (([MBS Session ID], service type, MBS information, [TMGI allocation indication]). If step 1-6 has not been executed before, the AF may provide a SSM or it may request that the network allocates an identifier for the MBS session (i.e., TMGI). The AF provides the service type (i.e. either multicast service or broadcast service). MBS session information may further include AF Service Identifier, service requirements (for each media component: Media type, Media format, bandwidth requirements, flow description, Application Identifier, Priority indicator) and Any UE indication (indicating whether a multicast MBS session is "open to any UEs"), MBS service area, start and end time of the MBS session and MBS session state (active/inactive). In addition, MBS information may also indicate whether the allocation of an ingress transport address is requested.

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to Cell ID list or TAI list.

For broadcast communication, the AF may determine MBS FSA ID(s) for the broadcast session based on business agreements and include them in the description of the MBS session.

9	NEF/MBSF checks authorization of content provider.

10.	NEF/MBSF discovers MB-SMF candidates and selects MB-SMF as ingress control node, possibly based on MBS service area. If a TMGI is included in step 8, NEF/MBSF finds MB-SMF based on that TMGI.

11.	NEF/MBSF sends Nmbsmf_MBSSession_Create Request (MBS Session ID, service type, TMGI allocation indication, MBS service area information, ingress transport address request indication) to MB-SMF, to request MB-SMF to reserve ingress resources for a MBS distribution session, The NEF/MBSF provides MBS Session ID or request allocation of a TMGI, and indicate the requested service type (either multicast service or broadcast service) and MBS session state (active/inactive). It also indicates that the allocation of an ingress transport address is requested if this was requested in step 8, or if the MBSF decides to insert an MBSTF into the user plane for the MBS session. The request also includes the Any UE indication if provided in step 8.

The MBS service area is provided by NEF/MBSF to the MB-SMF if provided by the AF in step 8.

MBS FSA ID(s) are provided by NEF/MBSF to the MB-SMF if provided by the AF in step 8.

If requested to do so, or if a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF allocates a TMGI.

For broadcast communication, if no MBS FSA ID(s) have been received, the MB-SMF selects MBS FSA ID(s) for the broadcast session based on local configuration.

12.	If a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF updates its NF profile at the NRF with the serving MBS Session ID. If an MBS service area information was received in step 11, the MB-SMF updates its NF profile at the NRF with that information.

NOTE 3:	If TMGI is used to represent an MBS Session, MB-SMF does not need to update NRF if the TMGI range(s) supported by an MB-SMF is already included in the MB-SMF profile when MB-SMF register itself into NRF.

13.	The MB-SMF derives the required QoS parameters locally.

14	MB-SMF selects the MB-UPF. If the allocation of an ingress transport address was requested in step 11, the MB-SMF requests the MB-UPF to reserve user plane ingress resources. If multicast transport of the MBS data towards RAN nodes is to be used, the MB-SMF also request the MB-UPF to reserve for the outgoing data a tunnel endpoint and the related identifiers (source IP address, SSM and GTP Tunnel ID) and to forward data received at the user plane ingress resource using that tunnel endpoint.

If the allocation of an ingress transport address was not requested in step 11, the MB-SMF provides the SSM received as MBS session ID to the MB-UPF and requests the MB-UPF to join the corresponding multicast tree from the content provider. The MB-SMF may also defer the configuration to join the corresponding multicast tree e.g. based on information that the session is inactive, QoS requirements and MBS start/end time until receiving the first query for the MBS session as part of the establishment procedure in clause 7.2.1.3, or until receiving a request to activate the MBS session via the MBS Session Update procedure in clause 7.1.1.6.

15.	If requested, MB-UPF selects an ingress address (IP address and port) and a tunnel endpoint for the outgoing data and provides it to MB-SMF.

16.	MB-SMF indicates the possibly allocated ingress address to the NEF/MBSF. MB-SMF may include TMGI if it is allocated in step 11. For broadcast communication, the MB-SMF includes any MBS FSA ID(s) selected in step 11. It also indicates the success or failure of reserving transmission resources.

17.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.1.

18.	[Optional] If the MBSF decides to use an MBSTF, the NEF/MBSF provides the ingress address received in step 16 towards the MBSTF as DL destination. If the allocation of an ingress transport address was requested in step 8, the MBSF requests the MBSTF to allocate the user plane ingress resources. If the allocation of an ingress transport address was not requested in step 8, the MBSF provides the SSM received as Multicast session ID in step 8 and requests the MBSTF to join the corresponding multicast tree from the content provider.

19.	[Conditional on step 19]	If requested, the MBSTF selects an ingress address (IP address and port) and provides it to NEF/MBSF.

20.	The NEF/MBSF-C indicates the possibly allocated ingress address and other parameters (e.g. TMGI) to the AF via an Nnef_MBSSession_Create response ([TMGI], [Allocated ingress address])). If MBS Session ID is not provided in step 8, or the MBS Session ID is SSM, the NEF/MBSF provides the allocated TMGI. If AF requested the allocation of an ingress transport address, the message also includes the allocated ingress address. For broadcast communication, the message also includes any MBS FSA ID(s) received in step 17.

21.	Same as step 7. The AF may also perform a service announcement at this stage.

22.	For multicast communication, depending on configuration UEs can join the MBS Session as specified in clause 7.2.1.

3rd change

#### 7.1.1.3	MBS Session Creation with PCC

Figure 7.1.1.3-1: MBS Session Creation with PCC

Steps 1 to 7 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1. to 10:	Same as in Figure 7.1.1.2-1. Step 1 may include an application ID.

11. Same as step 11 in Figure 7.1.1.2-1. In addition, the NEF/MBSF decides based on local configuration or based on parameters received in step 8 (e.g. whether the session comprises several data flows) whether it will invoke the Npcf_MBSPolicy Authorization service for the MBS session. If so, the NEF/MBSF indicates to the MB-SMF that it will also provide a policy authorization for the MBS session to the PCF.

12. Same as step 12 in Figure 7.1.1.2-1.

13.	[Optional] If the NEF/MBSF indicated in step 11 that it will also provide a policy authorization for the broadcast session to the PCF, the MB-SMF selects a PCF and sends an Npcf_MBSPolicyControl_Create Request (MBS session ID) for the MBS session towards the PCF, and defers step 25 until receiving an Npcf_MBSPolicyControl_UpdateNotify for the MBS session. Otherwise, the MB-SMF decides based on local configuration whether to invoke the Npcf_MBSPolicyControl service.

14.	[Conditional on step 13] The PCF registers at the BSF that it handles the MBS session by using Nbsf_management_Register Request (MBS Session ID, PCF ID). It provides an identifier that the policy association is for MBS and the MBS Session ID, its own PCF ID and optionally its PCF set ID.

15.	[Optional] The PCF may retrieve preconfigured policy information for the MBS session based on the multicast address as multicast session ID (e.g. applicable QoS, the MBS Session-AMBR and/or default 5QI) from the UDR.

16.	[Conditional on step 13] The PCF responds with Npcf_MBSPolicyControl_Create Response (MBS Policy, see clause 6.10) with policies for the MBS Session ID. The MBS Policy may include the Session-AMBR for the MBS session and 5QI for the MBS QoS Flow.

NOTE 1:	If there is no service requirement, the PCF may create the MBS policy according the operator's policy.

There are two alternatives to initiate the policy Authorization service operation.

In the Alt-A, the MB-SMF is the Npcf_MBSPolicy Authorization_Create service operation consumer. The step 17 to step 19 are performed after step 16.

In the Alt-B, the NEF/MBSF is the Npcf_MBSPolicy Authorization_Create service operation consumer. The step 20 to step 25 are performed.

17.	[Optional, Alt-A] The MB-SMF sends an Npcf_MBSPolicy Authorization_Create Request to PCF with the MBS session ID and MBS session information (DNN if available, S-NSSAI if available, AF Service Identifier, for each media component: Media type, Media format, bandwidth requirements, flow description, Application Identifier , Priority indicator).

18.	[Optional, Alt-A] The PCF may retrieve policy information for the application ID from the UDR.

19.	[Optional, Alt-A] The PCF sends an Npcf_MBSPolicy Authorization_Create Response to MB-SMF:

The PCF determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the MB-SMF and determines whether this QoS is allowed (e.g. according to the policy input configuration in the UDR).

If the request is not authorized or the required QoS is not allowed, the PCF indicates so in the response to the MB-SMF.

20.	[Optional, Alt-B]	The MB-SMF sends a Nmbsmf_MBSSession_UpdateNotify Request (MBS Session ID, PCC control established) to the NEF/MBSF

21-22.	[Optional, Alt-B] The NEF/MBSF uses the BSF Discovery service to discover the PCF serving the MBS session with the MBS session ID by using Nbsf_management_Discovery operation.

23.	[Optional, Alt-B] The NEF/MBSF sends an Npcf_MBSPolicy Authorization_Create Request to PCF with the MBS session ID and MBS session information (DNN if available, S-NSSAI if available, AF Service Identifier, for each media component: Media type, Media format, bandwidth requirements, flow description, Application Identifier, Priority indicator):

24.	[Optional, Alt-B] The PCF may retrieve policy information for the application ID from the UDR.

25.	[Optional, Alt-B] The PCF sends an Npcf_MBSPolicy Authorization_Create Response to NEF/MBSF:

The PCF determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (e.g. according to the policy input configuration in the UDR).

If the request is not authorized or the required QoS is not allowed, the PCF indicates so in the response to the NEF.

26.	When obtaining a request for the creation of a policy association (signal 21), for which it already performs policy control towards an MB-SMF, the PCF always provides a policy update to the MB-SMF; if no real policy update is required, the PCF repeats previous policies or sends an empty update message.

27-29	Same as steps 14-16 in Figure 7.1.1.2-1.

30.	When obtaining an MBS policy control update from the PCF (signal 16 for Alt-A or signal 29 for Alt-B) for a broadcast session, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.1 to request the allocation of resources to for the transmission of the broadcast session.

31-35	Same as steps 18-22 in Figure 7.1.1.2-1.

NOTE:	Steps 31-35 can be executed in parallel to step 30.

4th change

#### 9.1.3.1	General

Service description: The following are the key functionalities of this NF service:

-	(between AMF or SMF and MB-SMF) For multicast, NF Service Consumer can use this service to request the reception of MBS data or to terminate the reception of MBS data of a multicast session, or for a location-dependent MBS multicast session, the part of the MBS multicast session within a service area;

-	(between SMF and MB-SMF) For multicast, allow consumer NFs to query information (e.g. QoS information) about MBS Session(s), or for a location-dependent MBS multicast session, about the part of the MBS multicast session within a service area. This service will be invoked by SMF for UE join event;

-	(between SMF (only for multicast) or MBSF/NEF and MB-SMF) Allow consumer NFs to subscribe and unsubscribe for an Event ID and MBS Session, or for a location-dependent MBS session, related to the part of the MBS session within a service area (only for MBSF/NEF);

-	(between SMF (only for multicast) or MBSF/NEF and MB-SMF) Notifying events on the MBS Session, or for a location-dependent MBS session, related to the part of the MBS session within a service area (only for MBSF/NEF), to the subscribed NFs;

-	(between MBSF/NEF/AF and MB-SMF) Creation/Modification/Activation/Deactivation/Release of multicast session, or for a location-dependent MBS multicast session, the part of the MBS multicast session within a service area; and

-	(between MBSF/NEF and MB-SMF) Creation/Modification/Start/Stop/Release of broadcast sessions, or for a location-dependent MBS broadcast session, the part of the MBS broadcast session within a service area.

The following events related to MBS session context can be subscribed by SMF as consumer NF:

-	(between SMF and MB-SMF, only for multicast) QoS change: The event notification is sent when QoS within a multicast session changes, e.g. adding/removing QoS flow(s);

-	(between SMF and MB-SMF only for multicast) multicast session status (activated, deactivated);

-	(between SMF and MB-SMF only for multicast) multicast session service area change (for a location-dependent MBS broadcast session, the notification relates to the part of the MBS broadcast session within a service area);

-	(between SMF and MB-SMF only for multicast) multicast session release.

The following events related to an MBS session, or for a location-dependent MBS session, related to the part of the MBS session within a service area, can be subscribed by MBSF, NEF, or AF as consumer NF:

-	(between MBSF/NEF/AF and MB-SMF) MBS session release due to TMGI expiry;

-	(between MBSF/NEF/AF and MB-SMF) Broadcast delivery status;

-	(between MBSF/NEF/AF and MB-SMF) PCC control established.

NOTE:	Whether event IDs are needed can be determined by stage 3.

5th change

#### 9.2.2.1	General

Service description: NF Service Consumer, e.g. MB-SMF can create and manage a MBS Policy Association in the PCF through which the NF Service Consumer receives policy information for a MBS Session.

As part of this service, the PCF may provide the NF Service Consumer, e.g. MB-SMF with policy information about the MBS Session that may contain:

-	MBS Session related policy information.

-	PCC rule information.

Editor's note:	Details of MBS Session related policy information, PCC rule information, and Policy Control Request Trigger information need to be defined in an appropriate location of the TS and referenced.

At MBS Session establishment the NF Service Consumer, e.g. MB-SMF requests the creation of a corresponding MBS Policy Association with the PCF (Npcf_MBSSMPolicyControl_Create) and provides relevant parameters about the MBS Session to the PCF.

-	When the PCF has created the "MBS Policy Association", the PCF may provide policy information as defined above.

The PCF may at any time provide updated policy information (Npcf_MBSPolicyControl_UpdateNotify).

At MBS Session Release the NF Service Consumer, e.g. MB_SMF requests the deletion of the corresponding MBS Policy Association.

Editor's note:	Parameters of the service operations are FFS.

6th change

#### 9.2.3.2	Npcf_MBSPolicyAuthorization_Create service operation

Service operation name: Npcf_MBSPolicyAuthorization_Create

Description: Authorize the request, and optionally determines and installs MBS Policy Control Data according to the information provided by the NF Consumer.

Inputs, Required: MBS session ID, identification of the application session context.

Inputs, Optional: DNN if available, S-NSSAI if available, AF Service Identifier, for each media component: Media type, Media format, bandwidth requirements, flow description, Application Identifier, , Priority indicator.

Outputs, Required: Success (application session context) or Failure (reason for failure).

Outputs, Optional: The service information that can be accepted by the PCF.

7th change

#### 9.2.3.3	Npcf_MBSPolicyAuthorization_Update service operation

Service operation name: Npcf_MBSPolicyAuthorization_Update

Description: Provides updated information to the PCF.

Inputs, Required: Identification of the application session context.

Inputs, Optional: AF Service Identifier, for each media component: Media type, Media format, bandwidth requirements, flow description, Application Identifier, Priority indicator.

Outputs, Required: Success or Failure (reason for failure).

Outputs, Optional: The service information that can be accepted by the PCF.

Provides updated application level information and communicates with Npcf_MBSPolicyControl service to determine and install the policy according to the information provided by the NF Consumer. Updates an application context in the PCF.

8th change

#### 9.4.3.2	Nnef_MBSSession_Create service operation

Service operation name: Nnef_MBSSession_Create

Description: Create a new multicast session or broadcast session, or for a location-dependent MBS session, the part of the MBS session within a service area. Optionally implicitly subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (SSM or TMGI) or TMGI request, Service Type (broadcast or multicast).

Input, Optional: MBS service area (mandatory for broadcast. Can be Cell ID list and/or TAI list, or geographical area information, or civic address information), MBS start time, MBS stop time, service requirement (for each media component: Media type, Media format, bandwidth requirements, flow description, Application Identifier, Priority indicator), Input Transport Address Request, session activity status (active/inactive), for subscription to notifications notification target address. For a multicast session, indication that any UE may join the multicast session. AF Service Identifier. Request for location-dependent session. For a broadcast session, MBS FSA ID(s).

Output, Required: Result Indication.

Output, Optional: TMGI, Cause, MB-UPF tunnel info, MBS FSA ID(s), Area Session ID.

End of changes