SA WG2 Meeting #S2-151E	S2-2203971

16 - 20 May, 2022, Electronic meeting

# Title:	MSK update on MBS multicast session update procedure

Abstract of the contribution:  This discussion paper provides some background for the draft LS in S2-2203970

1 Discussion

At the last SA2 meeting, a reply to the CT1 LS on MSK update on MBS multicast session update procedure could not be agreed. It was agreed in principle to postpone related changes and ask SA3 for some clarifications related to their procedures in TS 33.501.

However, the related LS needed to be postponed because there was no agreement whether to ask SA3 questions related to session activation.

Proposal to extend the session activation procedure with session management related information have not been agreed in RAN3 and it appears undesirable to reopen related discussions at a very late stage.

Session activation is not mentioned at all in the related procedures in Clause W.4.1.2 of TS 33.501.  It is clear from those procedures that individual UEs need to be updated with the new MSK using the session update as specified in clause 7.2.6 in TS 23.247 to deliver the new MSK to the UE. It seems that only the PDU session modification (step 12 in clause 7.2.6) is suitable to deliver MSKs to the UE in a protected way.

To avoid that idle UEs are paged while the MBS session is inactive, an MBS session update could be deferred until the UE enters the connected mode. This is a general issue also affecting QoS profile and/or MBS Service Areas using those procedures. SA2 may consider allowing to postpone such updates until the related PDU session is activated. Related CRs can be discussed during SA2#152 when the agenda allows LS unrelated CRs.

However, it seems not appropriate to confuse SA3 with session activation related questions.

At the last CT4 meeting, the following CRs have already been agreed.

| C4-222327 | MBS Security Context Delivery | Samsung | Rel-17 | 29.532 | 0001 | B |
| --- | --- | --- | --- | --- | --- | --- |
| C4-222328 | MBS Security Context (MSK/MTK) Definitions | Samsung | Rel-17 | 29.571 | 0342 | B |

It could thus be possible to simply align stage 2 with the agreed stage 3, and an in-depth discussion in SA2 might not be required.

2 Proposal

It is proposed to agree the draft LS in S2-2203970 without additional questions related to MBS session activation.

SA2 may consider CRs allowing to postpone PDU session modifications related to MBS update procedures until the related PDU session is activated in SA2#152.