SA WG2 Meeting #S2-151E	S2-2203973

16 - 20 May, 2022, Electronic meeting

Source:	Nokia, Nokia Shanghai Bell

Title:	solution proposal for key issue 6 (Improvement for potential performance issues related to high numbers of public safety UEs)

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_MBS2 / Rel-18

Abstract of the contribution: this paper proposes a solution for key issue 6 (Improvement for potential performance issues related to high numbers of public safety UEs).

# 1		Discussion

Key issue 6 is defined as follows:

## 5.6	Key Issue #6: Improvement for potential performance issues related to high numbers of public safety UEs

### 5.6.1	Description

Public safety requirements are documented in TS 22.179 [9] and related procedures are documented in TS 23.280 [7] and TS 24.379 [8].

Specific 5MBS requirements for public safety are documented in Clause 4.2.

Based on the 5MBS requirements for public safety documented in Clause 4.2, this Key issue will study whether there are any performance issues for high number of public safety UEs, and for identified performance issues related enhancements to 5MBS.

NOTE:	Coordination with RAN WGs and SA6 WG will be required.

Clause 4.2 specifies related architectural requirements

## 4.2	Specific architectural requirements and principles for public safety service in a cell with large number of UEs

Public Safety specific architectural requirements and principles:

-	Solutions shall enable simultaneous reception of MBS session data for a higher number of UEs in a cell than can be operating in RRC_CONNECTED state, to participate in public safety group calls using MBS-based service.

-	Solutions shall ensure that the pre-emption and admission control mechanisms result in public safety UEs being able to complete the setup of the MBS services and then operate according to regulation and operator policy, when a mix of UEs participating in one or more services and/or one or more sessions within each service is present in the cell.

NOTE 1:	This issue, which involves other Working Groups, deals with aspects like recognizing which UEs may be moved to RRC_INACTIVE (service continues via MBS) vs. moved to RRC_IDLE (service may fail), not impacting the UE while the UE is setting up connections or waiting for the floor (permission to talk), ability to override user settings in order to be able to pre-empt, if necessary, etc.

-	Solutions shall enable provision of assistance information to NG-RAN from the application function (AF) via 5GC, if required/needed.

Editor's note:	Solutions may require information to NG-RAN and application function (AF) to enable proper provisioning. This includes, e.g. the SA6 and RAN WGs deciding on, what information to be provided on, which members of a public safety group should stay in RRC_CONNECTED and which one(s) are candidates for being transitioned to RRC_INACTIVE.

# 2		Proposal

It is proposed to include the following changes in TR 23.700-47.

*** 1st Change ***

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 MBS session reception in RRC Inactive | 2 MOCN network sharing | 3 On demand multicast MBS session | 4 Group Message Delivery | 5 Coexistence with existing power saving mechanisms for capability-limited devices | 6 Improvement on performance issues for public safety UEs |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 | X |  |  |  |  | X |
| 4 | X |  |  |  |  |  |
| 5 | X |  |  |  |  |  |
| 6 | X |  |  |  |  |  |
| 7 |  | X |  |  |  |  |
| 8 |  | X |  |  |  |  |
| 9 |  | X |  |  |  |  |
| 10 |  |  | X |  |  |  |
| 11 |  |  | X |  |  |  |
| 12 |  |  |  | X |  |  |
| 13 |  |  |  | X |  |  |
| 14 |  |  |  |  | X |  |
| 15 |  |  |  |  | X |  |
| 16 |  |  |  |  |  | X |
| 17 |  |  |  |  |  | X |
| x | x |  |  |  |  | x |

*** 2nd Change (all new)***

## 6.X	Solution #X: AF selects UEs to be kept in connected mode

### 6.X.1	Introduction

This solution addresses Key Issues #1 and #6.

### 6.X.2	Description

AF Identifies UEs that should be kept in connected state, e.g. UEs of likely talkers in an MBS session and provides this information to CN via Nnef_ParameterProvision_Create/Update service operations (see TS 23.502 Clause 4.15.6.2)

AF may use configuration of frequency of floor requests received from UE as criterion to select UERs and should preferably only keep a small fractions of the UEs in participating in an MBS session with many participants in connected state.

AF preferably removes information that UEs shall always be connected when related MBS session ends.

NEF stores in UDM users that preferably shall be kept connected.

For each indicated UE, the UDM stores as part of the UE subscription data that user shall preferably be kept connected

SMFs are notified about changes in UE subscription data based on previous subscription or inquire UE subscription data when PDU session is being established. This includes information that user shall preferably be kept connected.

SMF provides the information that UE shall preferably be kept connected as part of the SM management information via AMF to NG RAN node handling the user.

NG RAN nodes preferably keep UE for which such information was received in RRC and CM connected state.

As an additional improvement, UEs do not send „listener reports“ while in MBS service area and in  RRC inactive mode. Listener reports can become a problem in particular for UEs close to cell border moving frequently between cells.

Editor's note:	Coordination with SA6 is required.

### 6.X.3	Procedures

Figure 6.x.3.2-1: AF selects UEs to be kept in connected mode

- AF identifies UEs to be kept in RRC connected state

- AF uses parameter provisioning procedure to provide UEs to be kept in connected state to NEF.

- NEF stores in UDM users that preferably shall be kept connected. For each indicated UE, the UDM stores as part of the UE subscription data that user shall preferably be kept connected

- SMFs are notified about changes in UE subscription data based on previous subscription or inquire UE subscription data when PDU session is being established. This includes information that user shall preferably be kept connected.

5-6. SMF provides the information that UE shall preferably be kept connected as part of the SM management information via AMF to NG RAN node handling the user.

- NG RAN nodes preferably keep UE for which such information was received in RRC and CM connected state.

### 6.1.4	Impacts on services, entities and interfaces.

Editor's note:	This clause describes impacts to existing services, entities and interfaces.

AF

Select UEs to be kept in connected mode and provide those UEs to CN via parameter provisioning procedure

NEF/UDM/SMF

Updates to parameter provisioning procedure to transfer UEs to be kept in connected mode

SMF

Inform NG-RAN that UE shall be kept in connected state via PDU session modification

NG-RAN

Keep indicated UE in connected state

UE

Do not send listener reports while in MBS service area and inactive state