3GPP TSG-SA WG2#151E	S2-2204056

Elbonia, 16 - 20 May 2022	(was S2-22xxxxx)

Source:	CATT

Title:	KI #4 & #6, New Sol: QoS handling for PINE communication with 5GC via PEGC

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract of the contribution: This contribution proposes a solution for supporting QoS handling of PINE communication via PEGC.

- Introduction

This paper proposes a solution for QoS handling of PINE communication via PEGC.

- Proposal

It is proposed to capture the following texts into TR 23.700-88.

>>>>Start Changes (ALL TEXT NEW)<<<<

6.X	Solution #X: QoS handling for PINE communication with 5GC via PEGC

6.X.1	Description

The solution applies to Key Issue #4 "Communication of PIN" and Key Issue #6 "Policy and parameters provisioning for PIN".

The PIN architecture is depicted as in Figure 6.X.1-1. The PEMC and PEGC have Uu capability and can access 5GC using the procedures as defined in TS 23.502 [xx]. The PINE may have limited communication capability and can only support some forms of direct non-3GPP access, e.g. WiFi or BT.

Figure 6.X.1-1: Reference architecture for PIN

The PEMC is responsible for managing the PIN, including creating a PIN, adding/removing a PINE from the PIN, etc. The PEMC may be preconfigured with PIN information that it can support, or the PIN information can be provisioned by PCF to the PEMC.

The PEGC provides connectivity services for PINE to access the network. The PINE can communicate with PEMC, PEGC and other PINEs via direct non-3GPP access (e.g. WiFi and BT). The PINE can also communicate with 5GC via the PEGC, and the PEGC acts as an IP router to relay traffic between PINE and 5GC.

For a PINE accessing network via PEGC, the QoS requirement of the relayed traffic can be satisfied by the corresponding QoS control for the direct non-3GPP access (out of 3GPP scope) and the QoS control for the QoS flow. The end-to-end QoS can be met when the QoS requirements are properly translated and satisfied over the two legs respectively.

6.X.2	Procedures

6.X.2.1	Procedures for policy and parameters provisioning for PEGC

The policy and parameters for PIN and PIN element discovery may be provisioned by PCF.

The "PCF based Service Authorization and Provisioning to UE" procedure as defined in clause 6.2.2 of TS 23.304 [yy] is reused for PCF to provision PIN policy and parameters to PEGC.

The "AF-based service parameter provisioning for ProSe over control plane" procedure as defined in clause 6.2.5 of TS 23.304 [yy] is reused for PIN AS to provide PIN Service Parameters to the PCF.

The PIN policy and parameters include:

-	a mapping between a Uu QoS parameters (including 5QI, GFBR/MFBR, ARP) value and QoS information of direct non-3GPP access (including QoS characteristics, GFBR/MFBR, ARP, Periodicity)

6.X.2.2	Procedures for QoS handling of PINE communication with 5GC via PEGC

Depicted in figure 6.X.2.2-1 is the procedure for Network initiated QoS flow establishment for PINE.

Figure 6.X.2.2-1: Network initiated QoS flow establishment for PINE

1.	The PEGC registers to network and establishes PDU session for relaying PINE traffic.

2.	PINE discovers PEGC using solutions for KI#2 and establishes connection with PEGC over direct non-3GPP access (e.g. WiFi and BT).

The application layer signalling interacts between PINE and PIN AS.

3.	The PIN AS may trigger a new QoS flow establishment and that triggers network-initiated QoS flow establishment procedure as defined in clause 4.3.3 of TS 23.502 [xx].

4.	The PEGC receives 5QI for the new established QoS flow from SMF, and the PEGC determines the direct non-3GPP access QoS info based on the PCF provisioned mapping as described in clause 6.X.2.1.

5.	The PEGC applies direct non-3GPP QoS information to reserve resources over direct non-3GPP access.

The traffic from/to the PINE is relayed by the PEGC.

Depicted in figure 6.X.2.2-2 is the procedure for PINE initiated QoS flow establishment.

Figure 6.X.2.2-2: PINE initiated QoS flow establishment

1.	The PEGC registers to network and establishes PDU session for relaying PINE traffic.

2.	PINE discovers PEGC using solutions for KI#2 and establishes connection with PEGC over direct non-3GPP access (e.g. WiFi and BT).

3.	The PINE establishes new QoS flow with PEGC and provides direct non-3GPP access QoS info to the PEGC.

4.	The PEGC determines the 5QI and other QoS parameters based on the PCF provisioned mapping as described in clause 6.X.2.1.

5.	The PEGC initiates PDU session modification procedure to establish new QoS flow.

The traffic from/to the PINE is relayed by the PEGC.

6.X.3	Impacts on services, entities and interfaces

PEGC:

-	Request and Receive PIN policy/parameters from PCF.

PCF:

-	Provision PIN policy/parameters to PEGC.

>>>>End Changes<<<<