SA WG2 Meeting #151e	S2-2204094

# May 16th – 20th, 2022; Elbonia              		  (revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	(KI4) Solution #3 update

Document for: 	Approval

Agenda Item: 	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract of the contribution: Solution #3 update

# 1	Discussion

Void.

# 2 Proposal

It is proposed to update TR 23.700-74 as follows

## 6.3	Solution #3: use of SMF sets for 5G VN group communications.

### 6.3.1	Introduction

This solution addresses KI 4.

### 6.3.2	Functional Description

The Support of group communication for a 5G VN needs to ensure

- SMF redundancy for the reliability of the 5G VN group communications,

- Support of 5G VN groups with UE spread over a large country.

Such redundancy can be provided by the SMF set feature that has been supported by 5GC since Rel-16.

Any inter SMF interactions related with intra SMF set co-ordination about a 5G VN group ( e.g. on the control of the N19 tunnel between the UPF(s) involved in a 5G VN group communication) is left outside the scope of 3GPP specifications.

NOTE:	This solution does not assume that 5G VN group communication only happens in a local area, e.g. involving the same unique data centre as a SMF Set may be deployed across multiple sites/data centres. Furthermore, to handle 5G VN groups with UE spread over a large country, I-SMF may be used to connect remote UE(s) to the (unique) SMF set serving the 5G VN group.

### 6.3.3	Procedures

Following note of clause 5.29.3 of TS 23.501 [2] "PDU Session management" needs to be reworded to no more refer to a "SMF" but to a "SMF set".

NOTE 1:	The network is configured so that the same SMF is always selected for a certain 5G VN group, e.g. only one SMF registers on the NRF with the DNN/S-NSSAI used for a given 5G VN group.

Thus, the NOTE would read as follows:

NOTE 1:	The network is configured so that the same SMF *set* is always selected for a certain 5G VN group, e.g. only one SMF registers on the NRF with the DNN/S-NSSAI used for a given 5G VN group.

When the UE is not located in the service area of the SMF set associated with the DNN/S-NSSAI used for a 5G VN group, per 3GPP Rel-16 procedures, the AMF involves an I-SMF for the PDU Session.

NOTE 1:	The SMF(s) that are part of a SMF set can collaborate via one of or a combination of following implementation dependant means:

The SMF(s) can share contextual information associated with the 5G VN group (DNN + S-NSSAI) in the UDSF. The contextual information may e.g. relate to the N19 configuration, to the list of PDU Sessions established by 5G VN group members, etc.

Implementation dependant signalling can be used between SMF(s) that are part of a SMF set e.g. based on an implementation choice SMF(s) within the set can elect one SMF to control the N19 configuration.

The solution supports 5G VN groups with UE spread over a wide area (e.g. a whole country like China). In this case, the SMF(s) that control the PDU Session (SMF(s) that have the PCF and CHF interactions, etc.) are part of the same SMF set while I-SMF(s) handling the local connectivity to the UE do not need to be part of this SMF set. A SMF can control local traffic switching between group members via proper FAR(s) and PDR(s) rules sent over N16a. When a UE moves out of the service area of the SMF set that controls the communications within a 5G VN group an I-SMF is used as specified in 3GPP Rel-17 specifications.

NOTE 2:	Each 5G VN group can be associated with a specific slice, using e.g. the Slice Differentiator (SD), parameter of the S-NSSAI, to ensure that PDU Sessions of 5G VN users in the I-SMF serving areas are handled by I-UPF(s) supporting the necessary features possibly including the support of a N6 interface associated with the 5G VN group .

NOTE 3:	Even though signalling between SMF(s) that are part of a SMF set is based on an implementation choice, the SMF set or SMF instances in SMF set need to support functionality for 5G VN group communications across SMFs.

N16a is updated to allow the SMF that control the PDU Session (the SMF that has the PCF interactions, etc.) to control local switching at I-UPF between members of a 5G VN group.

-	This SMF control of local switching at I-UPF(s) reuses following capability defined in ETSUN: “SMF may then at any time invoke a Nsmf_PDUSession_Update Request to send N4 information to the I-SMF” (as defined in TS 23.502 clause 4.23.9.0)

-	But the SMF control of local switching requires N16a changes allowing the SMF to send TS 29.500 N4Information that is related to "5G VN internal" switching in the I-UPF(s) as defined in 23.502 clause 5.8.2.13.0

-	this allows local switching in the I-UPF(s) for traffic between “local” 5G VN group members.

-	Traffic of local 5G VN group members that is not locally switched (due to "5G VN internal" switching) is sent over the N6 interface of the local UPF(s) associated with the 5G VN group. This traffic may be handled as described in solution 16.

### 6.3.4	Impacts on existing entities and interfaces

The solution impacts are on:

-	Registering a SMF set (and not a SMF instance) as associated with the DNN + S-NSSAI representing a certain 5G VN group.

-	possibly associate a 5G VN group with a dedicated Slice Differentiator,

-	N16a changes allowing the SMF to send TS 29.500 N4Information that is related to "5G VN internal" switching in the I-UPF(s).