SA WG2 Meeting #S2-150E	S2-2204105

06 Apri – 12 April 2022, Electronic,	(revision of S2-2202029)

Source:	China Telecom

Title:	KI#1 New Sol:5GS DetNet node reporting

Document for:	Approval

Agenda Item:	9.6

Work Item / Release:	FS_DetNet / Rel-18

Abstract of the contribution: This contribution proposes a solution for the DetNet control plane to obtain 5GS DetNet node capability and resource occupation. In the mobile network, the DetNet control plane needs to know the capability and resource occupation of the node before issuing the strategy. As a DetNet node, 5GS enhances network functions to exposure and report the capability to the DetNet controller.

- Introduction

On the SA2 #149 e-meeting, one of the FS_DetNet key issues was recognized as 5GS DetNet node reporting. The description of the key issue was as follows:

For this Key Issue, the following areas should be studied:

- which information the 5GS needs to report;

- how the 5GS collects the information to report.

The solution should clarify whether the NEF could be deployed between the TSCTSF and the DetNet controller.

In 5G mobile network, in order to realize DetNet deterministic forwarding mechanism and ensure the certainty of wide area, the DetNet control plane requires the DetNet node to report relevant information before issuing the strategy.

IETF RFC8655 stipulates that DetNet nodes need to report corresponding information to DetNet control plane, including DetNet node capability information, current usage of DetNet node resources, and identification and link characteristics of adjacent DetNet nodes (In the RFC 8655 clause 4.8).

Therefore, as a DetNet node, 5GS system should also exposure corresponding information to the DetNet control plane to assist the DetNet control plane in making corresponding forwarding strategy decisions.

This contribution suggests the method for the DetNet control plane to obtain the capability and resource occupation of 5GS DetNet node, and defines the capability and reporting mechanism of 5GS as a DetNet node to the DetNet control plane. An enhanced architecture supporting the reporting of mobile network to DetNet control layer is designed. The architecture enhances the network functions of NEF, SMF, UPF and RAN respectively, so as to support the information collection, subscription and reporting of DetNet capability.

Proposal

It is proposed to add the following to the TR 23.700-46 "Study on 5GS DetNet interworking"

* * * Start of 1st proposed change * * * *

6.X	Solution #1 for Key Issue #1: Network function enhancement to support 5GS DetNet node reporting

6.X.1	Introduction

In 5G mobile network, in order to realize DetNet deterministic forwarding mechanism and ensure the certainty of wide area, the DetNet control plane requires the DetNet node to report relevant information to the DetNet control plane before issuing the strategy.

IETF RFC8655 stipulates that DetNet nodes need to report corresponding information to DetNet control plane, including DetNet node capability information, usage status of DetNet node resources, and recognization of adjacent DetNet nodes.

Therefore, as a DetNet node, 5GS system should also report corresponding information to the DetNet control plane to assist the DetNet control plane in making corresponding forwarding strategy.

This report puts forward the method for the DetNet control plane to obtain the capability and resource occupation of 5GS DetNet node, and defines the capability and opening mechanism of 5GS as a DetNet node to the DetNet control plane. An enhanced architecture supporting the reporting of mobile network information to DetNet control layer is designed. The architecture enhances the functions of NEF, SMF, UPF and RAN respectively, so as to support the information collection, subscription and reporting of DetNet capability.

6.X.2	Functional Description

Figure 1 Enhanced architecture and network function

As shown in the figure1, the 3GPP exposure architecture is enhanced to support DetNet node reporting. Extend NEF function to support the capability exposure of DetNet node, and extend SMF, UPF and RAN to support DetNet information reporting function. The related network functions are enhanced to support following features:

- Extend NEF function to support the exposure of DetNet capability:

  - Receive the capability exposure subscription from DetNet control plane, and report related ability to it;

  - Forward DetNet information reporting requirements to SMF based on DetNet controller subscription, including reporting contents, reporting frequency, etc;

- Expand SMF to support DetNet information reporting function. Based on the reporting requirements issued by NEF, SMF receives DetNet related information reported by UPF on N4 interface and DetNet related information reported by RAN on N2 interface which is forwarded by AMF, comprehensively processes the information from both sides, generates reporting information, and sends it to NEF as required;

- Expand UPF to support DetNet information reporting function, and report the following information through N4 interface;

  - Detnet capability on the network side : including packet delay. The delay can be achieved through the QoS monitoring procedure.

  - Resource usage on the network side: may including CPU / memory / cache occupation of UPF and the network load.

  - Identity and link with adjacent DetNet nodes: the identification of surrounding nodes can be obtained through routing broadcast messages, and extend the QoS monitoring to support N6 interface delay detection.

- Extend RAN to support DetNet information reporting function, and report the following information through N2 interface;

  - DetNet capability on the wireless side: including delay on the air interface. The delay can be collected by the RAN QoS monitoring procedure.

The collection and reporting methods of network information are as follows:

- Acquisition and reporting method of forwarding delay of N3 / N9 interface:

- UPF receives the request for delay monitoring from the SMF, selects one or more downlink packets related to each requested QoS flow, and inserts the time stamp into the GTP-U extended header of these downlink packets;

- UPF measures and calculates the time delay in the corresponding direction;

- UPF reports the measurement of time delay on N4 interface according to the requirements of cycle or event triggering.

- Reporting method of occupation of CPU / memory / cache of UPF and network service load:

- UPF reports its resource occupation and current network load to SMF through N4 interface (statistical data of OAM can be used), and forwards it to DetNet control plane;

- The control plane allocates the forwarded service flow according to the occupation of UPF. The control plane specifies the forwarding time of the service flow to avoid packet loss caused by overload.

- Reporting method of adjacent DetNet nodes of UPF:

- UPF obtains identification of DetNet nodes around the N6 interface through BGP and other routing broadcast messages. Then UPF carries the timestamp in the header of the routing protocol.

- UPF detects the delay between N6 interface and surrounding nodes, extends the QoS monitoring supporting N6 interface, and obtains the delay of links.

- UPF reports the identification and link of adjacent DetNet nodes to SMF through N4 interface, then opens the information to DetNet control plane. Therefore, the control plane can send accurate forwarding time and other information.

6.X.3	Procedures

Figure 2 Subscription and escalation process of DetNet ability opening

Supplementary steps

- Before issuing the forwarding strategy, or based on the need of periodic collection of the DetNet capability, the DetNet control plane subscribes the capability exposure to NEF, which includes the information to be reported ,reporting frequency and triggers;

- NEF forwards the requested capability reporting requirements to SMF;

- SMF subscribes the capability reporting of DetNet from UPF;

- SMF subscribes the capability reporting of DetNet from RAN by AMF;

- According to the subscription request, RAN collects the relevant information on the wireless and air interface side, including monitoring the delay of air interface, etc;

- According to the subscription request, UPF collects the relevant information of DetNet on the network side, including monitoring N3 / N6 / N9 delay, collecting equipment and interface load through local management information, etc;

- SMF integrates the capability reporting messages received by N2 and N4 interfaces to form the capability reporting data of 5GS as a DetNet node, such as capability / resource occupation / adjacent nodes, which can be collected by NEF and be reported to the DetNet controller;

- NEF reports capability information to DetNet control plane;

- The DetNet control plane generates the DetNet forwarding strategy based on the received reports.

In the above figure, 1-4 is the capability reporting subscription procedure, and 5-8 is the capability reporting procedure.

6.X.4	Impacts on existing entities and interfaces

Editor's note:	This clause lists impacts to existing entities and interfaces.

The new requirements are mainly aimed at the functional enhancement of NEF, SMF, UPF and RAN modules

Extend NEF to achieve

- Receiving subscription information from DetNet control plane and reporting related capabilities to the outside world,

- Based on subscription requirements, send DetNet information reporting requirements to SMF, including the information to be reported, reporting frequency, etc.

Extend SMF to achieve

- Based on the reporting requirements issued by NEF, SMF receives DetNet related information reported by UPF on N4 interface.

- Comprehensively process relevant information and send it to NEF according to the required frequency or conditions.

Extend UPF to achieve

- Report the network side capability information, network side resource usage and the identity and link with adjacent DetNet nodes through N4 interface.

Extend RAN to achieve

- Report the capability information of the wireless side and the resource usage of the wireless side

6.X.4	Solution evaluation

- Adjust the strategy in time when the network capability changes,

- Better ensure the implementation of certainty on WAN,

- Without significantly changing the original framework, the information requirements of DetNet control plane can be met by expanding the existing network function.