3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204133

Elbonia, May 16 – 20, 2022

Source:	Huawei, HiSilicon

Title:	KI#4, Solution#6: Update to clarify roaming of mobile base station relays

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR/ Rel-18

Abstract: It is proposed to update Solution #6 to clarify roaming of mobile base station relays.

1. Introduction

This contribution proposed the following changes:

- Reusing the existing IAB-indication for IAB operation in roaming

Currently, the solution#6 defines a new mechanism for mobile base station relay to send a Registration Request message with an indication that the mobile base station relay intends to operate as a relay on behalf of the VPLMN.

According to the IAB operation defined in the TS 23.501, the IAB-UE provides an IAB-indication to the IAB-donor-CU when the RRC connection is established and the IAB-donor-CU includes the IAB-indication to the AMF. The details for the IAB operation defined in TS 23.501 are captured in the following:

TS 23.501 5.35.2 5G System enhancements to support IAB In IAB operation, the IAB-UE interacts with the 5GC using procedures defined for UE. The IAB-node gNB-DU only interacts with the IAB-donor-CU and follows the CU/DU design defined in TS 38.401 [42]. For the IAB-UE operation, the existing UE authentication methods as defined in TS 33.501 [29] applies. Both USIM based methods and EAP based methods are allowed, and NAI based SUPIs can be used. The following aspects are enhanced to support the IAB operation: - the Registration procedure as defined in clause 4.2.2.2 of TS 23.502 [3] is enhanced to indicate IAB-node's capability to the AMF; - The IAB-node provides an IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB and includes the IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform IAB authorization. - the UE Subscription data as defined in clause 5.2.3 of TS 23.502 [3] is enhanced to include the authorization information for the IAB operation; - Authorization procedure during the UE Registration procedure is enhanced to perform verification of IAB subscription information; - UE Context setup/modification procedure is enhanced to provide IAB authorized indication to NG-RAN.

Therefore, it is proposed to reuse existing mechanism of IAB operation for roaming.

2. Some clean-up/ editorial changes for Soluton#6

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-05 V0.2.0.

# * * * * First change * * * *

6.6	Solution #6: Enabling roaming of mobile base station relays

6.6.1	General

This solution addresses Key Issue#4 as defined in clause 5.4.

6.6.2	Functional description

The high-level points of the proposed solution are:

1.	The mobile base station relay may be pre-configured with one or more of the following:

a)	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB node;

NOTE 1:	The geographic area format used for V2X policy in TS 24.588 [8] can be re-used. A geographic area is encoded as a list of coordinates, each corresponding to a summit of a polygon.

b)	a list of PDU session parameters, e.g. specific DNN and S-NSSAI, and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session toward a regional OAM server for IAB node configuration;

2.	When the mobile base station relay moves out of coverage of the HPLMN:

-	if the mobile base station relay is pre-configured with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB node as described in step 1a), the IAB-UE attempts to select the PLMNs in the list, in priority order;

-	otherwise, the mobile base station relay performs PLMN selection as specified in TS 23.122 [12];

3.	Upon selecting a VPLMN as described in step 2, the mobile base station relay initiates a registration procedure and indicates in the RRC connection message that it intends to operate an IAB node.

4.	The AMF in the VPLMN either:

a)	accepts the Registration Request of the mobile base station relay; or

b)	rejects the Registration Request of the mobile base station relay with a 5GMM cause value indicating that the mobile base station relay is not allowed to operate as a n IAB node in the VPLMN;

5.	If the Registration Request of the mobile base station relay is accepted and:

a)	the mobile base station relay:

i)	performs an IAB node integration procedure with the VPLMN as specified in TS 38.401 [6] clause 8.12.1. If the mobile base station relay is pre-configured with the PDU session parameters for the OAM PDU session for the VPLMN or the geographic area as described in step 1b), the UE uses these parameters to establish the OAM PDU session during the procedure; or

NOTE 2:	When performing i), the PDU sessions of any UE camped on the mobile base station relay will be interrupted.

ii)	performs an IAB inter-donor full migration procedure, where the source donor CU is in the HPLMN and the target donor CU is in the VPLMN.

NOTE 3:	The IAB inter-donor full migration procedure is to be specified by RAN.

NOTE 4:	When performing ii), the PDU sessions if any UE camped on the mobile base station relay will be transferred from the HPLM to the VPLMN. This assumes that inter-PLMN handover between the HPLMN and the VPLMN is supported.

6.6.3	Signalling flows

6.6.3.1	Signalling flow for roaming of mobile base station relay as an IAB node using IAB node integration procedure

Figure 6.6.3.1-1: Signalling flow for roaming of mobile base station relay operating as an IAB node  using IAB node integration procedure

0.	The mobile base station relay may be pre-configured with:

-	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB-node;

-	a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session toward a regional OAM server for IAB configuration.

1.	The mobile base station relay moves out of coverage of its HPLMN and selects a VPLMN as follows:

-	if the mobile base station relay is pre-configured with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB-node, the IAB-UE attempts to select the PLMNs in the list, in priority order;

-	otherwise, the IAB-UE performs PLMN selection as specified in TS 23.122 [12].

2.	The mobile base station relay (i.e. IAB-UE) provides an IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in clause 5.35.2 of the TS 23.501[2]. The mobile base station relay sends a Registration Request message.

3.  When the IAB-indication is received, the IAB-donor-CU includes the IAB-indication in the message as defined in clause 5.35.2 of the TS 23.501[2].

4.	The VPLMN AMF checks whether the mobile base station relay is authorized by performing verification of IAB subscription information with UDM. The UDM provides the IAB-Operation allowed indication to AMF as part of the Access and Mobility Subscription data as defined in clause 4.2.2.2.2 of the TS 23.502[5].

5.	If the AMF determines that the mobile base station relay is authorized, the AMF sends IAB authorized indication to NG-RAN and a Registration Accept message to the mobile base station relay.

6.	If the mobile base station relay is pre-configured with the PDU session parameters for the OAM PDU session for the VPLMN or the geographic area (see step 0), the UE uses these parameters to establish the OAM PDU session during the procedure. The mobile base station relay perform an IAB node integration procedure with the VPLMN as specified in clause 8.12.1 of TS 38.401 [6]..

6.6.3.2	Signalling flow for roaming of mobile base station relay operating as an IAB node using IAB inter-donor full migration procedure

Figure 6.6.3.2-1: Signalling flow for roaming of mobile base station relay operating as an IAB node using IAB inter-donor full migration procedure

0.	The mobile base station relay may be pre-configured with:

-	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB-node;

-	a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session toward a regional OAM server for IAB configuration.

1.	The mobile base station relay moves out of coverage of its HPLMN and selects a VPLMN as follows:

-	if the mobile base station relay is pre-configured with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB-node, the IAB-UE attempts to select the PLMNs in the list, in priority order;

-	otherwise, the mobile base station relay performs PLMN selection as specified in TS 23.122 [12].

2.	The mobile base station relay (i.e. IAB-UE) provides an IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in clause 5.35.2 of the TS 23.501[2]. The mobile base station relay sends a Registration Request message.

3.  When the IAB-indication is received, the IAB-donor-CU includes the IAB-indication in the message as defined in clause 5.35.2 of the TS 23.501[2].

4.	The VPLMN AMF checks whether the mobile base station relay is authorized by performing verification of IAB subscription information with UDM. The UDM provides the IAB-Operation allowed indication to AMF as part of the Access and Mobility Subscription data as defined in clause 4.2.2.2.2 of the TS 23.502[5].

5.	If the AMF determines that the mobile base station relay is authorized, the AMF sends IAB authorized indication to NG-RAN and a Registration Accept message to the mobile base station relay.

6.	The mobile base station relay performs an IAB inter-donor full migration procedure, where the source donor CU is in the HPLMN and the target donor CU is in the VPLMN.

6.6.4	Impacts on services, entities and interfaces

The following impacts have been identified:

-	UE:

-	No impact.

-	Mobile base station relay:

-	Support pre-configuration with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB node.

Editor's note:	It needs to be evaluated if existing UE PLMN selection operation defined in TS 23.122 [12] is sufficient.

-	Support pre-configuration with a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session toward a regional OAM server for IAB node configuration.

-	Support for PLMN selection based on the prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB node.

-

-	Support IAB inter-donor full migration procedure.

Editor's note:	The support of IAB inter-donor full migration needs to be coordinated with RAN WGs.

# * * * * End of changes * * * *