3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204159

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI #2, New Sol: Modification of Timers when in or out of Coverage

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract: A new solution for Key Issue #2 is proposed where the UE and network can switch to a different periodic registration/TAU timer while out of coverage.

1. Introduction

This contribution proposes a new solution about sending to UE a future periodic registration timer/pTAU timer which is to be activated when UE leaves satellite coverage during the initial registration/attach procedure. When UE leaves the satellite coverage, the UE and the AMF/MME activates the future periodic registration timer/pTAU timer.

According to this solution, there is no need to negotiate periodic registration timer/pTAU timer every time when UE becomes CM-CONNECTED from CM-IDLE, or when UE is about to leave the satellite coverage.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-28.

# * * * * First change * * * *

6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 |  |  |
| 1 |  | X |  |  |
| 2 |  | X |  |  |
| 3 |  | X |  |  |
| 4 | X |  |  |  |
| 5 |  | X |  |  |
| 6 | X |  |  |  |
| X |  | X |  |  |

# * * * * Second change (All new text)* * * *

6.X	Solution #X: Modification of Timers when in or out of Coverage

6.X.1	Description

This solution resolves Key Issue #2 about the power saving enhancements for UE in discontinuous coverage.

In this solution, AMF/MME sends to UE a future periodic registration timer/TAU timer which is to be activated when UE leaves satellite coverage during the Registration/Attach procedure.

The future periodic registration timer/TAU timer can be set so that the UE does not need to perform Periodic Registration or TAU while out of coverage while allowing the network to maintain its preferred timer settings while in coverage automatically.

6.X.2	Procedures

Figure 6.X.2-1 shows the procedure to send the future periodic registration timer/TAU timer during any registration, attach or any TAU procedure.

Figure 6.X.2-1: high-level procedure for power saving enhancement in 5GS/EPS

1.	The UE initiates Registration/Attach/TAU procedure by sending Registration/Attach/TAU request to AMF/MME.

2.	The AMF/MME determines a future periodic registration timer/TAU timer which is to be activated when UE leaves satellite coverage considering satellite coverage information.

For UEs with no mobility, the AMF/MME determines the future periodic registration timer/TAU timer based on the UE location and the satellite coverage information.

For UEs moving in limited area or with known/predictable trajectories, the AMF/MME determines the future periodic registration timer/TAU timer based on the UE location, the known/predicted mobility pattern and the satellite coverage information.

The AMF/MME may also determine power saving parameters which is to be used when UE is in satellite coverage and RRC_IDLE, e.g. active time, periodic registration timer/TAU timer, eDRX parameters etc.

3.	The AMF/MME sends Registration/Attach/TAU Accept message to UE including the future periodic registration timer/TAU timer which is to be activated when UE leaves satellite coverage.

The AMF/MME also sends to UE the power saving parameters which is to be used when UE is in satellite coverage and RRC_IDLE.

4.	If the UE is in satellite coverage and RRC_IDLE, the UE activates the received power saving parameters. When the UE leaves the satellite coverage, the UE stops the power saving parameters and activates the future periodic registration timer/TAU timer.

The UE determines it leaves satellite coverage based on broadcast satellite ephemeris.

5.	The AMF/MME predicts when the UE leaves the satellite coverage and activates the future periodic registration timer/TAU timer.

When the future periodic registration timer/TAU timer expires, the UE initiates registration/TAU procedure as normal and may be assigned another the future periodic registration timer/TAU timer during the registration/TAU procedure.

6.X.3	Impacts on existing nodes and functionalities

AMF/MME:

-	determines a future periodic registration timer/TAU timer which is to be activated when UE leaves satellite coverage based on the UE location, the known mobility pattern if UE moves with known mobility pattern and satellite coverage information.

-	activates the future periodic registration timer/TAU timer at the timer when the UE is predicted to leave coverage.

UE:

-	stop using current power saving parameters and activate the future periodic registration timer/TAU timer when UE leaves satellite coverage.

# * * * * End of changes * * * *