SA WG2 Meeting #151e	S2-2204165

# Elbonia, May 16-20, 2022               			            (revision of S2-220xxxx)

Source: 	Apple

Title: 	KI #4, New Sol: Re-balancing of the global maximum number of registered UEs/PDU session for the S-NSSAI

Document for: 	Approval

Agenda Item: 	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes a solution for Key Issue #4: Support of NSAC involving multi service Area in TR 23.700-41.

# 1 Discussion

Key Issue#4: Support of NSAC involving multi service Area:

"For one S-NSSAI, there is only one configured global Maximum allowed number value for NSAC. It is possible more than one service area is associated with one S-NSSAI, e.g. to split a PLMN into multi-service areas. This impacts the use cases below as there will be more than one NSACF handling the UE:

-	Multi NSACF deployed within one PLMN: For NSACF deployment more than one service area are defined within one PLMN. For each service area one NSACF or NSACF set is selected for slice admission control. This include the control of the maximum allowed number of UE or PDU session.

-	Roaming: when one user resides at the visit PLMN, the NSAC (Maximum PDU session) may be controlled by the NSACF in the VPLMN (e.g. for LBO PDU session), or the NSACF in the HPLMN (e.g. for HR PDU session).

-	EPS interworking: when the user establishes a HR PDN connection at the EPS network and move to 5GS later, the NSACF(Maximum UE number) selected by the SMF+PGW-C and AMF may be different.

This key issue addresses the above cases, to ensure consistent NSAC handling against the configured global Maximum allowed number. The following aspects will be covered by the key issue:

-	UE Registration.

-	PDU Session establishment.

-	Session continuity when UE move across the service area."

This solution extends Solution #13 (Hierarchical NSACF Architecture for Maximum UE/PDU Session number control) with additional capabilities for the Primary NSACF to dynamically re-allocate the global maximum number of registered UEs per network slice and/or the number of PDU Sessions per network slice among the Local NSACFs and revoke some part the local maximum number from an under-utilized NSACF and grant it to another NSACF that is approaching capacity.

# 2 Proposal

It is proposed to update TR 23.700-41 as follows:

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| Solution #1: Additional S-NSSAI associated with the PDU session | X |  |  |  |  |  |
| Solution #2: Slice Re-mapping Capabilities for Network Slice Service Continuity | X |  |  |  |  |  |
| Solution #3: Support of Network Slice Service continuity using SSC mode 3 | X |  |  |  |  |  |
| Solution #4: PDU Session on compatible network slice | X |  |  |  |  |  |
| Solution #5: PDU session handover to a target CN with an alternative S-NSSAI support | X |  |  |  |  |  |
| Solution #6: Extended SoR VPLMN Slice Information transfer to UEs |  | X |  |  |  |  |
| Solution #7: Enabling awareness of Network Slice availability in VPLMNs |  | X |  |  |  |  |
| Solution #8: Gracefully network slice termination |  |  | X |  |  |  |
| Solution #9: Support of a Network Slice with an AoS not matching existing TA boundaries |  |  | X |  |  |  |
| Solution #10: Associating a validity timer with a temporary slice |  |  | X |  |  |  |
| Solution #11: Enabling UEs to Request S-NSSAIs not uniformly available |  |  | X |  | X |  |
| Solution #12: Solution for Centralized Counting for Multiple Service Areas and 5GS-EPS Interworking |  |  |  | X |  |  |
| Solution #13: Hierarchical NSACF Architecture for Maximum UE/PDU Session number control |  |  |  | X |  |  |
| Solution #14: Maximum Number Distribution in multiple NSACFs |  |  |  | X |  |  |
| Solution #X: Re-balancing of the global maximum number of registered UEs/PDU session for the S-NSSAI |  |  |  | X |  |  |

NEXT CHANGE

## 6.X	Solution #X: Re-balancing of the global maximum number of registered UEs/PDU session for the S-NSSAI

### 6.X.1	Functional Description

This solution extends Solution #13 (Hierarchical NSACF Architecture for Maximum UE/PDU Session number control) with additional capabilities for the Primary NSACF to dynamically re-allocate the global maximum number of registered UEs per network slice and/or the number of PDU Sessions per network slice among the Local NSACFs and revoke some part the local maximum number from an under-utilized NSACF and grant it to another NSACF that is approaching capacity.

### 6.X.2	Procedures

#### 6.X.2.1	Registration Procedure

Figure 6.x.2.1-1: NSAC for the maximum number of registered UEs for an S-NSSAI

The enforcement of maximum number of UEs registered for an S-NSSAI is performed as following:

1-5.Same as steps 1-5 in clause 6.13.3.1.

6.	The Primary NSACF performs NSAC for the indicated S-NSSAI. If the update flag parameter from the Local NSACF-1 indicates increase, the Primary NSACF detects the indicated S-NSSAI has reached the global maximum number of registered UEs for the S-NSSAI.

7-8. The primary NSACF queries the other local NSACFs to determine the current number of UEs registered for the S-NSSAI at each of the Local NSACFs. This is performed by invoking a Nnsacf_SliceEventExposure_Request to the other local NSACFs.

9-11.The primary NSACF dynamically reallocates the global maximum number of registered UEs for the S-NSSAI among the local NSACFs in a way that the local maximum number of registered UEs for Local NSACF-1 can be increased. The primary NSACF should only decrease the local maximum number of registered UEs for a Local NSACF if the current number of registered UEs is below a configured usage threshold. The primary NSACF invokes Nnsacf_NSAC_NumberOfUEsUpdate_Request to those Local NSACFs that receive an updated value for the local maximum number of registered UEs.

12.	The Primary NSACF returns the Nnsacf_NSAC_NumberOfUEsUpdate_Response. The Primary NSACF includes the increased local maximum number of registered UEs configuration value.

13.	Same as step 9 in clause 6.13.3.1.

#### 6.X.2.2	Periodic re-balancing of global maximum number of registered UEs for the S-NSSAI

Figure 6.x.2.2-1: Periodic re-balancing of global maximum number of registered UEs for the S-NSSAI

The enforcement of maximum number of UEs registered for an S-NSSAI is performed as following:

1-3.For the primary NSACF to perform a periodic load balancing of the global maximum number of registered UEs for the S-NSSAI, the primary invokes the Nnsacf_SliceEventExposure_Subscribe_Request to each of the local NSACFs. This subscription results for each of the local NSACFs to periodically notify the current number of registered UEs for the S-NSSAI identified in the subscribe request.

4-5.The primary NSACF dynamically reallocates the global maximum number of registered UEs for the S-NSSAI among the local NSACFs so that each local NSACFs can admit approximately an equivalent amount of UEs for the S-NSSAI. The primary NSACF should only decrease the local maximum number of registered UEs for a Local NSACF if the current number of registered UEs is below a configured usage threshold. The primary NSACF invokes Nnsacf_NSAC_NumberOfUEsUpdate_Request to those Local NSACFs that receive an updated value for the local maximum number of registered UEs.

### 6.X.3	Impacts on Existing Nodes and Functionality

NSACF:

- Primary NSACF is able to dynamically re-allocate the global maximum number of registered UEs per network slice and/or the number of PDU Sessions per network slice and assign a decreased local maximum number value for an under-utilized NSACF and an increased local maximum number value for an NSACF that is approaching capacity.

END OF CHANGES