3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204169

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2, New Sol: Ranging service operation with assistant UE

Document for:	Approval

Agenda Item:	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: This contribution introduces a solution for KI#2 Ranging service operation with assistant UE.

1. Introduction/Discussion

This contribution introduces a new solution to for KI#2 Ranging service operation with assistant UE

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change * * * *

6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  | X |
| 2 | X |  |  |  |  |  |  |  |
| 3 |  |  |  | X |  |  |  |  |
| 4 |  |  | X | X |  |  |  |  |
| 5 |  |  | X | X |  |  |  |  |
| 6 |  |  |  |  | X |  |  |  |
| 7 |  |  |  |  | X |  |  |  |
| 8 |  |  |  |  | X |  |  |  |
| 9 |  |  |  |  |  | X |  |  |
| 10 |  |  |  |  |  | X |  |  |
| 11 |  |  |  |  |  | X |  |  |
| 12 |  |  |  |  |  |  | X |  |
| 13 |  |  |  |  |  |  | X |  |
| 14 | X | X | X | X | X | X | X | X |
| X |  | X |  |  |  |  |  |  |

# * * * * Second change * * * *

## 6.X	Solution #X: Solution for ranging operation with assistant UE

### 6.X.1	General

This solution resolves Key Issue #2 for Ranging service operation procedure with the assistance of another UE. In particular, this solution presents the procedure of the ranging operation with the assistant UE.

### 6.X.2	Functional descriptions

In general, this solution proposes the principles in the ranging operation with the assistant UE:

- The direct ranging procedure defined by the solutions for KI#4 can be directly used for Assistant UE to get the relative position of the UE1 and UE2 relative to the Assistant UE respectively.

- UE1 as the initiator UE performs the ranging result calculation based on the relative position of the UE1 and UE2 relative to the Assistant UE.

### 6.X.3	Procedures

Figure 6.X.3-1: high-level procedure for Ranging operation with assistant UE

1.	UE1 and UE2 get the ranging authorization policy from PCF during the registration procedure. The ranging authorization policy may include whether the UE is authorized to perform ranging operation with assistant UE. UE3 (i.e., Assistant UE) gets the ranging authorization policy from PCF during the registration procedure. The ranging authorization policy may include whether the UE is authorized to act as an assistant UE for ranging operation.

2.	When the UE1 gets the ranging request from the application layer, but the direct ranging operation between the UE1 and UE2 cannot be supported, so UE1 will perform assistant UE discovery procedure. Specifically, the UE1 sends a discovery message including the ranging indication and UE2 User Info, and if the UE receiving the message supports as Assistant UE based on the authorization policy in step 1 or its ranging capability, it will generate and send a discovery message including UE2 User Info. After UE2 performs the Assistant UE selection, and responses to the selected Assistant UE.

3.	UE1 sends the ranging request to the UE2, and the ranging request includes Assistant UE ID. Ranging request is sent over PC5 direct connection or UE-to-UE Relay connection.

4.	The UE1 obtains the relative position of the Assistant UE (named as relative position 1), using the direct ranging procedure defined by the solutions for KI#4.

5.	The UE2 obtains the relative position of the Assistant UE (named as relative position 2), using the direct ranging procedure defined by the solutions for KI#4.

6.	UE2 sends the relative position 2 to the UE1.

7.	UE1 performs the ranging result calculation according to the relative position 1 and the relative position 2.

### 6.X.4	Impacts on services, entities, and interfaces

UE:

-	The assistance UE discovery.

-	Interaction for ranging request and ranging result.

# * * * * End of changes * * * *