3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204172

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI #6, KI #7, New Sol: Combine Uu positioning capability and/or Ranging/SL positioning capability for the Ranging/SL positioning service

Document for:	Approval

Agenda Item:	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: this contribution proposes to combine the Uu positioning capability and Ranging/SL positioning capability for the Ranging/SL positioning service.

1. Discussion

For Key Issue #6 for Ranging/Sidelink positioning service exposure to a UE and Key Issue #7 for Ranging/Sidelink Positioning service exposure to Application server and for network assisted sidelink positioning, the current solutions focus on the Ranging/SL positioning on the PC5 interface (e.g. Sol#9). Actually, the LMF can combine the Uu positioning capability and Ranging/Sidelink Positioning capability together to satisfy the Ranging/SL positioning service which may improve the accuracy, enable Ranging/SL positioning service even when one UE lacks Uu/PC5 capabilities, enable Ranging/SL positioning service even when one UE is out-of-coverage, etc.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change * * * *

6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  | X |
| 2 | X |  |  |  |  |  |  |  |
| 3 |  |  |  | X |  |  |  |  |
| 4 |  |  | X | X |  |  |  |  |
| 5 |  |  | X | X |  |  |  |  |
| 6 |  |  |  |  | X |  |  |  |
| 7 |  |  |  |  | X |  |  |  |
| 8 |  |  |  |  | X |  |  |  |
| 9 |  |  |  |  |  | X |  |  |
| 10 |  |  |  |  |  | X |  |  |
| 11 |  |  |  |  |  | X |  |  |
| 12 |  |  |  |  |  |  | X |  |
| 13 |  |  |  |  |  |  | X |  |
| 14 | X | X | X | X | X | X | X | X |
| X |  |  |  |  |  | X | X |  |

# * * * * Second change * * * *

6.x	Solution#x: Combine Uu positioning capability and/or Ranging/SL positioning capability for the Ranging/SL positioning service

6.x.1	General

This solution is related to the Key Issue #6 and Key Issue #7.

6.x.2	Functional descriptions

The LMF combines the Uu positioning capability and Ranging/Sidelink Positioning capability together to satisfy the Ranging/SL positioning service, which may improve the accuracy, enable Ranging/SL positioning service even when one UE lacks Uu/PC5 capabilities, enable Ranging/SL positioning service even when one UE is out-of-coverage, etc.

6.x.3	Procedures

Figure 6.x.3-1: Procedure for the Ranging/SL positioning service

1. 	The AF (Step 1a) or 3rd party UE (Step 1b) requests the network for the Ranging/SL positioning service between the UE1 and UE2. The Ranging/SL positioning requirement may include the service requirement regarding Ranging/SL positioning demand (i.e. the distance and/or the direction between two UEs or more UEs), delay, accuracy, result feedback mode (e.g. immediately, periodically or at a later time).

2. 	When the AMF receives the Ranging/SL positioning service request from UE or AF via GMLC, it sends Ranging/SL positioning Request (UE1 ID, UE2 ID, service requirement) to the LMF. The AMF is the serving AMF of 3rd party UE if the request is sent from the 3rd party UE, or the AMF is the serving AMF of the initiator UE (i.e. UE1/UE2, which is used to perform signalling exchange with the LMF, and the initiator UE may be selected by GMLC or indicated by AF) if the request is sent from AF.

3. 	The LMF determines the appropriate positioning methods based on the service requirements, UE capability (e.g.  whether UE has the Uu capability/PC5 capability), UE current status (e.g. reachable or unreachable, CM-CONNECTED or CM-IDLE) and operator policy. The positioning methods can be Uu positioning, Ranging/SL positioning, or both.

4. 	The LMF initiates the Ranging/SL positioning between UE1 and UE2 if selected.

5a. The LMF initiates Uu positioning for UE2 if selected.

5b. The LMF initiates Uu positioning for UE1 if selected.

6. 	The LMF performs the calculation based on the Ranging/SL positioning result and/or Uu positioning result.

7. 	The LMF sends Ranging/SL positioning Response (Ranging/SL positioning result) to the AMF.

8. 	The AMF returns Ranging/SL positioning Service Response (Ranging/SL positioning result) to the AF (Step 8a) or 3rd party UE (Step 8b).

6.x.4	Impacts on services, entities, and interfaces

UE:

-	impacts in Sol#9 and Sol#12.

NG-RAN:

-	impacts in Sol#9 and Sol#12.

AMF:

-	impacts in Sol#9 and Sol#12.

LMF:

-	impacts in Sol#9 and Sol#12.

-	The LMF determines the appropriate positioning methods based on the service requirements, UE capability (e.g.  whether UE has the Uu capability/PC5 capability), UE current status (e.g. reachable or unreachable, CM-CONNECTED or CM-IDLE) and operator policy.

# * * * * End of changes * * * *