3GPP TSG SA2 Meeting #151E	S2-2204187

# E-Meeting, 16th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3645 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UE behaviour upon mobility between GERAN/UTRAN and 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1; TEI17 |  |  |  |  |  | Date: |  |  | 2022-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1) Currently, regarding the interworking between 5GS and the Circuit Switched domain when the GERAN or UTRAN network is operating in NMO II, it is left to UE implementation whether upon mobility from 5GS to GERAN/UTRAN - the UE (always) performs a location update, or - "act(s) as if it is returning after a loss of GERAN/UTRAN coverage (and e.g. only perform(s) a periodic LAU if the periodic LAU timer has expired)". However, if the UE is registered for IMS services and the UE returns to the location area in GERAN/UTRAN where it was registered before, and the UE does not initiate a new location update procedure, then the T-ADS as specified in TS 23.221, clause 7.2b, does not work properly. I.e., the network will erroneously assume that the UE is still camping on NG-RAN where it had the most recent radio contact to the 3GPP network, and it will try to deliver the MT session via that access network. (Note that the UDM cannot rely on the UE performing a GPRS attach and thus informing the network about the RAT change. E.g., the PS domain in GERAN/UTRAN could be temporarily unavailable for some reason.) 2) For the reverse direction, mobility from GERAN/UTRAN to 5GS, there is a requirement to perform an initial registration. In our understanding, if the UE was registered in 5GS before it moved from 5GS to GERAN/UTRAN and while in GERAN/UTRAN the UE performed a location update, but not a GPRS Attach procedure, then it is sufficient to perform a mobility registration update procedure. A UE registered for IMS services needs to initiate this mobility registration update procedure to enable T-ADS, even if the UE returns to the tracking area in 5GS where it was registered before. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the UE is registered for IMS services, then it shall perform a Location Update upon mobility from 5GS to GERAN/UTRAN and a Registration procedure upon mobility from GERAN/UTRAN to 5GS to enable T-ADS as specified in TS 23.221 [23]. The registration type depends on whether the UE was registered in 5GS before the mobility from 5GS to GERAN/UTRAN and whether it performed a GPRS Attach procedure while in GERAN/UTRAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Based on the current wording, a UE may perform mobility between CS GERAN/UTRAN and 5GS without initiating a location update or registration procedure so that T-ADS is not working properly and terminating sessions cannot be delivered to the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.17.2.4	Mobility between 5GS and GERAN/UTRAN

IP address preservation upon direct mobility between 5GS and GERAN/UTRAN is not supported.

Upon mobility from 5GS to GERAN/UTRAN (e.g. upon leaving NG-RAN coverage) the UE shall perform the A/Gb mode GPRS Attach procedure or Iu mode GPRS Attach procedure (see TS 23.060 [56]).

As a UE option, to support IP address preservation at mobility from EPC to 5GS for PDN connections without 5GS related parameters, a 5GS capable UE may:

-	Following mobility from GERAN/UTRAN to EPS, release those PDN connection(s) and re-establish them as specified in clause 4.11.1.5.4.1 of TS 23.502 [3] so that they support interworking to 5GS.

NOTE 1:	It is recommended that a UE using this option does not do this behaviour after every change to EPS in PLMNs that do not support 5GS, nor for APNs that do not support mobility to 5GS; and, that such a UE supports storage of the 5GS related parameters while in GERAN/UTRAN. Whether and how the UE is aware of which PLMNs support 5GS and which APNs do not support mobility to 5GS is out of scope of this specification.

To support mobility from EPC to 5GS to EPC to GERAN/UTRAN for PDN connections established in EPC:

NOTE 2:	For the use of N7 or N40 interfaces while the UE is in GERAN/UTRAN access, the SMF+PGW-C selected by the MME (using the existing selection procedures described in clause 4.11.0a of TS 23.502 [3] and clause 4.3.8 of TS 23.401 [26]) needs to support functionality (e.g. signalling of GERAN/UTRAN cell identification over N7) specified in Annex L.

-	in signalling sent on the N26 interface, the MME should send the TI and BSS Container in the EPS Bearer Context (see Table 7.3.1-3 of TS 29.274 [101]), if there is any, of the EPS bearer to the SMF (V-SMF / I-SMF) via the AMF in the Bearer Context within the PDN Connection IE in the Forward Relocation Request and Context Response messages (TS 29.274 [101]); the SMF (V-SMF / I-SMF) should store the TI and BSS Container and the SMF (V-SMF / I-SMF) should provide the TI and BSS Container to the AMF (as part of a procedure to deliver SM context to AMF) so that the AMF sends the TI and BSS Container of the related EPS bearer in the Bearer Context within the EPS PDN Connection information in any subsequent Forward Relocation Request and Context Response message sent to an MME.

NOTE 3:	At mobility from EPC, the SMF+PGW-C / V-SMF / I-SMF receives the TI and BSS Container as part of the UE EPS PDN Connection information from the AMF and stores the TI. At mobility to EPC, the SMF+PGW-C / V-SMF / I-SMF provides the AMF with the TI and BSS Container as part of the UE EPS PDN Connection information. The SMF+PGW-C / V-SMF / I-SMF is not meant to understand the TI/BSS Container nor to use it for any other purpose than providing it back to AMF.

NOTE 4:	GERAN/UTRAN Mobility Management Bearer Synchronisation procedures will release any dedicated QoS Flows established in 5GS.

NOTE 5:	When the UE access the network via GERAN/UTRAN over Gn/Gp interface, Secondary PDP Context Activation Procedure is not supported.

IP address preservation at mobility from EPC to GERAN/UTRAN for PDU sessions established in 5GS is not supported.

With regard to interworking between 5GS and the Circuit Switched domain when the GERAN or UTRAN network is operating in NMO II (i.e. no Gs interface between MSC and SGSN):

-	upon mobility from 5GS to GERAN/UTRAN, the UE shall either:

-	act as if it is returning after a loss of GERAN/UTRAN coverage (and e.g. only perform a periodic LAU if the periodic LAU timer has expired), or,

-	perform a Location Update to the MSC.

-	If the UE is registered for IMS services, then it shall perform a Location Update upon mobility from 5GS to GERAN/UTRAN and a Registration procedure upon mobility from GERAN/UTRAN to 5GS to enable T-ADS as specified in TS 23.221 [23]. The UE sets the Registration Type to Mobility Registration Update, if the UE was registered in 5GS before the mobility from 5GS to GERAN/UTRAN and the UE did not perform a GPRS Attach procedure while in GERAN/UTRAN; otherwise, it sets the Registration Type to Initial Registration.

If the UE performed a GPRS Attach or Routing Area Update procedure while in GERAN/UTRAN (see TS 23.060 [56]), upon mobility from GERAN/UTRAN to 5GS (e.g. upon selecting an NG-RAN cell) the UE shall perform the Registration procedure of "initial registration" type as described in TS 23.502 [3]. The UE shall indicate a 5G-GUTI as UE identity in the Registration procedure if it has a stored valid native 5G-GUTI (e.g. from an earlier registration in the 5G System). Otherwise the UE shall indicate a SUCI.

If a UE in MICO mode moves to GERAN/UTRAN and any of the triggers defined in clause 5.4.1.3 occur, then the UE shall locally disable MICO mode and perform the A/Gb mode GPRS Attach procedure or Iu mode GPRS Attach procedure (see TS 23.060 [56]). The UE can renegotiate MICO when it returns to 5GS during (re-)registration procedure.

In Single Registration mode, expiry of the periodic RAU timer, or, the periodic LAU timer shall not cause the UE to change RAT.

The 5G SRVCC from NG-RAN to UTRAN is specified in the TS 23.216 [88]. After the 5G SRVCC to UTRAN, all the PDU sessions of the UE are released.