SA WG2 Meeting #151e	S2-2204213

# May 16th - 20th, 2022; Elbonia               	                  (revision of S2-220xxxx)

Source: 	China Unicom

Title: 	KI#2, Sol#8: Update to remove ENs

Document for: 	Approval

Agenda Item: 	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: This paper proposes to update solution#8 for resolving ENs.

1	Discussion

This p-CR aims to resolve the following ENs:

Editor's note: User consent is needed for URSP enforcement reporting and the detailed descriptions and procedures of user consent is FFS.

Editor's note: Whether existing Npcf services (e.g. Npcf_UEPolicyControl service or Npcf_SMPolicyControl service) can be reused for URSP enforcement reporting is FFS.

Editor's note: What further actions will be taken by the network after URSP enforcement reporting is FFS.

2 Proposal

It is proposed to update TR 23.700-85 on FS_eUEPO as follows:

*** BEGIN CHANGES ***

6.8	Solution #8: PSI reporting for awareness of URSP enforcement

6.8.1	Description

This solution aims to address the Key Issue#2: 5GC awareness of URSP enforcement.

The URSP rules sent to the UE are contained in a Policy Clause identified by a PSI (Policy Clause Identifier), and each Policy Clause may include multiple URSP rules. For 5GC awareness of URSP enforcement, this solution proposes to use the parameter of Rule Precedence to identify a specific URSP rule within a Policy Section.

The UE can indicate to the network the URSP rule it is trying to enforce by reporting the combination of PSI + Rule Precedence. When an existing PDU session matches the RSD of selected URSP, or when the UE tries to establish a new PDU Session using the values specified by the selected RSD, the URSP enforcement reporting process can be done during a session management procedures such as PDU Session Establishment procedure or PDU Session Modification procedure.

To protect user privacy, the URSP enforcement reporting process should be subject to user consent. The user consent is obtained by the ASP and provided to the MNO. The information of user consent for URSP enforcement reporting is included per URSP rules and provisioned to the UE. The UE reports the combination of PSI + Rule Precedence of the URSP rule it is trying to enforce only when the user consent part of the URSP rule is "YES".

6.8.2	Procedures

As described in TS 23.503 [4], for every newly detected application the UE evaluates the URSP rules in the order of Rule Precedence and determines if the application is matching the Traffic descriptor of any URSP rule. When a valid Route Selection Descriptor is found, the UE determines if there is an existing PDU Session that matches all components in the selected Route Selection Descriptor.

If one or more matching PDU Session(s) exist(s), the UE associates the application to one existing PDU Session and initiates PDU Session Modification procedure for URSP enforcement reporting as described in clause 4.3.3.2 TS 23.502 [3]. Following impacts are applicable:

-	Step 1a: The UE includes UE Policy Container (the combination of PSI + Rule Precedence corresponding to the URSP rule enforced by the UE, and the operating system identifier) in PDU Session Modification Request message. The AMF transfers the message transparently to the SMF via Nsmf_PDUSession_UpdateSMContext message.

-	Step 2: After receiving UE Policy Container in the PDU Session Modification Request message, the SMF notifies the PCF the combination of PSI + Rule Precedence reported by the UE using Npcf_SMPolicyControl_Update service operation. The PCF returns the URSP rule indicated by the combination of PSI + Rule Precedence to the SMF.

The SMF compares the RSD of the URSP rule received from the PCF with the PDU session attributes (e.g. DNN, S-NSSAI, etc) of the PDU session the UE trying to associate to. If it is not matched, the SMF rejects the PDU Session Modification request with a proper cause value.

-	Step 2a/2b: The SMF generates N4 rules (PDR, URR) based on the Traffic Descriptor of the URSP rule received from the PCF and provisions the rules to the UPF using N4 Session Modification procedure. The SMF indicates the UPF to report the event when incorrect application traffic is detected for the QoS Flow indicated in the PDR.

When incorrect application traffic is detected, the UPF should block the corresponding application traffic and send a report to the SMF. The SMF may further inform to the UE that the URSP rule is wrongly enforced.

NOTE:	It is assumed that same PCF is selected by the SMF and the AMF.

If none of the existing PDU Sessions matches the selected Route Selection Descriptor, the UE tries to establish a new PDU Session as described in clause 4.3.2.2.1 TS 23.502 [3] using the values specified by the selected RSD. Following impacts are applicable for URSP enforcement reporting:

-	Step 1: The UE includes UE Policy Container (the combination of PSI + Rule Precedence corresponding to the URSP enforced by the UE, and the operating system identifier) in PDU Session Establishment Request message.

-	Step 3: The AMF transfers the PDU Session Establishment Request (UE Policy Container (the combination of PSI + Rule Precedence corresponding to the URSP rule enforced by the UE, and the operating system identifier)) to the SMF by Nsmf_PDUSession_CreateSMContext Request message.

-	Step 7b: After receiving UE Policy Container in the PDU Session Establishment Request message, the SMF notifies the PCF the combination of PSI + Rule Precedence reported by the UE using Npcf_SMPolicyControl_Create service operation. The PCF returns the URSP rule indicated by the combination of PSI + Rule Precedence to the SMF.

The SMF compares the RSD of the URSP rule received from the PCF with the PDU session attributes (e.g. DNN, S-NSSAI, etc) of the PDU session the UE trying to establish. If it is not matched, the SMF rejects the PDU Session Establishment request with a proper cause value.

-	Step 10a/10b: The SMF generates N4 rules (PDR, URR) based on the Traffic Descriptor of the URSP rule received from the PCF and provisions the rules to the UPF using N4 Session Establishment procedure. The SMF indicates the UPF to report the event when incorrect application traffic is detected for the QoS Flow indicated in the PDR.

When incorrect application traffic is detected, the UPF should block the corresponding application traffic and send a report to the SMF. The SMF may further inform to the UE that the URSP rule is wrongly enforced. If the QoS Flow is the only or the last QoS Flow of the PDU Session, the SMF should release the PDU session with a proper cause value.

6.8.3	Impacts on services, entities and interfaces

UE:

-	Initiates PDU Session Modification procedure when matching PDU Session is found for a URSP rule, and includes UE Policy Container (the combination of PSI + Rule Precedence corresponding to the URSP enforced by the UE, and the operating system identifier) in PDU Session Modification Request message.

-	During the new PDU Session Establishment procedure when none of the existing PDU Sessions matches the selected Route Selection Descriptor, includes UE Policy Container (the combination of PSI + Rule Precedence corresponding to the URSP enforced by the UE, and the operating system identifier) in PDU Session Establishment Request message.

SMF:

-	Forwards the combination of PSI + Rule Precedence reported by the UE to the PCF.

-	Compares the RSD of the URSP rule received from the PCF with the attributes of the PDU session the UE trying to establish or associate to.

-	Generates N4 rules (PDR, URR) based on the Traffic Descriptor of the URSP rule received from the PCF.

-	Indicates the UPF to report the event when incorrect application traffic is detected for the QoS Flow indicated in the PDR.

PCF:

-	Based on the combination of PSI + Rule Precedence received from the SMF, provides corresponding URSP rule to the SMF.

UPF:

-	Detects application traffic and reports to the SMF when incorrect application traffic is detected.

*** END CHANGES ***