TSG SA WG2 Meeting #151E	S2-2204214

16 - 20, May, 2022, Electronic meeting

Source:	Lenovo

Title:	Provision XR traffic configuration to 5GS

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM/ Rel-18

Abstract of the contribution: update of solution 26

1	Background

In TR 23.700-60[1], Solution 26 is approved for provision XR traffic configuration to 5GS, which maps to Key Issue#4 and #5. It is stated that, AF provisions configuration consisting of flow description and the associated XR traffic configuration information as follows,

-	Time assistance information: Burst Arrival Time, Periodicity

-	PDU set based QoS requirement: PDU set delay budget, PDU set error rate

-	Frame type/importance

However, it only applies for AF provides 5GS the flow description and the associated specific frame type/importance over control plane. Besides, it does not mention how to trigger UPF to perform PDU set identification and mark the GTP-U header with PDU set information. In this paper, the remaining issues are addressed.

2	Discussion

2.1 two use cases of AF provision XR traffic configuration to 5GS

Solution 26 is based on the assumption that different types of PDU set (e.g., frame type/importance) are classified into different QoS flows. There’re two possible ways for XR App server to provide packets with different frame type/importance (e.g., different types of PDU sets), in one single IP packet flow or in separate IP packet flows. In Figure 1, XR App server provides different IP packet flows associated with specific frame type/importance. In Figure 2, XR App server provides one IP packet flow with different frame types/importance. UPF splits one IP packet flow into different QoS flows based on specific frame type/importance.

Case 1: XR App server provides different IP packet flows associated with specific frame type/importance

In this case, XR application server identifies packets with different frame type/importance and marks the IP header with different Type of Service (IPv4), or traffic class (IPv6) and mark, or IPv6 Flow Label.

Figure 1. XR App server provides different IP packet flows associated with specific frame type/importance

AF provides 5GC flow description with the associated frame type/importance, which will further be contained in PCC rules provided to SMF. SMF provides UPF different IP packet filters with the associated frame type/importance and QFI. In this way, UPF is able to identify frame type/importance based on the IP packet filter and marks the packets with the associated QFI. Meanwhile, SMF provides RAN node QFI with the associated frame type/importance. RAN node maps different QoS flows to different DRBs, in order to reuse the existing RAN scheduling mechanism to realize the prioritized transmission of the XRM data.

Case 2: XR App server provides one IP packet flow with different frame types/importance

In this case, XR App server provides one IP packet flow, which contains packets of different frame types/importance. UPF splits one IP packet flow into different QoS flows based on specific frame type/importance identified in user plane.

Figure 2. XR App server provides one IP packet flow with different frame types/importance

One possible solution is, AF provides XR configuration information, which contains flow description and the possible values of frame type/importance. SMF provides UPF PDR together with the mapping relationship between QFI and frame type/importance. Specifically, frame type/importance can be in the form of 1) I/D field in RTP header extension or 2)TID/LID field in RTP header extension or 3)NRI value, PRID/DID filed in NAL unit header format as mentioned in other solutions. In TR 23.700-60, some solutions also mentioned that UPF is also able to identify frame type/importance based on assistant information from AF or traffic pattern.

2.2 indication of PDU set integrated handling

In TR 23.700-60, some solutions propose to consider of whether all PDUs are needed for the usage of PDU Set by application layer. That is, AF should provide the indication of PDU set integrated handling with the associated flow description if application layer expects all PDUs of one PDU set. However, different frame type/importance may have different treatments. E.g., for I-frame or frame with high importance, all PDUs of one PDU set are needed in application layer. While for P-frame or frame with low importance, not all PDUs of one PDU set are needed in application layer. Therefore, the indication of PDU set integrated handling for different frame type/importance may be different. AF shall provide the indication of PDU set integrated handling with the associated frame type/importance.

2.3 PDU set identification

Upon receiving XR traffic configuration from PCF, SMF shall sends UPF the packet filter set and the PDU set identification indication if the indication of PDU set integrated handling is true. The PDU set identification indication can be used to indicate UPF to perform PDU set identification and mark the GTP-U header with PDU set information.

3	Reference

[1] 3GPP TR 23.700-60, 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects;

Study on XR (Extended Reality) and media services (Release 18)

4	Proposal

* * * * Start of Change * * * *

## 6.26	Solution #26: Provisioning XR traffic configuration to 5GS

### 6.26.1	Key Issue mapping

This solution is for Key Issue #4 and #5, which addresses PDU set integrated packet handling and differentiated QoS handling.

### 6.26.2	Description

In order to enable 5GS to perform PDU set integrated packet handling and differentiatd QoS handling, AF provisions configuration consisting of flow description and the associated XR traffic configuration information.

There’re two possible ways for XR App server to provide packets with different frame type/importance (e.g., different types of PDU sets), in one single IP packet flow or in separate IP packet flows. In Figure 6.26-1, XR App server provides different IP packet flows associated with specific frame type/importance. In Figure 6.26-2, XR App server provides one IP packet flow with different frame types/importance. UPF splits one IP packet flow into different QoS flows based on specific frame type/importance.

Case 1: XR App server provides different IP packet flows associated with specific frame type/importance

In this case, XR application server identifies packets with different frame type/importance and marks the IP header with different Type of Service (IPv4), or traffic class (IPv6) and mark, or IPv6 Flow Label.

Figure 6.26-1. XR App server provides different IP packet flows associated with specific frame type/importance

AF provides 5GC flow description with the associated frame type/importance, which will further be contained in PCC rules provided to SMF. SMF provides UPF different IP packet filters with the associated frame type/importance and QFI. In this way, UPF is able to identify frame type/importance based on the IP packet filter and marks the packets with the associated QFI. Meanwhile, SMF provides RAN node QFI with the associated frame type/importance. RAN node maps different QoS flows to different DRBs, in order to reuse the existing RAN scheduling mechanism to realize the prioritized transmission of the XRM data.

Case 2: XR App server provides one IP packet flow with different frame types/importance

In this case, XR App server provides one IP packet flow, which contains packets of different frame types/importance. UPF splits one IP packet flow into different QoS flows based on specific frame type/importance identified in user plane.

Figure 6.26-2. XR App server provides one IP packet flow with different frame types/importance

One possible solution is, AF provides XR configuration information, which contains flow description and the possible values of frame type/importance. SMF provides UPF PDR together with the mapping relationship between QFI and frame type/importance. Specifically, frame type/importance can be in the form of 1) I/D field in RTP header extension or 2)TID/LID field in RTP header extension or 3)NRI value, PRID/DID filed in NAL unit header format as mentioned in other solutions. In TR 23.700-60, some solutions also mentioned that UPF is also able to identify frame type/importance based on assistant information from AF or traffic pattern.

In short, the XR configuration information provided flow description and the associated parameters as follows,

-	Time assistance information: Burst Arrival Time, Periodicity

-	PDU set based QoS requirement: PDU set delay budget, PDU set error rate

-	Frame type/importance (for case 1)

-  Possible values of frame type/importance (for case 2)

-  indication of PDU set integrated packet handling, which may be associated with frame type/importance for case 2

Where PDU set delay budget is defined as an upper bound for the time that a PDU Set may be delayed between the UE the N6 termination point at the UPF. PDU set error rate is defiend as an upper bound for the rate of PDU sets (e.g. set of IP packets constituting a PDU-Set) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Possible values of frame type/importance may be provided for case 2. Indication of PDU set integrated packet handling may be associated with flow description, and even with speicific frame type/importance.

PCF initiates SM Policy Association Modification to provide the service data flow fiter and the associated XR traffic configuration information. After that, the solution relies on the PDU Session Modification Procedure in clause 4.3.3 in TS 23.502 [3] for SMF to provision the XR traffic configuration to UPF and RAN node.

### 6.26.3	Procedures

#### 6.26.3.1	PDU session modification

Figure 6.26.3.1-1: PDU session modification procedure

0.	AF sends AF request with flow description and the associated XR traffic configuration information, where the XR traffic configuration information includes time assistance information (e.g., burst arrival time, periodicity), PDU set based QoS requirement (e.g., PDU set delay budget, PDU set error rate), frame type/importance or possible values of frame types/importance, indication of PDU set integrated packet handling and optionally the associated frame types/importance.

1.	PCF initiates SM Policy Association Modification procedure as defined in TS 23.502 clause 4.16.5.2 to notify SMF about the modification of policies, which further triggers PDU session modification procedure. PCF includes service data flow filter with the associated XR traffic configuration information in the PCC rules provided to SMF.

2a. SMF sends N4 Session Modification Request to UPF, which contains PDU set identification indication, packet filter set and the associated QFI and time assistance information. Besides, the mapping of QFI and frame type/importance may also be provided for case 2. The PDU set identification indication can be used to indicate UPF to perform PDU set identification and mark the GTP-U header with PDU set information.

2b. UPF responds SMF with N4 Session Modification Response.

3.	SMF invokes Namf_Communication_N1N2MessageTransfer to transfer N2 SM information.

4.	AMF transfers N2 SM information to RAN node, which contains the mapping of QFI, frame type/importance, time assistance information and PDU set based QoS requirement.

5-12 steps of PDU session Modification procedure are performed as in clause 4.3.3.2 in TS 23.502.

### 6.26.4	Impacts on services, entities and interfaces

The proposed solution is based on Rel-17 procedures but some enhancements may be needed to make it possible for the 5GS to configure UPF/RAN node with the XR traffic configuration information, such as:

# AF：

-	Include XR traffic configuration information associated with flow description in AF request

# PCF:

-	Include XR traffic configuration information associated with service data flow filter in PCC rules

# SMF：

-	Include XR traffic configuration information associated with packet filter set in N4 rules for UPF

-	Include XR traffic configuration information associated with QFI in N2 SM information for RAN node

* * * * End of Change * * * *