TSG SA WG2 Meeting #151E	S2-2204219

16 - 20, May, 2022, Electronic meeting

Source:	Lenovo

Title:	Clarification on traffic characteristics of XR and media services

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM/ Rel-18

Abstract of the contribution: clarifies that traffic characteristics in TR 23.700-60 Annex A are not universally applicable for all XR applications unless application indicates so

1	Introduction

During SA4#118 meeting, SA4 sent out the LS [1] Reply on QoS support with PDU Set granularity. In the LS, SA4 pointed out that, it is difficult and likely impossible to identify common “traffic characteristics” since the traffic characteristics heavily depend on the application choices, such as the application, the codec in use, the data formats, the encoding operation (bitrate control, usage of slices, error resilience such as intra frames, Gradual Decoder Refresh (GDR), or long-term prediction, etc.). That is, the traffic characteristics identified in TR 23700-60 Annex are not universally applicable for all XR applications unless the application indicates so.

2	Reference

[1] S4-220505, LS Reply on QoS support with PDU Set granularity, FS_XRM, FS_XRTraffic

3	Proposal

It is proposed to agree the following changes for TR 23.700-60 Annex A :

* * * * Start of Change * * * *

Annex A:
Traffic characteristics of XR and media services

It is difficult and likely impossible to identify common “traffic characteristics” since the traffic characteristics heavily depend on the application choices, such as the application, the codec in use, the data formats, the encoding operation etc. Therefore, the following traffic characteristics are not universally applicable for all XR applications unless the application indicates so.

# A.1	Frame and GOP

A video consists of a sequence of consecutive video frames. Each video frame is a picture encoded/compressed using different codec mechanisms (e.g. H.264/H.265/H.266, AV1 or AVS) for efficient storage and transmission. During encoding, the video spatial and temporal redundancy can be removed using, e.g. inter-frame/intra-frame prediction, motion estimation/compensation. Typically, three major frame/picture types can be used during encoding, i.e. I-frame, P-frame and B-frame:

-	I-frame, as an intra-coded picture, is a complete picture and can be encoded and decoded independently, like a JPG image file.

-	P-frame, as a predicted picture, is not a complete frame and only contains the image changes compared to the previous frame. If the reference frame is lost, the P-frame cannot be decoded and displayed.

-	B-frame, as a bidirectional predicted picture, contains the changes between the previous and following reference frames. With more reference frames, the compression ratio can be higher. However, the B-frame can only be decoded when the previous and following reference frames are available.

A Group of Pictures (GOP) includes a collection of successive video frames. The first frame in a GOP is an I-frame. And the following frames can be P-frames or B-frame.

# A.2	Other traffic characteristics

# 1)	Periodic and jitter

Data burst of video frame are always periodic with variable packet size. For example, audio frame ususally have a periodicity of 20ms, while video stream can have different frame rate, e.g. 60/90/120 fps. Besides, pose/control packets are always periodic (e.g. 4ms) with fixed packet size. Due to the unpredictable jitter, the packet arrival time might not be exacly periodic, which leads to transmission delay or packet loss. 5GS should take both periodicity and jitter into consideration for XR and media service.

# 2)	Frame importance

For XR and media service, different types of frame coexist and have different importance. For example, in Group of Picture (GOP), the first I-frame is the most important frame, other P-frames are encoded based on the I-frame. That is, the decoding of P-frames depends on the decoding of I-frame in the same GOP. Therefore, frame importance should be provided to enable 5GS to perform efficient packet dropping, e.g. the non-transmitted part with low importance of GOP will be discarded at the transmitter if the packets with high importance of the GOP have been lost or exceed the delay budget.

# 3)	PDU set granularity

XR applications impose requirements in terms of PDU sets(e.g. video/audio frame/tile, Application Data Units etc.), rather than in terms of single packets/PDUs. Packets of one PDU set need to be jointly processed for XR traffics. 5G system should be aware of the PDU set in order to improve transmission performance. Basically, the PDU set SN, the packet SN within the PDU set, the total packet number of the PDU set or the last packet indication should be provided to 5G system for each packet.

# 4)	Traffic correlation

Some advanced XR or media services may include multiple traffic streams such as video, audio, haptic data or sensor data etc. which are called multi-modality communication. Different traffic streams with different QoS parameters are always mapped to different QoS flows. Packets within the same traffic stream with different packet characteristics (e.g. packet importance, frame type, etc.) may also be classified and mapped into different QoS flows. These QoS flows belong to the same XR and media service have strong correlations, which should be coordinated for QoS requirement, packet processing and time synchronization etc.

* * * * End of Change * * * *