3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204258

Elbonia, May 16 – 20, 2022

Source:	Huawei, HiSilicon

Title:	KI#4, New Sol: External parameter provisioning by AF for AI/ML data transport

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys/ Rel-18

Abstract: In order to assist the Application AI/ML operation, the 5GC maintains and uses the external parameters provisioned by AF to decide the policy to transport the Application AI/ML traffic(s).

1. Introduction/Discussion

According to TS 22.261, the characteristics of AI/ML model data is different from the other Application AI/ML traffic, and different policy e.g. QoS, may be needed for the transport of the different kinds of AI/ML data. Since the AF knows well the characteristics of the AI/ML data to be transported, and the AF can provision specific policy parameters to the 5GS to support the transport of the AI/ML data.

The transport of the AI/ML model happens discontinuously and has less strict end-to-end latency requirements, and the AF can provision the transport policy by setting a policy for a future AF session procedure as for background data transfer as defined in clause 4.15.6.8 of the TS 23.502 for such AI/ML model distribution to a large amount of UEs. For model transport to a specific UE or model distribution to a small amount of UEs, AF session with required QoS update procedure as defined in clause 4.15.6.6a of the TS 23.502 is more appropriate to be used by the AF to provision the transport policy. Other Application AI/ML traffics, such as intermediate data, inference results, or model performance data, are continuous and have relatively stringent latency requirement, and are more suitable to be transported with specific QoS provisioned by setting up an AF session with required QoS procedure as defined in clause 4.15.6.6 of the TS 23.502.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-80 v0.2.0.

# * * * * First change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | X |  |  |  |  | X |  |
| 2 |  | X |  |  |  |  |  |
| 3 |  | X |  |  |  |  |  |
| 4 |  | X |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |
| 6 |  |  | X |  |  |  | X |
| 7 |  |  | X |  |  | X |  |
| 8 |  | X | X |  |  |  |  |
| 9 |  |  |  | X |  |  |  |
| 10 |  |  |  |  | X |  |  |
| 11 | X |  |  |  | X |  |  |
| 12 |  |  |  |  | X |  |  |
| 13 |  |  |  |  | X |  |  |
| 14 |  |  |  |  |  | X | X |
| 15 |  |  |  | X |  | X |  |
| 16 | X |  |  |  | X | X | X |
| 17 |  |  |  |  |  |  | X |
| 18 |  |  |  |  |  |  | X |
| 19 |  |  |  |  |  |  | X |
| 20 |  |  |  |  |  |  | X |
| 21 |  |  |  |  |  |  | X |
| 22 |  |  |  |  |  |  | X |
| 23 |  |  |  |  |  |  | X |
| 24 |  |  |  |  |  |  | X |
| 25 |  |  |  |  |  |  | X |
| 26 |  |  |  |  |  | X | X |
| 27 |  |  |  |  |  |  | X |
| X |  |  |  | X |  |  |  |

# * * * * Second change * * * *

6.X	Solution #X: External parameter provisioning by AF for AI/ML data transport

6.X.1	Description

This solution addresses KI#4.

The AI/ML model, intermediate data, inference results, or model performance data at the UE are transported by 5GS for the Application AI/ML operation. In order to assist the Application AI/ML operation, the 5GC can maintain and use the external parameters provisioned by AF to decide the policy to transport the Application AI/ML traffic(s).

According to TS 22.261, the characteristics of AI/ML model data is different from the other Application AI/ML traffic, and different policy e.g. QoS, may be needed for the transport of the different kinds of AI/ML data. Since the AF knows well the characteristics of the AI/ML data to be transported, and the AF can provision specific policy parameters to the 5GS to support the transport of the AI/ML data.

The transport of the AI/ML model happens discontinuously and has less strict end-to-end latency requirements, and the AF can provision the transport policy by setting a policy for a future AF session procedure as for background data transfer as defined in clause 4.15.6.8 of the TS 23.502 for such AI/ML model distribution to a large amount of UEs. For model transport to a specific UE or model distribution to a small amount of UEs, AF session with required QoS update procedure as defined in clause 4.15.6.6a of the TS 23.502 is more appropriate to be used by the AF to provision the transport policy. Other Application AI/ML traffics, such as intermediate data, inference results, or model performance data, are continuous and have relatively stringent latency requirement, and are more suitable to be transported with specific QoS provisioned by setting up an AF session with required QoS procedure as defined in clause 4.15.6.6 of the TS 23.502.

6.X.2	Procedures

When the Application AI/ML begins to transport intermediate data, local training data, inference results, or model performance data through 5GS, the AF sends Nnef_AFsessionWithQoS_Create request message as described in clause 4.15.6.6 of the TS 23.502 [4], including the specific QoS parameters for the AF session. The QoS parameters include the service requirements described in clause 7.10 of TS 22.261 [2].

For AI/ML model transmission to a specific UE or model distribution to a small amount of UEs, the AF may update the QoS request for the AF session by sending Nnef_AFsessionWithQoS_Update request message as described in clause 4.15.6.6a of the TS 23.502 [4], including the Flow Description(s) and QoS parameters for the model transmission or distribution.

When distributing the AI/ML model to large-scale UEs, the AF schedules the model distribution by invoking the Nnef_ApplyPolicy_Create service operation as described in clause 4.15.6.8 of the TS 23.502 [4], to negotiate a policy for background data transfer with the service requirements described in clause 7.10 of TS 22.261 [2].

6.X.3	Impacts on services, entities and interfaces

No impact.

# * * * * End of changes * * * *