3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2204305

Elbonia, May 16 – 20, 2022	(revision of S2-22xxxxx)

Source:	Huawei, HiSilicon

Title: 	KI#1/KI#2, Update Sol#8: AF request predefined SFC for traffic flow(s) related with target UE(s)

Document for: 	Approval

Agenda Item: 	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution: This paper tries to resolved open issue in solution 8.

1 Discussion

There are two open issues left in the solution#8.

- It is FFS how the 5GC selects the UPF with the appropriate predefined SFPs corresponding to the AF request of the 3rd party. The operator can have SLAs with multiple 3rd parties and the predefined SFPs for distinct 3rd parties can be deployed in distinct UPFs.

It is possible that not all the UPFs support the pre-defined SFPs corresponding to the AF request of the 3rd party. If the current PSA UPF doesn’t support the pre-defined SFPs corresponding to the AF request of the 3rd party, based on the operator’s policy the SMF insert or reselect an I-UPF/L-PSA which supports the pre-defined SFC corresponding to the AF request of the 3rd party.

- The handling that the N6-LAN traffic steering control and AF-influenced traffic steering control are both applied to the traffic is FFS.

It is possible that N6-LAN traffic steering control and AF-influenced traffic steering control are both applied to the same application traffic simultaneously, e.g. the AF may request pre-processing performed by the SFC before the traffic is routed to the application server deployed in the local network or for two different directions. The SMF needs select a UPF which supports both N6-LAN traffic steering enforcement control and AF-influenced traffic steering control. If the SMF can’t find such kind of UPF, the SMF will inform the AF two traffic steering control cannot be supported simultaneously. In this case, the AF can select one and return it to SMF. The SMF then enforce it.

If the N6-LAN traffic steering enforcement control in downlink is applied when the AF-influenced traffic steering enforcement control is applied, the PCF include N6-LAN traffic steering policy identifier in downlink, and AF-influenced traffic steering policy identifier or N6 traffic routing information per DNAI within a PCC rule. In this case, the SMF generates two PDR and FAR pairs. One PDR and FAR pair is associated the downlink N6-LAN traffic steering policy handing. The other PDR and FAR is associated with the uplink AF influenced traffic steering policy handling. In order to do this, only the NOTE 18 in table 6.3.1 of TS23.503 needs to be removed.

If the N6-LAN traffic steering enforcement control in uplink is applied when the AF-influenced traffic steering enforcement control is applied, the PCF shall include N6-LAN traffic steering policy identifier in uplink and AF-influenced traffic steering policy identifier or N6 traffic routing information per DNAI within a PCC rule. The SMF generates two PDR and FAR pairs. The first PDR and FAR pair is associated the N6-LAN traffic steering policy handing. The second PDR and FAR is associated with the AF influenced traffic steering policy handling for traffic after N6-LAN traffic steering handling In order to do this, the NOTE 18 in table 6.3.1 of TS23.503 needs to be removed, and the SMF is enhanced to instruct the UPF to steer the traffic based on N6-LAN traffic steering policy identifier in uplink and AF-influenced traffic steering policy identifier in an order.

2 Proposal

It is proposed to update 23.700-18 as follows.

FIRST CHANGE

6.8	Solution #8: AF request predefined SFC for traffic flow(s) related with target UE(s)

6.8.1	Description

The AF-influenced traffic steering enforcement control information is used to influence SMF routeing decisions for an identified traffic of PDU Session. It steers the user traffic to a local access to a Data Network (identified by a DNAI) per AF request. The AF-influenced traffic steering enforcement control information can be derived by the PCF based on the N6 Traffic Routing requirements provided by the AF. Until Rel-17 this is supported in non-roaming scenarios and to LBO deployments only

The N6-LAN traffic steering enforcement control steer the identified traffic to appropriate operator 3rd party service functions in the N6-LAN. When the PCF generate the PCC rule for the SMF, the N6-LAN traffic steering enforcement control information is generated based on the operator's policy. This is supported in non-roaming and home-routed scenarios only. However that information is not generated or derived based on AF request.

The AF-influenced traffic steering enforcement control is only applied to the uplink traffic in the existing specification. The N6-LAN traffic steering enforcement control can be applied to the downlink traffic, uplink traffic or both. The N6-LAN Traffic steering policy identifiers can be different for uplink and downlink direction.

Table 6.3.1 of TS 23.503[4] indicates that either N6-LAN Traffic Steering Enforcement Control or AF influenced Traffic Steering Enforcement Control can be included within a PCC rule. It means that traffic steering and SFC can’t be applied to the same application traffic simultaneously.

From above analysis, it can be concluded that the N6-LAN traffic steering control (i.e. SFC policy) is different from the AF-influenced traffic steering enforcement control (i.e. traffic steering policy).

It is proposed that SFC policy and traffic steering policy is separated. An Application Function may send a request to influence SFC enforcement for the application traffic. In the request, the AF shall include the N6-LAN traffic steering control information, which includes target UE Identifier(s), DNN, S-NSSAI, SFC identifier (uplink and/or downlink), to the 5GC based on the SLA between the operator and the 3rd party. The PCF authorizes the N6-LAN traffic steering control information received from AF based on the SFC identifier (uplink and/or downlink) and operator's policy when generating the PCC rule. And then the N6-LAN traffic steering enforcement control information provisioning and enforcement defined in clause 6.1.3.14 and 6.2.2.6 of TS 23.503[4] can be applied, i.e. installing the related rule at UPF.

Figure 6.8.1-1 Architecture and traffic steering control flow

NOTE 1:	based on the local configuration, the UPF can differentiate the packet coming from N6-LAN or from N6 interface as described in clause 5.4.8 of TS29.244[XX]

It is possible that N6-LAN traffic steering control and AF-influenced traffic steering control are both applied to the same application traffic simultaneously.

- If the N6-LAN traffic steering enforcement control in downlink is applied when the AF-influenced traffic steering enforcement control is applied, the PCF include N6-LAN traffic steering policy identifier in downlink, and AF-influenced traffic steering policy identifier or N6 traffic routing information per DNAI within a PCC rule.

- If the N6-LAN traffic steering enforcement control in uplink is applied simultaneous as the AF-influenced traffic steering enforcement control is applied, the PCF shall include N6-LAN traffic steering policy identifier in uplink and AF-influenced traffic steering policy identifier or N6 traffic routing information per DNAI within a PCC rule.

Similar with the AF influenced traffic steering control defined in clause 4.3.6.1 of TS 23.503[4], the following cases can be distinguished:

-	AF requests targeting an individual UE by a UE address; these requests are routed (by the AF or by the NEF) to an individual PCF using the BSF. This is described in clause 6.8.2.1.

-	AF requests targeting a group of UE(s), or any UE accessing a combination of DNN and S-NSSAI, or targeting individual UE by a GPSI. This is defined in clause 6.8.2.2.

Same general handing as described in clause 4.3.6.1 of TS23.502 [3] are executed, e.g. how the NEF map the AF request to the DNN, S-NSSAI.

NOTE 2:	The solution assumes that the 5GC and the AF have agreed via SLA on the mapping between a SFC identifier and for each DNAI/ N6 interface: the corresponding SFP, the metadata that the UPF should send to the SFP once it has done its SFC related classification.

6.8.2	Procedures

6.8.2.1	Handling an AF request targeting an individual UE address

Figure 6.8.2.1-1: Handling an AF request targeting an individual UE address

- Step 1 to Step 5 of figure 4.3.6.4 of TS 23.502[3] are performed with following modifications and exceptions:

- In step 1, the AF provides the N6-LAN traffic steering control information in the request. If the AF-influenced traffic steering control for the same application traffic is required simultaneously, the AF-influenced traffic steering control are provided additionally in the request.

- In step 4, the AF or the NEF provides the N6-LAN traffic steering control information in the request. If the AF-influenced traffic steering control for the same application traffic is required simultaneously, the AF-influenced traffic steering control are provided additionally in the request.

In both case the N6-LAN traffic steering control information includes SFC identifier (uplink and/or downlink).

- In step 5, the PCF authorizes the N6-LAN traffic steering enforcement control information based on the received the N6-LAN traffic steering control information and the operator’s policy. The N6-LAN traffic steering enforcement control information includes the appropriate N6-LAN Traffic steering policy identifier(s) (uplink and/or downlink), which is derived from the AF provided SFC identifier (uplink and/or downlink). If both the N6-LAN traffic steering control information and AF-influenced traffic steering control information are provided, the PCF authorizes the N6-LAN traffic steering enforcement control information and AF-influenced Traffic Steering Enforcement Control information separately.

- Step 1 to Step 8 of Figure 6.8.2.3-1 is performed.

6.8.2.2	Handling an AF request not identified by an UE address

Figure 6.8.2.2-1: Handling an AF request not identified by an UE address

- Step 1 to Step 5 of figure 4.3.6.2 of TS 23.502 are performed with following modifications and exceptions:

- In step 1, the AF provides the N6-LAN traffic steering control information in the request, which includes SFC identifier (uplink and/or downlink). If the AF-influenced traffic steering control for the same application traffic is required simultaneously, the AF-influenced traffic steering control information are provided additionally in the request.

- In step 3, the NEF stores the AF request information in the UDR (Data Set = Application Data; Data Subset = N6-LAN traffic steering control information, Data Key = AF Transaction Internal ID, S-NSSAI and DNN and/or Internal Group Identifier or SUPI).

- In step 4, the PCF(s) that have subscribed to modifications of AF requests (Data Set = Application Data; Data Subset = N6-LAN traffic steering request information, Data Key = S-NSSAI and DNN and/or Internal Group Identifier or SUPI) receive(s) a Nudr_DM_Notify notification of data change from the UDR.

- In step 5, the PCF authorizes the N6-LAN traffic steering enforcement control information based on the received N6-LAN traffic steering control information and the operator’s policy. The N6-LAN traffic steering enforcement control information includes the appropriate N6-LAN Traffic steering policy identifier(s) (uplink and/or downlink), which is derived from the AF provided SFC identifier (uplink and/or downlink). If both the N6-LAN traffic steering control information and AF-influenced traffic steering control information are provided, the PCF authorizes the N6-LAN traffic steering enforcement control information and AF-influenced traffic steering enforcement control information separately.

- Step 1 to Step 8 of Figure 6.8.2.3-1 is performed.

6.8.2.3	Traffic steering enforcement control

Figure 6.8.2.3-1: Traffic steering enforcement control

- The PCC rule which include the N6-LAN traffic steering enforcement control information is installed at the SMF as described in clause 6.8.2.1 or 6.8.2.2.

- If N6-LAN Traffic steering enforcement control information is provided and the SMF determines the UPF (PSA) doesn’t support the N6-LAN Traffic steering policy identifier(s), it inserts/reselect an I-UPF/L-PSA which supports the pre-defined SFC corresponding to the AF request.

If both the N6-LAN traffic steering enforcement control information and AF-influenced traffic steering enforcement control information are provided, the SMF needs to discover the UPF which supports both N6-LAN traffic steering enforcement control and AF-influenced traffic steering control.

If the SMF finds the suitable UPF to execute the N6-LAN traffic steering or the N6-LAN traffic steering and AF-influenced traffic steering cannot be executed simultaneously, step 3-8 are skipped. Otherwise those steps are executed.

3-5 The SMF notify the AF via the PCF and NEF that N6-LAN traffic steering enforcement control cannot be enforced, or AF-influenced traffic steering and N6-LAN traffic steering cannot be enforced simultaneously.

6-8. If the AF-influenced traffic steering and N6-LAN traffic steering cannot be enforced simultaneously, the AF may select one of the traffic steering policy and return it to SMF via NEF and PCF. The SMF then may execute the UPF selection as defined in step 2 again.

NOTE 1: 	If the network deployment guarantees that at least one UPF per DNAI supports N6-LAN traffic steering enforcement control, step 3 – step 8 does not occur.

9. 	The SMF generates the N4 rule based on the received PCC rule as follows:

- if only N6-LAN traffic steering enforcement control is enforced, besides the PDR SMF derives the forwarding policy identifier (uplink and/or downlink) within the FAR based on the authorized the N6-LAN traffic steering enforcement control information.

- if only AF-influenced traffic steering enforcement control is enforced, besides the PDR SMF derives the forwarding policy within the FAR based on the authorized the AF-influenced traffic steering enforcement control information.

- if both N6-LAN traffic steering enforcement control and AF-influenced traffic steering enforcement control are enforced,

- If the N6-LAN traffic steering enforcement control in downlink is applied when the AF-influenced traffic steering enforcement control is applied, the SMF generates two PDR and FAR pairs. The first PDR and FAR pair is associated the downlink N6-LAN traffic steering policy handing. The second PDR and FAR is associated with the uplink AF influenced traffic steering policy handling.

- If the N6-LAN traffic steering enforcement control in uplink is applied when the AF-influenced traffic steering enforcement control is applied, the SMF generates two PDR and FAR pairs. The first PDR and FAR pair is associated the N6-LAN traffic steering policy handing. The second PDR and FAR is associated with the AF influenced traffic steering policy handling for traffic after N6-LAN traffic steering handling.

NOTE 2: 	The UPF enforce the first PDR and FAR pair to steer the uplink traffic to the N6-LAN and then enforce the second PDR and FAR pair to steering the uplink traffic, which has been processed by the SF(s) and routed back to the UPF, to the local network.

The SMF provisions the N4 rule to the UPF (PSA) by initiating the N4 session establishment or modification.

10.	Per the received forwarding policy, when the UPF (PSA) detects the related traffic, the UPF steers the corresponding traffic to the service function(s) deployed in the N6-LAN if applicable, and further steer the traffic to the local data network if applicable. How to steering the traffic is out of 3GPP scope.

6.8.3	Impacts on Existing Nodes and Functionality

The solution has impacts in the following entities:

AF:

-	Needs to support to include the N6-LAN traffic steering control request information in the request.

-	Needs to support to select which traffic steering control is to be executed, if the N6-LAN traffic steering control and AF-influenced traffic steering control information cannot be supported by the 5GC simultaneously.

NEF:

-	Needs to support to transfer the N6-LAN traffic steering control request information.

-	Needs to support to transfer the notification that the N6-LAN traffic steering control information is not supported or two traffic steering control cannot be is not supported to be executed by the 5GC simultaneously.

UDR

-	Needs to support to store the N6-LAN traffic steering control request information.

PCF:

-	Needs to support to authorize the N6-LAN traffic steering enforcement control information based on the received the N6-LAN traffic steering control information.

-	Needs to support to transfer the notification that the N6-LAN traffic steering control information is not supported or two traffic steering control cannot be is not supported to be executed by the 5GC simultaneously.

SMF:

-	Needs to support to select the I-UPF/L-PSA which supports N6-LAN traffic steering control information or UPF which supports both N6-LAN traffic steering control information and AF-influenced traffic steering control information

-	Needs to support to derive two PDR and FAR pairs. The first PDR and FAR pair is associated the N6-LAN traffic steering policy handing. The second PDR and FAR is associated with the AF influenced traffic steering policy handling.

-	Needs to support to send the notification that the N6-LAN traffic steering control information is not supported or two traffic steering control cannot be is not supported to be executed by the 5GC simultaneously.

END OF CHANGES