3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204364

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update on Solution#10 (KI#1): LBO PDU Session establishment using PLMN criteria in RSD

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_eEDGE_5GC_ph2 / Rel-18

Abstract: Update on Solution#10. Clarification on how HPLMN determines the PLMN criteria.

1. Introduction/Discussion

In current specification, Solution#10 describes the content of enhanced URSP with PLMN criteria, and may re-use the procedure in current TS 23.502 [9]. In order to determine PLMN criteria for any RSD, H-PCF shall get the information related to PLMN(s) which Service 1 deployed. The information can be either pre-configured in H-PCF or provisioned by AF.

This contribution clarifies how the H-PCF gets the information, which is not clearly specified in current Solution#10.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-48 v0.2.0.

# * * * * First change * * * *

6.10	Solution 10 (KI#1): LBO PDU Session establishment using PLMN criteria in RSD

6.10.1	Description

This solution corresponds to KI#1, and addresses the scenario which UE accessing EHE in VPLMN using LBO PDU Session.

For better illustration, it is assumed that Service 1 is deployed in both HPLMN and EHE in VPLMN, and Service 2 is deployed only in HPLMN (or deployed in EHE in VPLMN, but the UE is not allowed to access to EHE in VPLMN for Service 2).

When UE accesses to VPLMN, UE may access to EHE in VPLMN via a LBO PDU Session, and access to HPLMN via a HR PDU Session. That means URSP rules with different RSDs shall be used for Service 1 accessing to EHE in VPLMN and Service 2 accessing to HPLMN.

When UE access to HPLMN, it is unnecessary to access to Service 1 and Service 2 via different PDU Sessions. That means URSP rules with same RSD shall be used for Service 1 and Service 2.

Adding Route Selection Validation Criteria related to PLMN(s) is a way to meet both requirements on S-NSSAI/DNN pair mentioned above. Table 6.10.1-1 illustrates the enhanced RSD, and Table 6.10.1-2 gives an example of the enhanced RSD for Service 1.

Table 6.10.1-1: Route Selection Descriptor with PLMN criteria

| Information name | Description | Category | PCF permitted to modify in URSP | Scope |
| --- | --- | --- | --- | --- |
| Route Selection Descriptor Precedence | Determines the order in which the Route Selection Descriptors are to be applied. | Mandatory (NOTE 1) | Yes | UE context |
| Route selection components | This part defines the route selection components | Mandatory (NOTE 2) |  |  |
| Network Slice Selection | Either a single value or a list of values of S-NSSAI(s). | Optional (NOTE 3) | Yes | UE context |
| DNN Selection | Either a single value or a list of values of DNN(s). | Optional | Yes | UE context |
| … | … | … | … | … |
| Route Selection Validation Criteria (NOTE 6) | This part defines the Route Validation Criteria components | Optional |  |  |
| Time Window | The time window when the matching traffic is allowed. The RSD is not considered to be valid if the current time is not in the time window. | Optional | Yes | UE context |
| Location Criteria | The UE location where the matching traffic is allowed. The RSD rule is not considered to be valid if the UE location does not match the location criteria. | Optional | Yes | UE context |
| PLMN Criteria (new) | The PLMN(s) where the matching traffic is allowed. The RSD is not considered to be valid if the serving PLMN does not match the PLMN criteria. | Optional (NOTE X) | Yes | UE context |
| NOTE X: If PLMN Criteria is not included in a RSD, the RSD shall be considered as valid in all PLMNs. |  |  |  |  |

Table 6.10.1-2: An example of RSDs with PLMN criteria

|  | Information name |  | Content |
| --- | --- | --- | --- |
| URSP rule | Traffic Descriptor |  | Service 1 |
|  | RSD 1 | Network Slice Selection | S-NSSAI 1 |
|  |  | DNN Selection | DNN 1 |
|  |  | PLMN Criteria | HPLMN |
|  | RSD 2 | Network Slice Selection | S-NSSAI 1 |
|  |  | DNN Selection | DNN 2 |
|  |  | PLMN Criteria | VPLMN 1, VPLMN 2, … |

NOTE:	Before concluding this solution for normative work, alignment check with FS_eUEPO shall be done in order to avoid any conflict with the conclusion of KI#1 in TR 23.700-85 [10]. Final decision on generic URSP enhancement for supporting LBO roaming case should be made within FS_eUEPO study.

6.10.2	Procedure

The provision of the enhanced URSP with PLMN criteria re-uses the procedure in current specification TS 23.502 [9], or UE may be pre-configured the URSP. In order to determine PLMN criteria, H-PCF shall get the information related to PLMN(s) which the service deployed. The information can be either pre-configured in H-PCF or provisioned by AF. When UE triggers PDU Session selection or PDU Session establishment based on URSP, the UE takes the serving PLMN into consideration.

6.10.3	Impacts on services, entities and interfaces

PCF:

-	controls the UE to use different DNN+S-NSSAI in different PLMN by including PLMN Criteria in URSP RSDs.

UE:

-	shall consider the serving PLMN when it determines whether to re-use or establish PDU Session, if PLMN Criteria is included in the selected Route Selection Descriptor.

# * * * * End of changes * * * *