3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-224365

Elbonia, May 16 – 20, 2022	(revision of S2-2202407)

Source:	Huawei, HiSilicon

Title:	New solution for KI#7: AF obtaining target DNAI

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_eEDGE_5GC_Ph2 / Rel-18

Abstract: The solution addresses KI#7: Obtain and maintain mapping table between IP address/IP range with DNAI.

1. Introduction/Discussion

In SA2#149E, KI#7: Obtain and maintain mapping table between IP address/IP range with DNAI was approved. For this KI, the following issues shall be studied:

-	what information the AF should provide to 5GC (e.g., IP address or range thereof (i.e.IPv4 subnetwork and IPv6 prefixes) , FQDN) and how it should provide it to help determine proper DNAI if the AF does not have knowledge of the DNAI information;

-	whether and how the 5GC can provide a DNAI to AF to help the subsequent AF influence/request.

To address the first issue, the following information is proposed to be provided to 5GC by the AF:

-	EHE information. Including one or more of the following:

-  EAS IP address/IP range. The IP address or IP range of the requested EAS.

-  FQDN(s). The FQDN (list) of the requested EAS.

-  (Local) DNS server information. The (L-)DNS server address for each DNAI.

-	DNN(s). The DNN(s) related to the requested EAS.

-  S-NSSAI(s). Optional. The S-NSSAI(s) related to the requested EAS.

To address the second issue, 5GC determines and sends the DNAI to the AF by the following way:

-	NEF may determine target DNAI based on NF local configuration.

-	NEF may obtain target DNAI from UDR.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-48 v0.2.0.

# * * * * First change * * * *

6.0	Solution-Key issue matrix

The solutions in clause 6 can apply to one or more key issues described in clause 5 of this report. Table 6.0-1 describes the relationship between solutions and key issues.

Editor's note:	The table below will be updated with actual content when generating the TR with approved contributions. Page number is automatically updated to ease reference (ctrl-left click to reach the solution).

Table 6.0-1: Solution-Key issue matrix

| Solution |  | Key issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | (page) | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 | KI#7 |
| 01: EAS discovery in Home Routed roaming scenario | 18 | X |  |  |  |  |  |  |
| 02: Session Breakout in Visited PLMN | 21 | X |  |  |  |  |  |  |
| 03: EAS (re)discovery procedure in roaming scenario | 23 | X |  |  |  |  |  |  |
| 04: Support EAS discovery in VPLMN via HR PDU Session | 28 | X |  |  |  |  |  |  |
| 05: Accessing V-EHE via HR PDU session | 31 | X |  |  |  |  |  |  |
| 06: URSP solution to support roamers access to EHE in a VPLMN | 33 | X |  |  |  |  |  |  |
| 07: Using URSP Rules to Establish an LBO PDU Session | 35 | X |  |  |  |  |  |  |
| 08: V-ECS Discovery during Steering of Roaming | 36 | X |  |  |  |  |  |  |
| 09: PDU Session configuration from EASDF | 38 | X |  |  |  |  |  |  |
| 10: LBO PDU Session establishment using PLMN criteria in RSD | 40 | X |  |  |  |  |  |  |
| 11: Exposure of Network Congestion | 41 |  | X |  |  |  |  |  |
| 12: Efficient exposure of RAN information | 43 |  | X |  |  |  |  |  |
| 13: Fast and efficient network exposure improvements | 47 |  | X |  |  |  |  |  |
| 14: Group Management | 50 |  |  |  | X |  |  |  |
| 15: Selection of common DNAI | 54 |  |  |  | X |  |  |  |
| 16: Selecting the same EAS/DNAI for collection of UEs | 62 |  |  |  | X |  |  |  |
| 17: Application layer EAS selection for collections of UEs | 64 |  |  |  | X |  |  |  |
| 18: Discovery of the same EAS for collections of UEs | 66 |  |  |  | X |  |  |  |
| 19: Influencing UPF and EAS (re)location for collections of UEs | 70 |  |  |  | X |  |  |  |
| 20: Global EASDF | 71 |  |  |  |  | X |  |  |
| 21: EAS Deployment information differentiated by PLMN ID | 73 |  |  |  |  | X |  |  |
| 22: EAS discovery for federated OPs | 74 |  |  |  |  | X |  |  |
| 23: Improvements for EHE operated by separate party | 80 |  |  |  |  | X |  |  |
| X: AF obtaining target DNAI provided by NEF. |  |  |  |  |  |  |  | X |

# * * * * Second change, all new texts * * * *

6.X	Solution X (KI#7): AF obtaining target DNAI provided by NEF

6.X.1 Description

This solution corresponds to KI#7: Obtain and maintain mapping table between IP address/IP range with DNAI.

The AF may provide one or more the following information set to 5GC, in order to request for target DNAI(s):

-	EHE information. Including one or more of the following:

-  EAS IP address/IP range. The IP address or IP range of the requested EAS.

-  FQDN(s). The FQDN (list) of the requested EAS.

-  (Local) DNS server information. The (L-)DNS server address for each DNAI.

-	DNN(s). The DNN(s) related to the requested EAS.

-  S-NSSAI(s). Optional. The S-NSSAI(s) related to the requested EAS.

After receiving the information above, the 5GC determines and sends the DNAI to the AF by the following way:

-	NEF may determine target DNAI based on NF local configuration.

-	NEF may obtain target DNAI from UDR.

6.X.2 Procedure

Figure 6.X.2-1 Procedure of AF obtaining target DNAI

1. 	The AF has the requirement to obtain target DNAI, and creates an AF request, which may include the following information to 5GC, in order to request for target DNAI:

-  EHE information. Including one or more of the following:

-  EAS IP address/IP range. The IP address or IP range of the requested EAS.

-  FQDN(s). The FQDN (list) of the requested EAS.

-  (Local) DNS server information. The (L-)DNS server address for each DNAI.

-  DNN(s). The DNN(s) related to the requested EAS.

-  S-NSSAI(s). Optional. The S-NSSAI(s) related to the requested EAS.

2.	The AF invokes Nnef_EASDeployment_Query Request service to send its request to the NEF. The NEF authorizes the AF request.

The NEF may determine the target DNAI by local configuration, or obtain the DNAI from UDR, as described in steps 3-4.

3.	The NEF invokes Nudr_DM_Query Request service to request for target DNAI, including information received in the AF request.

4.	The UDR sends Nudr_DM_Query Response to the NEF, including the target DNAI.

5.	The NEF sends Nnef_EASDeployment_Query Response to the AF, including the target DNAI.

6.X.3 Impacts on services, entities and interfaces

AF:

-  Provide EHE information (described in clause 6.X.2), DNN(s), S-NSSAI(s) via invoking Nnef_EASDeployment_Query service.

NEF:

-  Provide target DNAI in Nnef_EASDeployment_Query Response.

-  Request DNAI information (described in clause 6.X.2) via invoking Nudr_DM_Query service.

UDR:

-  Provide target DNAI in Nudr_DM_Query Response.

# * * * * End of changes * * * *