3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-224366

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2 and KI#3, Sol#4: Update to remove ENs

Document for:	Approval

Agenda Item:	9.12

Work Item / Release:	FS_5GSATB/ Rel-18

Abstract: An update on Solution 4 for Key Issue #2 and Key Issue #3.

1. Introduction

This contribution proposes to solve the following two Editor’s Notes.

Editor’s Note: How does the SMF knows the GEO satellite ID and corresponding UPF on the GEO satellite are FFS.

Editor’s Note: How to support that case that one UE quit the UE-UE communication is FFS.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-27.

# * * * * First change * * * *

6.4	Solution for Key Issue #2 and Key Issue #3: Enable Satellite Edge Computing or local data switching via on-board UPF acting as UL CL/BP and local PSA

6.4.1	Description

This solution corresponds to KI#2 and KI#3.

UE establishes a PDU Session with PSA on ground via GEO satellite backhaul. SMF needs to

1) determine whether to insert a UL CL/BP on the GEO UPF and

2) obtain the target IP of traffic to be local routed to configure the traffic steering rule to the GEO UPF (UL CL/BP/local PSA).

6.4.2	Procedures

Figure 6.x.2-1 shows the procedure to enable satellite edge computing or local data switching via on-board UPF acting as UL CL/BP and local PSA.

Figure 6.4.2-1: Enabling GEO satellite edge computing via on-board UPF

1. SMF determines to insert GEO UPF as UL CL/BP and local PSA with the following triggers:

1a.	(for edge computing case) During the PDU Session establishment procedure, SMF determines the UE traffic can be local routed at a DNAI corresponding to the GEO satellite based on the following information: 1) UE location, 2) GEO satellite backhaul category, 3) an indication that UE is allowed to access the local services on the DNAI based on subscription from UDM or policy from PCF, 4) EAS Deployment Information.

SMF determines the traffic steering rules based on the IP range of the EAS deployed on the GEO UPF.

1b.	(for UE-to-UE case) AF requires UE-to-UE communication for two UEs by providing UE ID/IP to the SMF. The SMF determines two UEs are under the same GEO satellite.

The SMF determines whether two UEs are served under the same GEO satellite based on stored correlation between the GEO satellite ID and UE IP/DNN. If the two UEs are under same GEO satellite, then SMF determines the two UEs can be served by the same GEO UPF.

If different SMFs are used for two UEs, the SMF checks whether the two UEs are served by the same GEO satellite via UDM. The SMF retrieves UE IP list corresponding to the GEO satellite from UDM and checks whether the two UEs are included in the UE IP list. If yes, the SMF determines the two UEs are served by the same GEO UPF. The SMF also registers the UE IP/DNN and the GEO satellite ID to the UDM.

When one UE quits the UE-to-UE communication, the SMF is notified by the AF. The SMF initiates PDU Session Modification procedures for both of the UEs to update the traffic steering rules or release the UL CL/BP/local PSA.

SMF determines traffic steering rules for the two UEs based on the target UE IP.

1c.	(for edge computing and UE-to-UE case) UE sends UL packet to PSA UPF on ground and the PSA UPF on ground reports related information, e.g. FQDN in DNS query as described in clause 6.2.3 in TS 23.548 [6], target IP, to SMF.

For edge computing case, the SMF determines UE is accessing local services on the GEO UPF via e.g. mechanisms described in clause 6.2.3 in TS 23.548[6].

For UE-to-UE case, the SMF determines the two UEs, i.e. the source UE and the target UE, are under the same GEO satellite.

SMF determines traffic steering rules based on the target IP address detected and reported by the PSA UPF.

NOTE 1:	For step 1a/1b/1c, a specific DNAI needs to be configured by the operator for a specific GEO satellite ID, so that existing UPF selection and other EC procedures can be reused to support the local routing on the GEO satellite.

2.	SMF selects and inserts the GEO UPF as UL CL/BP and local PSA and sends traffic steering rules to the GEO UPF.

For edge computing case, the traffic steering rules indicate the ULCL/BP to perform local routing for the packets targeting the EC services on the GEO satellite.

For UE-to-UE case, the traffic steering rules indicate the ULCL to perform local routing for the packets to the target UE.

6.4.3	Impacts on services, entities and interfaces

AF:

For UE-to-UE case:

-	Provide/Update UE ID/IP for UE-to-UE communication to the SMF.

SMF:

-	Be configured by the operator a specific DNAI for a specific satellite ID.

For edge computing case,

-	Determine whether a ULCL/BP should be inserted on the GEO Satellite based on 1) UE location, 2) GEO satellite backhaul category, 3) an indication that UE is allowed to access the local services on the DNAI based on subscription from UDM or policy from PCF, 4) EAS Deployment Information.

For UE-to-UE case,

-	Determine whether local data switching should be performed for UEs served by same GEO satellite based on AF request or PSA UPF reporting.

-	Interact with UDM to check whether two UEs are served by same GEO satellite in case of different SMFs serving UEs in communication.

UDM:

For UE-to-UE case, when different SMFs are used:

-	Receive and store GEO Satellite ID and UE IP/DNN from SMF.

# * * * * End of changes * * * *