3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-224369

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update on arch assumption, traffic characteristics and video slice definition

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: Based on SA4 LS, this paper proposes an arch assumption and removal of video slice definition and traffic char in Annex.

1. Introduction

According to the LS from SA4 (S2-2203658/S4-220505), the traffic characteristics in Annex and the definition of video slice may be not complete and applicable to all cases. Therefore, this contribution intends to remove the Annex and definition of video slice, and add abstracted principle into the architecture assumption in the SA2 TR.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change * * * *

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services), which are of same importance requirement at application layer. All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In some cases, the application layer can still recover parts of the information unit, when some PDUs are missing.

Multi-modal Data: Multi-modal Data is defined to describe the input data from different kinds of devices/sensors or the output data to different kinds of destinations (e.g. one or more UEs) required for the same task or application. Multi-modal Data consists of more than one Single-modal Data, and there is strong dependency among each Single-modal Data. Single-modal Data can be seen as one type of data.

NOTE:	This definition was taken from TR 22.847 [6].

# * * * * Second change * * * *

4.1	Architectural Assumptions

-	The architecture, framework and the QoS model as specified in TS 23.501 [2], TS 23.502 [3], and TS 23.503 [4] are regarded as the baseline for this study.

NOTE 1:	The study focuses on using NR as access technology. Specific enhancements to other access types are not required to be studied in this TR (although not prohibited).

-	The functional split in 5GS between UE, RAN and CN remains unchanged, i.e. packet classification of DL packets is performed in CN, and the packet classification of UL packets is performed in UE.

-	XRM services are assumed to use the IP PDU session types (however other PDU types are not excluded).

-	XRM services shall be able to coexist within a PLMN or SNPN with existing services simultaneously

-	XRM services can be between client-server (i.e. UE - application server) and/or peer-to-peer (i.e. between two UEs routed via the 5G CN).

-	Architecture enhancements should support XRM applications and its traffic characteristics. However, media codec mechanisms are not in the scope of this study.

-	XR and media application data may be encrypted by the client and/or server in some cases and unencrypted in other cases.

-	The traffic characteristics (e.g. Burst periodicity, PDU Set importance and traffic correlation) can vary for different media applications, and may be determined based on agreements between applications and 5GS.

NOTE 2:	It is assumed that the some header information necessary for the identification of PDUs is not encrypted.

# * * * * Third change * * * *

# * * * * End of changes * * * *