3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204394

Elbonia, May 16th – 20th, 2022	(revision of S2-2204394)

Source:	Huawei, HiSilicon

Title:	KI#3, Sol#13 Update to remove ENs

Document for:	Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract: This paper proposes to update solution #13 in TR 23.700-87.

- Introduction

This pCR updates the solution 13 in TR 23.700-87.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-87.

# * * * * First change * * * *

6.13.2	Procedures

6.13.2.1	third party ID Access via SIP Trunk

Figure 6.13.2.1-1: third party ID Access via SIP Trunk

1.	The BSS provisions the third party subscription data to the EDS according to the description in 6.x.1.4.

2.	The third party PBX establishes TLS connection with the IBCF. The PBX needs to store the IBCF domain name and the CA of the IBCF certificate for the TLS handshake.

3.	The IBCF sends domain name and provisioned info to the EDS to verify if the third party PBX is a trusted one.

4.	After verifying provisioned info, EDS sends response indicating the request has succeeded.

5.	The TLS connection is established.

6.	The third party subscriber sends a SIP INVITE through TLS connection, containing the third party ID and token. The third party subscriber gets the token when login into the third party auth. server.

7.	The IBCF forwards the SIP request to the S-CSCF.

8.	The S-CSCF finds the corresponding EDS based on the domain info in the third party ID, and send the third party ID and the token to the EDS for authentication.

9.	The EDS forwards the third party ID and the token to the corresponding third party auth. server for authentication. The TLS link between EDS and third party Auth. Server is established in previous stage with mutual TLS authentication.

10.	The third party auth. server verifies whether the Token is a valid one. If so, the third party auth. server sends success response with the relevant third party subscriber data (e.g. E.164 number, name, department etc.) corresponding to the provisioned third party ID to the EDS. The EDS caches the data.

11.	The EDS informs the S-CSCF the authentication is succeeded. The S-CSCF downloads temporarily generated user data from the EDS.

NOTE: Security related aspects should be studied by SA WG3.

12.	The S-CSCF forwards the messages to the AS.

13.The MMTel AS requests user data from the EDS.

14.	The EDS responses, returning data of the third party subscriber.

15.	The MMTel AS requests the MRF to process name card media.

16.	The MMTel AS responses, and perform asynchronous processing.

17.	The MMTel changes the value of From and P-Asserted-Identity headers to E.164 number that allocated to the given third party, then forwards the INVITE to the S-CSCF.

18.	The S-CSCF forwards the messages to the terminating IMS network.

19.	The MRF shows the rich name card (e.g. through the CRS service defined in TS 24.183[20]) during ringing.

NOTE: 	the structure of the name card should be discussed in stage 3.

20.	The called party responds 18x and 200 to the S-CSCF.

21.	The S-CSCF forwards 18x and 200 messages to the calling UE.

22.	The MRF stops playing rich name card in media plane, and redirects the call to calling party.

23.	Call established.

24.	Call ended.

25.	Billing and record for later audit use.

# * * * * Second change * * * *

6.13.2.5	Terminating IMS network verifying third party specific identities and name card

This clause aims to give solution to the KI “how the terminating IMS network can support the called party to verify third party specific identities during a call.” and “potential impacts on STIR/SHAKEN procedures defined in TS 24.229[8]”.

This procedure is optional to the third party id main solution described in the above paragraph, and with this procedure, the caller name card can be transmitted avoid malicious tampering. Usually it is not used inside a carrier’s IMS network.

Figure 6.13.2.4-1: terminating IMS network verifying third party specific identities and name card

1.	The third party subscriber initiates a call.

2.	The originating MMTel AS gets a private key, generates the JWT and signature, and put them in SIP Identity header.

3.	The originating MMTel AS indicates in the INVITE message that the caller side support media SHAKEN by carrying “Supported: early-media-verification-with-shaken”.

4.	The originating IBCF forwards the message to the terminating IBCF.

5.	The terminating IBCF forwards the message to the terminating MMTel AS.

6.	The terminating MMTel AS verifies the signature in Identity header with the public key.

7.	The terminating MMTel AS generates shared key and shared salt, encrypts them with the public key.

8.	The terminating MMTel AS forwards the call to the terminating UE, and uses verstat parameter in the From and PAI URI header to indicate caller’s level.

9.	The terminating UE responds 18x.

10.	The terminating MMTel AS forwards the 18x to the originating MMTel AS, carrying the Require and P-Media-Verification header.

11.	The originating MMTel AS decrypts the shared key and shared salt with the private key.

12.	The terminating MMTel AS forwards the shared key and shared salt to the terminating media checkpoint, and inform the terminating media checkpoint to initiate media check.

13. The terminating media checkpoint responds.

14. The originating MMTel AS forwards the shared key and shared salt to the MRF, and request the MRF to initiate media integrity protection tag in SRTP tag.

15. The MRF responds.

16.	The originating MMTel AS informs the MRF to start playing name card.

17.	The MRF start sending the SRTP with integrity protection.

18.	The terminating media checkpoint checks the integrity of the media.

19.	The terminating media checkpoint sends the result to the terminating MMTel AS.

NOTE:	it is recommended that media checkpoint locates in IMS-AGW.

20.	The terminating MMTel AS sends the results to the terminating UE.

NOTE:	Security related aspects should be studied by SA WG3.

# * * * * End of changes * * * *