3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204396

Elbonia, May 16th – 20th, 2022	(revision of S2-2204396)

Source:	Huawei, HiSilicon

Title:	KI#1, Sol#4, Update to Editor’s Notes in clause 6.4.1.2.1

Document for:	Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract: This paper gives suggestions on Editor’s Notes in clause 6.4.1.2.1 of TR 23.700-87.

1. Introduction/Discussion

This paper lists solutions to Editor’s Notes in clause 6.4.1.2.1, that is:

Editor's note:	How the UE discovers the data channel capability of the VPMN in roaming scenario, is FFS.

Furthermore, a typo of uncompleted statement is corrected.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-87.

# * * * * First change * * * *

6.4.1.2.1	UE discovers the data channel capability of its home network

It’s suggested that the mechanism defined in IETF RFC 6809 [11] is applied for the UE to discover the data channel capability of its home network. That is, the S-CSCF in its home network includes a Feature-Caps header field, with the "+g.3gpp.webrtc-datachannel" header field parameter in the 200 OK response during IMS registration procedure defined in section 5.1.1 of 3GPP TS 24.229 [2], if the home network supports IMS data channel capability.

Editor's note:	This feature-capability indicator "+g.3gpp.webrtc-datachannel", is to be registered with IANA.

In roaming scenario, the HPLMN should ensure that the requested QoS sent to a VPLMN is within the limits of the roaming agreement. Therefore, if the S-CSCF knows that the VPLMN does not support IMS data channel capability based on local configuration according to the roaming agreement with the VPLMN, it should not include a Feature-Caps header field, with the "+g.3gpp.webrtc-datachannel" header field parameter in the 200 OK response during IMS registration procedure.

# * * * * End of changes * * * *