SA WG2 Meeting #151E	S2-2204405

# 16 – 20 May 2022, e-meeting

Source:	Lenovo, Apple

Title:	KI#1: Update of Solution 2 to remove an editor's note

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution:

# 1		Discussion

The following editor's notes is addressed:

Editor's Note: Since the allowed S-NSSAI in one PLMN can also apply to the equivalent PLMNs indicated in a TAI list in registration accept. When UE moves from one PLMN to another equivalent PLMN, the allowed S-NSSAI still need to be used. How to guarantee it in this solution is FFS.

As part of clause 15.15.4.1.1 of 3GPP TS 23.501 when the UE receives the Allowed NSSAI within a registration accept or UCU the AMF may include the RA/TAI list of the PLMN(s) where the Allowed NSSAI is applicable.

When the UE evaluates URSP rules the UE considers the RSD valid if the received Allowed NSSAI is part of the S-NSSAI component in the RSD (if available). If the RSD includes a PLMN as a validity condition then the UE should consider the RSD valid in the scenario where the UE moves to an equivalent PLMN (i.e. the UE moves from TA1 of PLMN 1 to TA2 of PLMN 2 that is is equivalent to PLMN 1).

Such scenario is already addressed in the solution as shown below.

-	If an RSD in the list of RSDs contains PLMN validity conditions the UE checks if the PLMN identity within the validity conditions matches the PLMN identity of the registered PLMN or an equivalent PLMN of the registered PLMN.

-	If there is match the UE considers the RSD valid and and routes the application traffic according to the contents of the route selection descriptor

-	If there is no match the UE considers the RSD as invalid and continues to process a second RSD in the list of RSD of the matched URSP rules if available.

Hence the editor's note is not needed and removed from the solution description.

# 2		Proposal

The following solution is proposed.

******************************** First change  *******************************

### 6.2.2	Procedures

#### 6.2.2.1	VPLMN AF guidance for URSP rule creation at an HPLMN.

A call flow of the procedure is shown below.

Figure 6.2.2.1-1: Procedure to influence URSP rules for a different PLMN

Steps are as follows:

1.	AF in VPLMN is triggered to request to the home PLMN of a UE a specific handling of routing traffic to the VPLMN.

2.	The AF invokes a service operation with the NEF including in the request updated service information as described in TS 23.502 [3] clause 4.15.6.10. The updated information includes a validity conditions to apply a URSP rule in a specific PLMN (or a list of PLMN). The request is applicable for any UE or the AF may include external identifiers of the UE.

yup

2a-2c	The NEF may authorize the request with the UDM according to TS 23.502 [3].

3a	When the authorization is successful the NEF updates the data in the UDR including within Application Data the updated service information. The NEF invokes an Nudr_DM_Create (or Update) service request requested the UDR to store the service information in the "Application Data" Data Set within the Service Specific Information Data Subset identifier. The NEF may also include a Data Key the target UEs or Group of UEs.

3b.	The UDR acknowledges the NEF request.

3c.	The NEF acknowledges the AF request.

4a.	If the PCF has subscribed to Service Specific Information the UDR notifies the PCF of the updated service information.

4b.	When a UE registers to the H-PLMN (or VPLMN) the UE includes a list of PSIs. The AMF (in HPLMN) or V-PCF (in VPLMN) initiates a UE Policy Association Create request with the PCF including in the request the list of PSIs associated to the HPLMN.

4c.	The PCF may retrieve Application Data subscription information from the UDR.

5.	The PCF creates updated URSP rules including URSP rules with validity conditions on per PLMN basis according to the information stored in the UDR.

6.	The PCF provides updated URSP rules to the UE by initiating a UE Configuration Update for transparent policy delivery (via the V-PCF if the UE is roaming) to deliver updated URSP rule to a UE.

6b.	The UE acknowledges successful installation of rules.

7.	If the AF has subscribed to notification of successful policy delivery the PCF notifies the UDR.

8a-8b	The UDR informes the NEF of successful policy delivery.

9a-9b.	The NEF informs the AF of successful policy delivery.

******************************** End of change *******************************