# SA WG2 Meeting #151E (e-meeting)	S2-2204453

# 16-20 May 2022, Elbonia

Title:		Draft LS on PRU mobility and Reference UE

Reply to:	-

Release:	Release 18

Work Item:	FS_eLCS_ph3

Source:	SA2

To:		RAN1, RAN2, RAN3

Cc:

Contact person:	Lars Nord

Lars.nord@sony.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

SA2 have started a Rel-18 study FS_eLCS_ph3 that investigates how to support Positioning Reference Unit (PRU) in release 18. RAN1 and RAN2 agreements states that a PRU should be managed as a UE by the network. SA2 have discussed some aspects related to the Positioning Reference Unit (PRU) and have some questions related PRU mobility.

- SA2 assumes that a PRU can be mobile as the PRU is to be viewed and managed as a UE. What positioning technology should be used to acquire a new accurate known location of the PRU? SA2 believes that is important that the new known location needs to be trusted by the network.

-	If the PRU is mobile then the time for the estimation of the known accuracy is very important and must be done at the same time as when the capturing of the PRU measurements is done. These two events need to be link/time synchronized. Have RAN1 or RAN2 discussed this time synchronization?

Furthermore, TR 23.700-71 v0.2.0 includes two types of Reference UEs i.e. a PRU and Reference UE.

Positioning Reference Unit (PRU): A node with a known location used by the network to calibrate positioning measurement results when positioning a different UE. A Positioning Reference Unit is managed by the network as a UE.

Reference UE: A UE with a known location that can participate in the positioning of a different UE by providing a Line-of-Sight path for that UE. The Reference UE has the same capability as a gNB/TRP i.e., it can transmit a positioning reference signal or measure on UL SRS from a different UE.

It is clear from RAN1 and RAN2 agreements that the PRU has been discussed and agreed.

SA2 has discussed and agreed in previous meetings to include the use of reference UE with known locations for positioning in the FS_eLCS_ph3 and SA2 observes the benefit of reference UE for enhancing the positioning performance. Note that SA2 are studying Mobile IAB-UE and Sidelink positioning in other studies i.e. FS_VMR and FS_Ranging_SL. The “Reference UE” in FS_eLCS_ph3 would therefore not be an IAB-UE but still use Uu and not sidelink.

After studying RAN1 and RAN2 agreements, LS exchange, email discussion and the RAN rel-18 scope, SA2 now have the understanding that the above described “Reference UE” will not be introduced in RAN level in rel-18. Therefore, SA2 have agreed to remove the above described “Reference UE” from the study FS_eLCS_ph3. SA2 kindly seek feedback, if any, from the RAN groups on the SA2 agreement.

# 2	Actions

To RAN1, RAN2 and RAN3

ACTION: 	SA2 asks RAN1, RAN2 and RAN3 for feedback on PRU mobility and on the SA2 agreement to remove the “Reference UE” type, which is different from the PRU, from the Study FS_eLCS_ph3.

# 3	Dates of next TSG SA WG2 meetings

3GPPSA2#152	22 - 26 August 2022	Goteborg

3GPPSA2#153-e	10 - 14 October 2022	Electronic Meeting