SA WG2 Meeting #S2-151e(e-meeting)	S2-2204472

16 - 20 May 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	CATT

Title:	Discussion on scenarios to improve location service for Multi-USIM UE

Document for:	Discussion

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3

Abstract of the contribution: this discussion paper is proposed to discuss the scenarios to improve location service for Multi-USIM UE.

1. Background

The architectural requirements to support the Multi-USIM UE in clause 4.1 in TR 23.761 are as follows:

-	The study shall focus on Dual-USIM devices, the expectation being that the enablers for Dual-USIM devices will also apply to Multi-USIM devices.

-	The study shall consider all of the following cases:

-	UE connected over 3GPP Access with EPS on one USIM and 5GS on the other USIM.

-	UE connected over 3GPP Access with EPS on both USIMs.

-	UE connected over 3GPP Access with 5GS on both USIMs.

NOTE 1:	The use of "3GPP Access" above is related to issues that can occur due to concurrent transmission and/or reception in Multi-USIM device via two or more 3GPP RATs.

-	Specific to Dual-USIM devices the study shall focus on Single Rx / Single Tx UEs and Dual Rx / Single Tx UEs.

NOTE 2:	Dual Rx allows the Dual-USIM device to simultaneously receive traffic from two networks. Single Rx allows the Dual-USIM device to receive traffic from one network at one time. Single Tx allows the Dual-USIM device to transmit traffic to one network at one time.

-	The Multi-USIM device shall handle Emergency calls using clause 10.9 of TS 22.101 [5] as a basis.

-	The problem statement is common to 5GS and EPS, but the solutions for 5GS and EPS need not be the same.

-	The system enablers for Multi-USIM devices are expected to apply for the cases where the multiple USIMs are owned by the same or by different MNOs.

NOTE 3:	While the solutions developed as part of this study might also be applicable to DR-mode (single USIM) interworking between 5GS and EPS, 5GS-EPS interworking use cases with DR-mode (single USIM) are considered outside the scope of this study.

-	The solutions shall not require network coordination for the case where the multiple USIMs in the Multi-USIM device are served by different serving networks.

-	A multi-USIM device with different USIMs may be camping with all USIMs on the same serving network RAN node, or it may be camping on different serving networks RAN nodes.

-	For a multi-USIM device, the solutions may require in-device co-ordination between the UEs represented by each USIM within the multi-USIM device. The mechanism for in-device co-ordination shall be left for the multi-USIM device implementation.

-	MUSIM features requiring new UE-network interactions are optional, hence the UE may use MUSIM features requiring new UE-network interactions in one PLMN when it has learnt that this PLMN supports these MUSIM features.

Based on the architectural requirements above, the multiple USIMs in the Multi-USIM UE may be owned by the same or by different MNOs and the UE may connect over 3GPP access with EPS on one USIM and 5GS on the other USIM.

For the Multi-USIM UE, whether the location service can be improved or not has not been discussed.

This discussion paper is proposed to describe the scenarios that the location service can be improved for Multi-USIM UE, analyse the improvements to location service in the scenarios and propose to add a new key issue about how to improve location service for Multi-USIM UE in FS_eLCS_Ph3 SID.

2. Scenarios

2.1	Scenario#1: Different network deployments supported by Multi-USIM UE registered PLMNs

Positioning accuracy is impacted by the network deployment (e.g. the deployed TRP), the number of Line of Sight (LoS) path, environment and so on.

In this scenario, as shown in Figure 1, the UE has two active USIMs, i.e. USIM1 and USIM2. The UE registers in PLMN1 for USIM1 and registers in PLMN2 for USIM2. The PLMN1 and PLMN2 deploy different TRPs in the access network. Based on the network deployment, to obtain measurements to calculate UE location, there are two TRPs (i.e. TRP1 in Figure 1) in PLMN1 can be used and four TRPs (i.e. TRP2 in Figure 1) in PLMN2 can be used.

Figure 1: Different network deployment supported my Multi-USIM UE registered PLMNs

In this scenario, the LMF in the PLMN2 can obtain positioning measurements from four TRPs, thus it can obtain more measurements to calculate UE location. So the PLMN2 can provide more accurate UE location than PLMN1.

If this information can be considered during the UE positioning procedure, the higher accuracy can be achieved.

2.2	Scenario#2: Different UE positioning capabilities supported by EPS and 5GS

UE positioning capabilities refer to the ability of a UE to support different position methods (e.g. ECID, OTDOA, A-GNSS), different aspects of a particular position method (e.g. different types of assistance data for A-GNSS) and common features not specific to only one position method (e.g. ability to handle multiple LPP transactions). Because different position methods support different location accuracy, different type of assistance data of a particular position method also supports different location accuracy, thus UE positioning capabilities impacts location accuracy.

The UE positioning capabilities for EPS and 5GS may be different. When a Multi-USIM UE connects over 3GPP access with EPS on one USIM and 5GS on the other USIM, different location accuracy can be achieved via different USIM.

In this case, the location accuracy can be improved if the information that the Multi-USIM UE connects over 3GPP access with EPS on one USIM and 5GS on the other USIM and the UE positioning capabilities for EPS and 5GS is considered.

2.3	Scenario#3: Improved location service based on simultaneously UE positioning on active USIMs

When UE or LCS Client requests UE location, based on current specification, the UE positioning procedure is performed via one PLMN. The location accuracy is decided by the PLMN.

For a Multi-USIM UE which has more than one USIM active, if the UE positioning is performed simultaneously on active USIMs, the accuracy of final UE location estimate can be improved by considering the UE locations provided by different PLMNs that UE registered on active USIMs.

2.4	Scenario#4: UE location sharing

For a Multi-USIM UE with two active USIMs, i.e. USIM1 and USIM2, it registers in PLMN1 for USIM1 and registers in PLMN2 for USIM2. When LCS Client1 (MT-LR) or UE (MO-LR) requests UE location from PLMN1 and LCS Client2 (MT-LR) or UE (MO-LR) requests UE location from PLMN2, based on the current specification, the UE positioning procedures are performed in PLMN1&USIM1 and PLMN2&USIM2 separately and independently. The scenario is shown in Figure 2.

Figure 2: separate positioning procedure on active USIMs

In this scenario, if the UE location can be shared between the PLMN1 and PLMN2, then only one positioning procedure via one PLMN needs to be performed. Thus the signalling cost via another PLMN can be avoided and the positioning latency and UE power also reduced.

2.5	Scenario#5: UE LCS privacy profile sharing

For a Multi-USIM UE with two active USIMs, i.e. USIM1 and USIM2, it registers in PLMN1 for USIM1 and registers in PLMN2 for USIM2 as shown in Figure 3. The UDM1 and UDM2 store the UE LCS privacy profile which includes privacy profile in UE subscription data and UE/AF provided location privacy indication.

Figure 3: UE LCS privacy profile sharing

Because the USIM1 and USIM2 in the UE are used by one user, thus it is possible that the privacy profile stored in UDM1 and UDM2 is same, especially for the UE provided location privacy indication. If the common part of the LCS privacy profile can be shared between PLMN1 and PLMN2, then the maintenance cost of the LCS privacy profile can be reduced.

2.6	Scenario#6: PLMN selection for positioning procedure

For a Multi-USIM UE with two active USIMs, i.e. USIM1 and USIM2, it registers in PLMN1 for USIM1 and registers in PLMN2 for USIM2. Because the mobility management in PLMN1 and PLMN2 are independent, it is possible that UE in PLMN1 is in CM_IDLE or RRC_INACTIVE state and UE in PLMN2 is in CM_CONNECTED state.

In this case, if the positioning procedure is performed via PLMN1, then paging (for MT-LR case) or Service Request (for MO-LR case) is needed. But if the PLMN2 is selected, the paging cost can be avoided.

3. Analyses of Scenarios

The scenarios listed in clause 2 are related to positioning for Multi-USIM UE with more than one active USIM. This clause analyses the improvements to location service if the scenarios are supported. The analyses are shown in Table 1.

Table 1: scenarios analyses on the improvements to location service

| Scenarios | #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- | --- |
| Improve location accuracy | Yes | Yes | Yes |  |  |  |
| Reduce signaling cost |  |  |  | Yes | Yes | Yes |
| Reduce positioning latency |  |  |  | Yes |  | Yes |
| Save UE power |  |  |  | Yes | Yes | Yes |

Based on the analyses in Table 1, if the scenarios in clause 2 are supported, the location service for Multi-USIM UE can be improved in the following aspects:

-	Improve location accuracy:

-	PLMN supports higher positioning accuracy is selected to perform UE positioning based on network deployment and UE positioning capabilities for EPS and 5GS, or

-	UE positioning procedures are performed simultaneously via different PLMN that Multi-USIM UE registered on different USIMs and the final UE location is determined based on the UE location provided by different PLMNs.

-	Reduce signalling cost and positioning latency and power saving:

-	UE location and UE LCS privacy profile are shared between PLMNs that Multi-USIM UE registered via different USIMs; or

-	UE positioning procedure is performed via the PLMN which is selected considering UE connectivity status to avoiding paging or service request.

4. Proposal

Based on the analyses in clause 3, this discussion paper proposes to add a new key issue to study how to improve location service for Multi-USIM UE.