3GPP TSG-WG SA2 Meeting #151E 	S2-2204487

Elbonia, May 16th – 20th, 2022	(revision of S2-220xxxx)

Source:	CATT

Title:	KI #3, Sol #6: Update to clarify the additional desired QoS Set

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: Abstract: it is proposed to update the solution with the alternative QoS defined for the GBR QoS Flow to the non-GBR QoS Flow.

- Non-GBR QoS Flow QNC

The solution#6 for the KI#3 is proposed to clone the GBR QoS Flow QNC to the Non-GBR QoS Flow. In order to support such non-GBR QoS Flow QNC, the mean bitrate is proposed as corresponding to the GFBR of the GBR QoS Flow and the averaging window for the non-GBR 5QI is also proposed to be reused to measure the "mean bitrate" as corresponding to the Averaging Window is used to measure the GFBR.

The Alternative QoS Profile for the GBR QoS Flow is also proposed to be cloned for the non-GBR QoS Flow. To make a clear difference between the "Alternative QoS" for the GBR QoS Flow and for the non-GBR QoS Flow, the "additional desired QoS Set(s)", "additional QoS Parameter Set(s)" and "additional desired QoS Profile(s)" is proposed to be used for the non-GBR QoS Flow in different interfaces.

As defined in TS23.503, each Alternative QoS can include its own UL/DL -GFBR, PDB, PER. It is also proposed that the additional desired QoS Set includes UL/DL mean bitrate, PDB, PER and possible the Priority Level and Averaging Windows for the shared 5QI.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-60.

# * * * * First change * * *

## 6.6	Solution #6:Mean bit rate change report

### 6.6.1	Key Issue mapping

This solution is for KI #3 5GS information exposure for XR/media Enhancements.

### 6.6.2	Description

In the Pre-Rel-18, the QoS Notification Control (QNC) with and without alternative QoS profile are defined in clause 5.7.2.4 of TS 23.501 [2].

Since there is no explicit bitrate defined for the non-GBR Flow, but all the non-GBR Flows share the APN-AMBR and UE-AMBR, i.e. in fact, there is the available mean bit rate for the non-GBR flow. The available mean bit rate is the bitrate with a time windows, e.g. averaging windows, i.e. the (default) averaging windows (e.g. 2000ms in the Table 5.7.4-1 of TS 23.501 [2]) is also applicable to the Non-GBR 5QI. The available mean bitrate is the bitrate that can be provided by the non-GBR QoS Flow based on the available scheduling radio resource to the non-GBR QoS Flow, The available mean bitrate can be treated as the near-future bitrate in the radio interface for the non-GBR QoS Flow. Since the APN-AMBR are shared by the non-GBR QoS Flows, the available mean bitrate for a non-GBR QoS Flow can be (near) to the APN-AMBR. If the change of the available mean bitrate is over a threshold based on the available scheduling radio resources for the non-GBR QoS Flow, the RAN will provide a non-GBR QoS Notification Control Report message that "The available mean bitrate has changed to x kbps" to the SMF, PCF and AF (via NEF), the AF can change the codec based on the report available x kbps bitrates.

To control the reporting rates, there are two possible way. First one is that the SMF can provide different bitrate span to the RAN, e.g. the three level bitrate span are 0~500kbps, 500~2500kbps, above 2500kpb ~ Ͼ. Only after the available mean bitrate is changed from one bitrate level to another bitrate level, the RAN can generate the non-GBR QoS Notification Control Report Message. The second on is that the SMF can provide different bitrate changes percentage to RAN, only after the available mean bitrate is changed UP/Down above the change percentage, the RAN will generate the non-GBR QoS Notification Control Report message. One example of 3 level bitrate change percentage can be 50%, 100%, 200%.

Like the alternative QoS, AF can provide different level mean bitrate (the absolute mean bitrate value, or the bitrate change percentage) to the PCF, and PCF generates an 5QI with different level mean bitrates and associated average window to the SMF, then the SMF provides the QoS profile to the RAN with an 5QI with different level mean bitrates and associated average window. When the available mean bitrate is changed to different bitrate, the RAN generates the non-GBR QoS Notification Control as described above.

### 6.6.3	Procedures

#### 6.6.3.1	Non-GBR QoS Notification Control Report

Figure 6.6.3.1-1: Non-GBR QoS Notification Control Report

0.	The PDU Session for the XRM service is established, and a non-GBR QoS Flow is established for the XRM service stream.

1.	The RAN detects the available mean bitrate of the non-GBR QoS Flow to be changed to another level and sends a N2 QoS Notification Message "the available mean bitrate of the Non-GBR QoS Flow changes to x kbps" or "the available mean bitrate of the non-GBR QoS Flow increases or decreases x%" to the AMF. The multiple levels of mean bitrate can be provisioned to the RAN by the SMF during the Non-GBR QoS Flow Establishment procedure or during the Non-GBR QoS Flow Modification procedure. The multiple levels of mean bitrate can be bitrate value in kbps or can be bitrate change percentage.

2.	The AMF sends Nsmf_PDUSession_UpdateSM Context request ("the available mean bitrate of the Non-GBR QoS Flow changes to x kbps" or "the available mean bitrate of the non-GBR QoS Flow increases or decreases x%") to the SMF.

3.	The SMF sends Npcf_SMPolicyControl_Update request ("the available mean bitrate of the SDF changes to x kbps" or "the mean bitrate of the SDF increases or decreases x %") to the PCF.

4.	If NEF is used, the PCF sends the Npcf_PolicyAuthorization_Notify ("the available mean bitrate of the SDF changes to x kbps" or "the available mean bitrate of the SDF increases or decreases x %") to the NEF.

If the AF locates the trusted domain and the NEF is not used, the PCF sends the Npcf_PolicyAuthorization_Notify ("the available mean bitrate of the SDF changes to x kbps" or "the available mean bitrate of the SDF increases or decreases x %") to the AF.

5.	After receiving the Npcf_PolicyAuthorization_Notify from the PCF, the NEF sends the Nnef_EventExposure ("the available mean bitrate of the stream changes to x kbps" or "the available mean bitrate of the stream increases or decreases x %") to the AF.

6.	After receiving the information from PCF/NEF, the AF can determine to change the codec with indicated bitrate.

#### 6.6.3.2	Non-GBR QoS Notification Control

Figure 6.6.3.2-1: Non-GBR QoS Notification Control

- The PDU Session for the XRM service is established.

- If the XRM AF does not locate at trusted domain, the AF sends the Nnef_AFsessionwithQoS_Create Request (XRM stream information and request QoS and additional desired QoS Set(s)) to the NEF. The request QoS and additional desired QoS set(s) share the same 5QI, ARP and each additional desired QoS Set includes a mean bitrate indicate the different possibilities for the codec and a PER, PDB, Priority Level and Averaging Window for the shared 5QI).

- If the XRM AF locates at trusted domain, the AF sends the Npcf_PolicyAuthorization_Create Request (XRM stream information and request QoS and additional desired QoS Set(s)) to the PCF.

- If the NEF is used, the NEF sends The AMF sends ends the Npcf_PolicyAuthorization_Create Request (XRM stream information and request QoS and additional desired QoS Set(s)) to the PCF as described in step 1b.

- The PCF sends Npcf_SMPolicyControl_UpdateNotify request (SDF PCC rule) to the SMF. The SDF PCC rule also includes the additional desired QoS Parameter Set(s) (e.g. multiple mean bitrate(s), PER(s), PDB(s), Priority Level(s) and Averaging Window(s) for the associated 5QI).

- The SMF creates a new non-GBR QoS Flow based on received PCC rule and sends the Namf_Communication_N1N2MessageTransfer request (N1 Message, N2 Message (QFI, non-GBR QoS profile with the 5QI , ARP , additional desired QoS Profiles (e.g. mean bitrate(s), PER(s), PDB(s), Priority Level(s) and Averaging Window(s) for the associated 5QI))).

- AMF sends the N2 Message (QFI, non-GBR QoS profiles with the same 5QI, ARP, and additional desired QoS Profile(s)) to the RAN to create a non-GBR QoS Flow. After the Non-GBR QoS Flow is established, the RAN will generate the current available mean bitrate and report the desired mean bitrate that is closest to it to the AMF as described in clause 6.6.3.1 and the AF may change its codec based on the desired mean bitrate reported from the 5GS.

When the available mean bitrate changes in such a way that a different desired mean bitrate is closest to it, the RAN generates the non-GBR QoS Flow Notification message with the new desired mean bitrate as described in clause 6.6.3.1 and the AF can change its codec based on the desired mean bitrate reported from the 5GS.

### 6.6.4	Impacts on services, entities and interfaces

AF:

-	Provides the stream QoS requirements which include additional desired QoS Set(s) (e.g., mean bitrate(s), PER(s), PDB(s), Priority Level(s), Averaging Window(s)).

-	Change its codec based on the available mean bitrate or the desired mean bitrate reported from 5G network.

PCF:

-	The PCC rules for the non-GBR 5QI include additional desired QoS Parameter Set(s) (e.g., mean bitrate(s), PER(s), PDB(s), Priority Level(s), Averaging Window(s)).

-	Forwards the current available mean bitrate or the desired mean bitrate from the SMF to the AF (via NEF).

SMF:

-	Creates a non-GBR QoS Flow with additional desired QoS Parameter Set(s) based on the PCC rules from PCF.

-	Forwards the current available mean bitrate or the desired mean bitrate from the RAN to the PCF.

RAN:

-	Creates a non-GBR QoS Flow with additional desired QoS profile(s) in the QoS profile.

-	Measures the current available mean bitrate and reports the current available mean bitrate or the desired mean bitrate to the SMF.