3GPP TSG-SA WG2 Meeting #151E (e-meeting)	S2-2204489

# May 16 – 20, 2022, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR |  | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.503 –Supporting PCF to make AM policies decision based on UE’s accumulated usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_DCAMP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Policy decision based on usage monitoring control requirement is a function that allows PCF taking actions related to the accumulated usage of network resource. In Rel-17, the specification only defines usage monitoring for session management related policy control requirement and does not elaborate on how PCF for UE could retrieve usage monitoring report related to AM policies when PCF for PDU session and PCF for UE are not the same one. See more in DP S2-2204434 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new requirement in clause 4.2.1 to indicate that PCF for UE can make AM policies decisions based on UE’s accumulated usage when the PCF for UE and PCF for PDU Session are not the same one |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PCF for UE can’t make dynamic AM policy decisions based on UE’s accumulated usage when PCF for UE and PCF for PDU Session are not the same one. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

## 4.2	Non-session management related policy control requirements

### 4.2.1	Access and mobility related policy control requirements

The policy framework shall provide following functionality for the access and mobility enforcement:

-	Policy Control Function (PCF) shall support interactions with the access and mobility policy enforcement in the AMF, through service-based interfaces.

-	The PCF shall be able to provide Access and Mobility Management related policies to the AMF.

-	The PCF shall be able to evaluate operator policies that are triggered by events.

-	The AF may influence Access and Mobility related policy control performed by the PCF.

-	The PCF for the UE shall be able to make Access and Mobility Management related policies decisions based on UE’s accumulated usage when PCF for the UE and PCF for the PDU Session are different.

* * * * End of Changes * * * *