SA WG2 Meeting #151e	S2-2204514

May 16th – 21th, 2022	  (revision of S2-220xxxx)

Source: 	China Mobile

Title: 	KI #2, New Sol: Support QoS Monitoring results exposure by UPF

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS/ Rel-18

Abstract of the contribution: Solution proposal for UPF exposing QoS Monitoring results to AF/Local NEF/NEF directly.

1 Discussion

The solution proposal presented here addresses the Key Issue #2, which reads:

-	How and what specific information the UPF can expose to NEF/Local NEF/trusted AF, e.g. the information which can be exposed in Rel-18 SA WG2 studies such as FS_EDGE_Ph2 and FS_XRM, or information which has been justified for exposure.

This paper proposes to support the UPF exposes QoS Monitoring results to NEF directly to facilitate the local AF requesting information with low latency.

2 Proposal

It is proposed to update TR 23.700-62 v0.2.0 as follows.

****************************First change****************************

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1: UPF event exposure service framework enhancements to support registration, deregistration and discovery via NRF | X |  |
| #2: UPF event exposure service for TSC management |  | X |
| #3: using the proper subscription mechanism depending on the event targeted by the UPF event consumer |  | X |
| #4: upgrading N4 to pass necessary event filtering information to the UPF |  | X |
| #5: registering UPF(s) serving a PDU session at UDM |  | X |
| #6: Determining the UPF(s) that serve a UE address |  | X |
| #7: Support to existing (R16-R17) data analytics with PDU Session Data Usage Events |  | X |
| #8: Support to existing (R16-R17) data analytics with QoS Flow level measurements |  | X |
| #9: NWDAF collects information from UPF by event exposure |  | X |
| #10: UPF event exposure service to NWDAF |  | X |
| #11: UPF event exposure service to NWDAF |  | X |
| #X: QoS Monitoring results exposure by UPF |  | X |

****************************Second change (all text is new) ****************************

6.X	Solution #X: QoS Monitoring results exposure by UPF

6.X.1	Key Issue mapping

This is a solution for KI#2.

6.X.2	Description

Some real time network information, e.g. user path latency, is useful for application layer. In R17, in order to expose network information timely to local AF, the L-PSA UPF may expose network information i.e. QoS monitoring results as defined in TS 23.501 [2], clause 5.33.3, to the local AF.

The UPF may be instructed to report information about a PDU Session directly to the local NEF/NEF/AF i.e. by passing the SMF and the PCF. The PSA UPF may support Nupf_EventExposure_Subscribe service operation.

6.X.3	Procedures

6.X.3.1	QoS Monitoring results subscription directly from the UPF under the same PCF policy

Figure 6.X.3.1-1: QoS Monitoring results subscription directly from the UPF under the same PCF policy

0.	The UE establishes a PDU Session and UPF notifies the QoS monitoring information to Local NEF/NEF/AF as described in step 0-5 in clause 6.4.2.1 of TS 23.548 [7]. The L-PSA UPF sends the direct subscription notification to the local AF or the local NEF together with the notification related with QoS monitoring information over Nupf_EventExposure_Notify service operation.

1.	(when the reporting goes via local NEF) For the subsequent AF request to subscribe direct notification of QoS monitoring for the same service data flow via Local NEF, the local NEF may directly invoke the Nupf_EventExposure_Subscribe service operation.

The L-UPF sends the notification related with QoS monitoring information over Nupf_EventExposure_Notify service operation to the local NEF, and local NEF reports to AF.

6.X.3.2	QoS Monitoring results subscription directly from the UPF without PCF policy control

Figure 6.X.3.2-1: QoS Monitoring results subscription directly from the UPF without PCF policy control

0.	The AF finds the appropriate UPF(s) by using the UPF selection method as described in clause 6.1.2.5.

1.	The AF sends an Nupf_EventExposure_Subscribe request to the UPF possibly via Local NEF or NEF for QoS monitoring results exposure. The request includes the AF identity information (e.g., IP address, FQDN).

2.	The UPF reports the indication of direct event notification to the SMF.

The SMF may activate the end to end UL/DL packet delay measurement between UE and PSA UPF for a QoS Flow to make the UPF obtain the QoS monitoring information, as described in TS 23.501 [2] clause 5.33.3.

3.	The UPF sends an Nupf_EventExposure_Subscribe response message to the AF possibly via Local NEF or NEF. The response includes an indication to whether the request in step 1 is successful or not. If the subscription request is not successful, the response includes a Cause value indicating that the subscription failed.

4.	The UPF sends the notification related with QoS monitoring information over Nupf_EventExposure_Notify service operation to the AF or the Local NEF/NEF.

If Local NEF/NEF is used, it reports the QoS monitoring information to AF by invoking Nnef_EventExposure_Notify service operation.

6.X.4	Impacts on services, entities and interfaces

UPF:

-	Newly introduced UPF Service Operations to support AF/Local NEF/NEF to subscribe the UPF event exposure service directly.

-	Expose QoS monitoring information to AF/Local NEF/NEF directly.

SMF:

-	Activate the end to end packet delay measurement based on the UPF’s indication.

**End of change**