3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204525

Elbonia, 16th – 20th May, 2022	(revision of S2-220xxxx)

Source:	ZTE

Title:	FS_eNS_Ph3 KI#4 Update on Solution 14

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This paper updates solution 14 to resolve FFS

1. Introduction/Discussion

This paper propose solution update to support the service continuity issue and resolve FFS in solution 14.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41 V0.2.0.

# * * * * First change * * * *

6.14	Solution #14: Maximum Number Distribution in multiple NSACFs

6.14.1	Introduction

This solution aims to address the KI#4: Support of NSAC involving multi service Area. This solution can be applicable to both non roaming scenario and roaming scenario.

6.14.2	Functional Description

In this solution, there is one centralized NSACF controlling the overall maximum number of UEs and maximum number of PDU Sessions of the S-NSSAI. The distributed NSACF requests the local maximum number from the centralized NSACF. The centralized NSACF may also update the local maximum number to the related NSACFs,

In roaming case the NSACF in VPLMN interacts with the centralized NSACF in the HPLMN to retrieve the local maximum number and perform the NSAC locally.

The central NSACF may invoke Nnsacf_SliceEventExposure service to request the number of UE and number of PDU session in each distributed NSACF.

In order to support service continuity, if the requested S-NSSAI is included in the old Allowed NSSAI received from the old AMF, the new AMF sets the update flag value to “update” in the NSAC procedure. The NSACF adds the UE ID into the list of UEs registered with the network slice and increases the number of the UE registered in the network slice. However the NSACF skips the checking whether the maximum number of UEs per network slice for that S-NSSAI is reached or not. This means that the NSACF performs maximum number checking only when the UE performs initial registration in the network slice.

NOTE:	The central NSACF manages the overall maximum number of UEs and maximum number of PDU Sessions of the S-NSSAI, and may perform NSAC procedure as the distributed NSACF.

6.14.3	Procedure

6.13.3.1	Procedure for Maximum Number Distribution in multiple NSACFs

Figure 6.14.3-1: Procedure for Maximum Number Distribution in multiple NSACFs

1.	When there is no local maximum number in NSACF, or the number of UE or number of PDU session of the S-NSSAI will reach the local maximum number in the NSACF , the NSACF sends Local Maximum Number Request to centralized NSACF to request a new local maximum number.

2.	If the request is accepted, the central NSACF sends a new maximum number to the NSACF. The NSACF stores the new local maximum number of the S-NSSAI.

3.	At any time the centralized NSACF may send Local Maximum Number update message to update the local maximum number in the NSACF which has requested the local maximum number.

4.	The NSACF stores the new local maximum value and send response message to the centralized NSACF.

6.13.3.2	Procedure for service continuity across multiple NSACFs

Figure 6.14.3-2: Procedure for service continuity across multiple NSACFs

1.	During UE mobility the old AMF transfers the UE context to new AMF by invoking Namf_Communication_CreateUEContext service operation. The UE context includes the old Allowed NSSAI

2.	If the Requested S-NSSAI is included in the old Allowed NSSAI, when the new AMF invokes the Nnsacf_NSAC_NumberOfUEsUpdate service operation, the new AMF sets the update flag to "update". In this case the NSACF add the UE ID into the list of UEs registered with the network slice and increase the number of the UE registered in the network slice, without checking whether the maximum number of UEs per network slice for that S-NSSAI is reached or not.

3.	If the distributed NSACF return nsacf_NSAC_NumberOfUEsUpdate response to new AMF. The new AMF then can continue the UE registration procedure.

NOTE 1:	The number of UE in new NSACF may go over the local maximum number of UE because of UE mobility from old AMF. However, the overall number of UE within the network slice will not exceed the maximum value since the old AMF will decrease the number of UE in the old NSACF.

NOTE 2:	The new NSACF may request new local maximum number from the centralized NSACF before the local maximum number is reached. This will trigger the centralized NSACF to update each distributed NSACF in order to reach the balance in the whole network slice.

6.14.4	Impacts on services, entities and interfaces

NSACF:

-	new interaction between NSACFs for maximum number distribution.

-	support of new update flag

AMF:

-	support of new update flag

# * * * * End of changes * * * *