3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204526

Elbonia, 16th – 20th May, 2022	(revision of S2-220xxxx)

Source:	ZTE

Title:	FS_eNS_Ph3 KI#5 New solution Rejected S-NSSAI with new cause value

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This paper propose a new solution for KI#5

1. Introduction/Discussion

This paper propose a new solution for KI#5.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41 V0.2.0.

# * * * * First change * * * *

6.14	Solution #x: Rejected S-NSSAI with new cause value

6.14.1	Introduction

This solution aims to address the KI#5: Improved support of RAs including TAs supporting Rejected S-NSSAIs. In particular, this solution proposes new cause value for the Rejected NSSAI so the UE can differentiate the rejection cause and initiate a registration for an S-NSSAI when the UE enters a TA that is part of the RA and the TA supports this S-NSSAI.

6.14.2	Functional Description

During UE registration, the UE sends Requested NSSAI in RRC message to the RAN. The RAN selects the AMF based on the Requested NSSAI. The selected AMF may not support all S-NSSAIs in the Requested NSSAI. In previous releases, the AMF rejects the unsupported S-NSSAI with cause value “rejected for the RA” and the UE shall not request this S-NSSAI again before the UE moves outside of the RA. This solution introduces a new cause value “rejected due to AMF not support”, so the UE can still request the rejected S-NSSAI in UE registration procedure without moving outside of the RA, and the RAN may select a new AMF, which supports this S-NSSAI.

In another case, if the requested S-NSSAI is not supported in some TAs but is supported in other TAs within the RA, the AMF rejects the unsupported S-NSSAI with cause value “rejected for the RA” and the UE shall not request this S-NSSAI again before the UE moves outside of the RA. This solution introduces a new cause value “rejected for TA list” and a new list of TAs which are part of RA and support the rejected S-NSSAI. So the UE can request the rejected S-NSSAI in UE registration procedure when it enters the area of the TA list within the RA.

6.14.3	Procedure

During UE registration procedure, the AMF provides new cause values for the rejected S-NSSAI in the Registration Accept/Reject message to UE

During UCU procedure, the AMF provides new cause values for the rejected S-NSSAI in the UE Configuration Update message to UE

6.14.4	Impacts on services, entities and interfaces

AMF:

-	providing two new cause values for the rejected S-NSSAI

-	providing optionally a list of the TA which are part of RA and supports the rejected S-NSSAI

UE:

-	support handling of new cause value

# * * * * End of changes * * * *