SA WG2 Meeting #151E (e-meeting)	S2-2204534

May 16 – 20, 2022	(was S2-220XXXX)

Source:	vivo

Title:	KI#8, Sol#33: Update to remove ENs

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: it proposes update Sol#33 to clarify the periodicity related and remove the EN.

Discussion

So far, both Sol#33 and sol#34 consider to convey periodicity to RAN to optimize the CDRX configuration. But there is an EN unfixed in Sol#33, which is also applicable for Sol#34.

# Editor’s Note: It is FFS whether the existing TSCAI can be used to convey the UL/DL Traffic Pattern information.

During display, frame rate (FPS) can be considered as periodicity of frames. but during the transmission, the frames may or may not have stable periodicity

Observation1: the transmission of frames may or may not have periodicity.

Let’s focus on the scenario that the transmission of frames has periodicity and see whether the existing TSCAI can be used or not. The existing TSCAI support to provide a per QoS flow’s periodicity to the RAN.

Table 5.27.2-1: TSC Assistance Information (TSCAI)

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two data bursts. |
| Burst Arrival Time (optional) | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or the egress interface of the UE (uplink flow direction). |
| Survival Time (optional) | Survival Time, as defined in TS 22.261 [2], is synonymous with the time period an application can survive without any data burst. |

Figure-1

As Figure-1 shown, frames transmission has periodicity, but the GOP is not stable (i.e. the continuous GOPs are not the same), then I frame doesn’t have the periodicity, so does P frame or B frame.

- If different kinds of frames are mapped to different QoS flows, the periodicity is applicable to a group of QoS flows.

- If different kinds of frames are mapped to same QoS flows, the periodicity is applicable to a group of QoS flows. Then the per QoS flow periodicity cannot be reused.

Observation2: when frames transmission has periodicity, but the GOP is not stable, then different kinds of frames doesn’t have periodicity. If different kinds of frames are mapped to different QoS flows, the periodicity is applicable to a group of QoS flows. Then the per QoS flow periodicity cannot be reused.

Figure-2

As Figure-2 shown, frames transmission has periodicity and GOP is stable (i.e. the continuous GOPs are the same), then each kind of frame (e.g. I frame, P frame or B frame) has the periodicity but may have multiple burst arrival time.

- No matter different kinds of frames are mapped to different QoS flows or one QoS flow, the per QoS flow’s periodicity can be reused

- If different kinds of frames are mapped to different QoS flows, the per QoS flow’s burst arrival time need to be extended to multiple ones, e.g. the B frame in Figure-2)

Observation3: when frames transmission has periodicity and GOP is stable, then each kind of frame has the periodicity. No matter different kinds of frames are mapped to different QoS flows or one QoS flow, the per QoS flow’s periodicity can be reused. If different kinds of frames are mapped to different QoS flows, the per QoS flow’s burst arrival time need to be extended to multiple ones.

Proposal: it is proposed the consider multiple burst arrival times in a QoS flow, per QoS flow periodicity and per QoS flow group periodicity in different scenarios.

Proposal

It is proposed to add the following solution to the TR 23.700-60 "Study on architecture enhancement for XR and media services"

START OF CHANGES

6.33	Solution #33: support of CDRX enhancement for power saving handling

6.33.1	Key Issue mapping

This solution is for Key Issue #8, which address the assistance to RAN for CDRX configuration and potential enhancement for power saving handling, including:

-  What information (if any) can be used to assist the CDRX enhancement?

-	Where does such information come from?

6.33.2	Description

In order to support the CDRX enhancement for power saving handling, the CN can send the UL/DL traffic patterns to assist RAN for CDRX setting and configuration.

1.	The UL/DL traffic patterns may include the traffic periodicity, etc. which can be provisioned by the third-party AF or derived by UPF as the statistic results.

2.	During PDU Session Establishment/Modification procedure, SMF receives the UL/DL traffic patterns from PCF or UPF and delivers the UL/DL traffic patterns to RAN, which can be taken into consideration when setting CDRX.

3.	Optionally, the UPF may detect the traffic bursts and add marks in the GTP-U layer of DL packets to assist RAN with identifying the end of the traffic bursts.

4.	With the awareness of the UL/DL traffic patterns, the RAN can configure and enable the CDRX accordingly.

# NOTE: How to use the UL/DL traffic patterns for CDRX setting/configuration depends on RAN WG.

6.33.3	Procedures

Figure 6.33.3-1 shows how 5GS acquires and distributes the UL/DL traffic patterns to RAN for support of power saving handling.

# Figure 6.33.3-1 UL/DL traffic pattern provisioning from AF

1a. Optionally, the AF may provision the UL/DL traffic patterns to PCF. The traffic patterns may include UL/DL traffic periodicity, traffic description to localize the target application traffic, etc.

1b.	During PDU Session Establishment/Modification, PCF may send the UL/DL traffic patterns to SMF via the PCC rule.

1c.	Besides, the UPF may optionally derive the UL/DL traffic patterns based on the statistical analysis, e.g. via the identification mechanisms detailed in Sol#X (S2-2202408), up to UPF implementations. The UL/DL traffic patterns may include the UL/DL traffic periodicity, etc.

2.	SMF generates the UL/DL traffic patterns and sends that to RAN. Optionally, the burst detection instruction for the UL/DL traffic is also sent to RAN by SMF. The UL/DL traffic periodicity may be used for CDRX cycle determination.

When different kinds of PDU Sets (e.g. related to I-frames, B-frames, P-frames) bind to different QoS flows, the traffic periodicity is applicable to the group of the QoS flows.

When different kinds of PDU Sets (e.g. related to I-frames, B-frames, P-frames) bind to the same QoS flow, the traffic periodicity is applicable to the group of the QoS flows.

3.	Optionally, SMF may send the N4 rule to UPF. The N4 rule may include the burst marking rule, which can help the UPF to mark the DL traffic burst accordingly. If Step 3 is not performed, the N4 rule may also include the burst detection rule to assist UPF for burst detection.

4.	Complete the remaining steps of PDU Session Establishment/Modification procedure.

5.	After the target service started, the UPF may detect the periodic burst and mark the DL burst end in the GTP-U header based on the DL traffic rule. Then the DL packets with marks in the GTP-U header can help RAN to identify the end of the DL traffic bursts.

6.	Based on the UL/DL traffic pattern, the RAN can determine the end of the traffic bursts. For example, based on the end marks in the GTP-U header, the RAN can determine the current burst transmission is over and enable the CDRX, e.g. sending DRX Command MAC CE to trigger the start of CDRX.

7.	Based on the UL/DL traffic patterns, RAN enables the CDRX setting/configuration accordingly.

6.33.4	Impacts on services, entities and interfaces

# Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

PCF:

-	Receive the traffic patterns from AF and send it to SMF.

SMF:

-	Receive the UL/DL traffic patterns from PCF or UPF.

-	Generate the UL/DL traffic patterns and send it to RAN.

-	Send N4 rule to assist UPF for burst detection.

UPF:

-	Statistically collect and report the UL/DL traffic patterns.

-	Report the statistical UL/DL traffic patterns to SMF.

-	Detect and mark the DL traffic burst as instructed by SMF.

RAN:

-	Receive the UL/DL traffic patterns.

-	Detect the end of DL traffic burst as instructed by SMF.

END OF CHANGES