3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204542

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI #8, New solution: LCS session continuity due to UE mobility

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	5G_eLCS_Ph3 / Rel-18

Abstract: this is the new solution for LCS session continuity

1. Introduction/Discussion

This pCR proposes a new solution to address key issue 8.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change (all texts new)* * * *

## 6.X	Solution #X: LCS session continuity due to UE mobility

### 6.X.1	Key Issue mapping

This solution address key issue#8: LCS session continuity.

### 6.X.2	Description

- When N26 interface is supported, and UE performs interworking procedure from 5GS to EPS, AMF will send the UE location context to MME.

- MME, based on the UE location context, determines an E-SMLC which handles the deferred location request for the UE. E-SMLC is obtained of the LCS correlation ID retrieved from the AMF, and sends an LPP message to UE with the same correlation ID.

- UE continues to report measurement report to E-SMLC, same as it reports to LMF in the 5GS.

### 6.X.3	Procedures

Figure 6.X.3-1 Support of service continuity when UE changes serving cell

- Same as step 1-13 in clause 6.3.1.

- LMF sends Namf_EventExposure subscribe (LCS correlation ID, event type=UE location (cell ID)) to AMF.

- AMF sends location reporting control message to NG-RAN, to request for report of change of UE cell ID, as defined in TS 38.413[X].

- NG-RAN detects the UE changes its serving cell.

- NG-RAN reports the latest cell ID of the serving cell to AMF.

- AMF sends Namf_EventExposure Notify message (cell ID, LCS correlation ID) to LMF.

- If UE handovers to a target NG-RAN.

- AMF obtains the cell ID of the serving cell in the target NG-RAN.

- AMF sends Namf_EventExposure Notify (cell ID, LCS correlation ID) to LMF.

### 6.X.4	Impacts on Services, Entities, and Interfaces

# * * * * Second change * * * *

6.0	Mapping Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |  |  |  |
| 1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| 18 |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| x |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |

# * * * * End of changes * * * *