3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204544

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI #2, New Solution: SIB-based UAV remote ID broadcasting

Document for:	Approval

Agenda Item:	9.5

Work Item / Release:	FS_UAS_Ph2 / Rel-18

Abstract: this is a new solution for UAV remote ID broadcasting

1. Introduction/Discussion

This pCR proposes SIB-based UAV remote ID broadcasting

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-58.

# * * * * First change (all texts new)* * * *

## 6.X	Solution #X: SIB-based UAV remote ID broadcast

### 6.X.1	Key Issue mapping

This solution address key issue#2: Support of Broadcast Remote ID

### 6.X.2	Description

Following are the principles of the solution:

- UAV identification can be GPSI or CAA-level UAV identity

- UAS NF needs to support:

  - Rel-17 UAV tracking mechanism, including UAV location reporting, UAV presence monitoring, and list of Aerial UEs in a geographic area.

  - (new capability) determine what is to be broadcast in a container, transparent to gNB.

- UAS NF transfer a UAV container to RAN by invoking the AMF_nonUE N2 message transfer service. AMF further forwards the UAV container to RAN using an NG-AP non-UE associated signal/procedure.

- The AMF selected by the UAS NF can be any AMF.

- UAS NF is able to determine the appropriate scope of gNBs to do the broadcast

- New SIB message needs to be defined by RAN for broadcasting the received container

### 6.X.3	Procedures

Figure 6.X.3-1 SIB-based UAV remote ID broadcast

### 6.3.4	Impacts on Services, Entities, and Interfaces

UAS NF:

- Deliver the UAV container to RAN for broadcast.

RAN:

- New SIB message used for UAV remote ID broadcast.

# * * * * End of changes * * * *