3GPP TSG-SA WG2#151E e-meeting	S2-2204589

Elbonia, 16-20 May 2022

Source:	Nokia, Nokia Shanghai Bell, Ericsson

Title:	Conclusions for KI#2

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper proposes conclusions and way forward for KI#2 in TR 23.700-25

# 1.	Discussion

After SA2#150E two solutions (solution #6 and #7) were proposed and agreed to address KI#2 in TR 23.700-25. Both solutions share the same fundamental principles and in recent updates for SA2#151E, solution #7 is converging to a new merged version including inputs from solution #6. Therefore, it is our understanding that there is a common agreement on how to address KI#2 at this state of the study.

# 2.	Text proposal

In line with the proposals made in the previous clause, it is proposed to agree the following changes to TR 23.700-25:

>>>> Start of Changes <<<< (all new text)

# 8	Conclusions

### 8.x	Key Issue #2: Time synchronization service enhancements

Solution #7 is taken as the basis for the normative work to address KI#2. The following bullet points summarize the principles for the way forward:

- To request a Requested Coverage Area for time synchronization services, AFs within the operator’s domain formulates a spatial validity condition using a list of Tracking Area or Cell identities, but AFs outside the operator’s domain use a geographical area (e.g., a civic address or shapes) instead, while NEF transforms this information into a list of Tracking Area or Cell identities.

- The TSCTSF is responsible of enable/disable time synchronization services (ASTI or (g)PTP based) considering the spatial validity condition provided by the AF. To enable/disable time synchronization services, TSCTSF reuses Release-17 time synchronization methods.

-	The TSCTSF discovers the AMF(s) serving the list of TA(s)/Cell(s) that comprise the spatial validity condition using the NRF.

-	The TSCTSF subscribes to UE’s presence in Area of Interest at the discovered AMF(s).

-	The TSCTSF determines (based on notifications from the discovered AMF(s)) whether the targeted UE(s) are inside the AF Requested Coverage Area.

-	The TSCTSF activates or deactivates time synchronization services for UE(s) inside or outside the Requested Coverage Area, respectively.

>>>> End of Changes <<<<