3GPP SA WG2 Meeting #151E	S2-2204612

e-meeting, May 16-20, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#4, New sol: PDU Set integrated packet handling with QoS differentiation

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This paper proposes a solution for the Key Issue #4 “PDU Set integrated packet handling” and Key Issue # 5, “Differentiated PDU Set Handling” for inclusion in TR 23.700-60.

# Discussion

This solution proposes to capture in a solution the basic components needed to support Key Issue #4 and Key Issue #5.

# Proposal

It is proposed to add the following solution to TR 23.700-60.

*** Start of changes ***

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| #1:Tactile and multi-modality traffic flows coordination transmission for one UE | X |  |  |  |  |  |  |  |  |
| #2: Group policy for Multi-modal Traffic among Multiple UEs |  | X |  |  |  |  |  |  |  |
| #3: Same PCF selection for Multiple UEs with XRM services |  | X |  |  |  |  |  |  |  |
| #4：QoS policy coordination for multiple UEs' QoS flows |  | X |  |  |  |  |  |  |  |
| #5: Exposure of the UE data rate, normal data transmission interruption event and congestion information |  |  | X |  |  |  |  |  |  |
| #6: Mean bit rate change report |  |  | X |  |  |  |  |  |  |
| #7: Identification and importance of packets in PDU set |  |  |  | X | X |  |  |  |  |
| #8: Introduction of PDU Set information in RTP extension header and related QoS parameters |  |  |  | X |  |  |  |  |  |
| #9: GTP extension header for PDU Set Sequence Number, and, Number of PDUs in the PDU Set |  |  |  | X |  |  | X |  |  |
| #10: different bearers for PDUs with different importance levels |  |  |  | X | X |  |  |  |  |
| #11: Handling PDU Set within QoS flow |  |  |  | X | X |  |  |  |  |
| #12: PDU Set integrated QoS handling |  |  |  | X |  |  |  |  |  |
| #13: DL data handling |  |  |  |  | X |  |  |  |  |
| #14: PDU Set integrated packet handling |  |  |  | X |  |  |  |  |  |
| #15: Leverage RTP layer info for PDU Set handling |  |  |  | X | X |  |  |  |  |
| #16: RTP/SRTP based PDU Set identification |  |  |  | X | X |  |  |  |  |
| #17: Sub QoS Flow based QoS and PCC Architecture |  |  |  | X | X |  |  |  |  |
| #18: PDU Set Identification and Marking |  |  |  | X | X |  |  |  |  |
| #19: Application-aware QoS |  |  |  | X | X |  |  |  |  |
| #20: UPF based PDU Set identification |  |  |  | X | X |  |  |  |  |
| #21: PDU Set ID in the GTP-U header |  |  |  | X |  |  |  |  |  |
| #22: N6 tunnelling and GTP-U header extension for conveyance of PDU Set-related information |  |  |  | X | X |  |  |  |  |
| #23: PDU set related packet handling enhancements |  |  |  | X |  |  |  |  |  |
| #24: Support differentiated QoS Handling for different PDU Sets |  |  |  |  | X |  |  |  |  |
| #25: PDU Set QoS Profile, QoS Flow parameters and QoS Characteristics |  |  |  | X |  |  |  |  |  |
| #26: Provisioning XR traffic configuration to 5GS |  |  |  | X | X |  |  |  |  |
| #27: RT latency split for two QoS flows used for UL and DL respectively |  |  |  |  |  | X |  |  |  |
| #28: RAN split RT latency for single QoS flow |  |  |  |  |  | X |  |  |  |
| #29: Two Way Delay Budget |  |  |  |  |  | X |  |  |  |
| #30: Policy enhancements to minimize jitter |  |  |  |  |  |  | X |  |  |
| #31: 5GS Policy enhancements to minimize the jitter |  |  |  |  |  |  | X |  |  |
| #32: Periodicity Jitter Control for XRM Service |  |  |  |  |  |  | X |  |  |
| #33: Support of CDRX enhancement for power saving handling |  |  |  |  |  |  |  | X |  |
| #34:5GC and UE Assistance to RAN for CDRX Optimization |  |  |  |  |  |  |  | X | X |
| #35: Provision of conditional QoS profile(s) |  |  |  |  |  |  |  |  | X |
| #x: PDU Set integrated packet handling with QoS differentiation |  |  |  | X | X |  |  |  |  |

*** Next change ***

## 6.x	Solution #x:  PDU Set integrated packet handling with QoS differentiation

### 6.x.1	Introduction

To support PDU Set integrated packet handling with QoS differentiation, this solution proposes key principles. The general architecture to support this is shown in Figure 6.x.1-1.

Three principles are adhered to:

- PDU Set QoS treatment is determined by policy using dynamic or non-dynamic PCC. Policy is enforced at the RAN based on NG-AP signaling between the RAN and SMF. User plane packet marking at the UPF should indicate the characteristics of a given PDU, not QoS handling such as the “priority” to be provided to a PDU. Note that “priority level” is already a QoS parameter sent via NG-AP.

- RTP header extensions for scalable media streams are proposed by the IETF in draft-ietf-avtext-framemarking. The header extension identifies the Temporal and Spatial media layers associated with a packet as well as dependencies of enhancement layer instances on base layer instances. This allows layers to be selectively forwarded or otherwise handled in the network. The solution adopted for this Key Issue should provide comparable capability for the 5GS.

- In the 5GS, a QoS Flow can carry multiple application flow instances of the same type (e.g. two H.26x video streams) as well as application flow instances of different types (e.g. H.26x video, AC Audio, haptic data). To maintain this capability while providing per-PDU set handling, it must be possible to distinguish between these application / media sub-flows within a QoS Flow. One option for doing so is to consider the Application / media-stream / sub-flow as a PDU Set to which a PDU may belong, and to identify it as such in the GTP-U header.

Figure 6.x.1-1: 5GS Support for PDU Family integrated packet handling

### 6.x.2	Description of Solution Components

1. Packet Identification and Marking

-  The rules that should be used by UPF for identifying the PDU sets to which a packet belongs are either provisioned in the UPF or received via N4 from the SMF. PDU Set identification rules provisioned on the UPF or sent by the SMF may specify the RTP header type to be detected.

-  Which PDU Sets the UPF identifies is provisioned on the UPF, and may include one or more of:

1.  A Frame / Slice PDU Set

All PDUs that carry segments of the same Frame / Slice belong to the same set.

2.  A Temporal Layer PDU Set

All PDUs that carry Frames / Slices that belong to the same Temporal Layer belong to the same set.

3.  A Spatial Layer PDU Set

All PDUs that carry Frames / Slices that belong to the same Spatial Layer belong to the same set.

4.  An Application / Media-Stream / Sub-Flow PDU Set

All PDUs that carry Packets / Frames / Slices that belong to same application or media stream (e.g., audio or video or haptics) belong to the same set.

-	Packets are identified and the GTP-U header extension is populated by examining PDU RTP header fields, when present.

-  PDU Set identification and marking is supported for downlink packets

Editor’s Note: Support for uplink is FFS

-

-  To support scalable / layered media or multiple media streams mapped into the same QoS flow (because they are multiplexed into the same UDP/IP traffic flow), a PDU may be identified at the UPF and marked as belonging to more than one PDU Set. Thus, the UPF can also identify one or more PDU Sets to which a packet belongs.

For example, an IP packet that carries a segment of a video frame belonging to a specific Spatial Layer and a specific Temporal Layer, would belong to all four PDU Set Types and could thus receive four different PDU Set markings, which could all affect the QoS treatment of the packet depending on the QoS policy.

-  For each PDU Set to which a PDU is associated, the following information may be provided in a GTP-U header extension

1.  The PDU Set Type:

- Frame or Slice type (e.g., I-frame or P-frame) for a Frame/Slice PDU Set

- Layer ID for Temporal and Spatial Layer PDU Set

- Application or Media-Stream or Sub-Flow ID for Application / Media-stream / Sub-Flow PDU Set

2.  PDU Set Instance Number - a rolling counter that sequentially numbers each PDU Set instance within a PDU Set, independent of PDU Set Type

3.  Dependent PDU Set Instance, which is the PDU Set Instance Number previously assigned to the PDU Set instance on which this PDU Set is dependent.

4.  PDU Set Instance Start / End Indicator for Frame / Slice PDU Sets

2.  QoS framework and policies

-  As per current QoS framework, packet handling is determined according to PCF “policy”.

-  Non-Dynamic PCC is supported via configuration of PCC rules with per-PDU set granularity on the SMF.

-  Dynamic PCC is supported via provisioning by the PCF on the SMF of PCC rules with per-PDU set granularity.

-  The NG-RAN is not aware of media characteristics. It is only aware that a packet belongs to one or more PDU Sets for which the QoS treatment is governed by policy.

-  Per-PDU Set PCC granularity is reflected in QoS Profiles sent to the NG-RAN.

-  Per PDU set QoS is enforced in the NG-RAN by applying policy as reflected in the QoS Profile to PDUs according to the PDU GTP-U header extension markings indicating the PDU’s membership in one or more PDU sets.

-  Per-PDU Set QoS Profiles enable QoS differentiation according all the PDU Sets to which a packet may be associated.

-  Per PDU set QoS parameters may include Priority, PDB, PER and Validity Time.

NOTE: 	PDU Set Validity Time indicates for each PDU set type, the maximum delay for a given PDU Set instance to be delivered to the recipient. Beyond this time, the PDU Set instance is not considered valid, and thus all un-transmitted packets of the PDU Set can be discarded at the RAN. For example, that for an I-Frame PDU set, the Validity Time may take into account the usefulness of I-Frames for subsequent P-Frame decoding, which may occur at a later time than is required to render the I-Frame.

3.  AF Request

- The AF may (optionally) send a request to support PDU Set identification and differentiated handling

- The AF request may contain the following to support PDU Set identification and per PDU Set QoS

1. Flow description of the target media service data flows for per PDU Set handling

2. An RTP/RTPSP header type to be detected (for media with RTP/RTSP headers).

3. Assistance Information describing the media and characteristics including

a. The number of Temporal and Spatial media layers

b. PDU Set periodicities (e.g. for Temporal and Spatial Layers)

c. PDU Set dependencies (e.g. the existence of Referenced Frames or Slices such as a P-Frame with forward references).

4. Requested Priority, PDB and PER per-PDU Set type.

5. Validity Time per-PDU Set type

### 6.x.3	Procedures

PDU Set integrated packet handling is described in the procedure shown in Figure 6.x.3-1.

0.	The UE establishes a PDU Session as defined in clause 4.3.2.2.1 of TS 23.502 [3] for a given DNN/S-NSSAI.

1.	The AF may send the information described in item 3 of clause 6.X.2 to PCF via Nnef_AFsessionWithQoS_Create request as defined in clause 4.15.6.6 of TS 23.502.

2. 	PCF initiates the PDU Session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3]. The PCF generates appropriate PCC rules based on the information from AF or based on provisioned policy. The PCF sends the PCC rules to SMF.

3.	SMF generates the QoS profiles and N4 rules based on the PCC rules from PCF. SMF sends the N4 rules to UPF and sends the QoS profiles to the RAN node via AMF.

4.	Based on received N4 rules or locally configuration on the UPF, the UPF identifies the info in above in item 1 of clause 6.x.2.

5.	UPF marks the PDU Set information in the User plane packets sent to RAN according in the GTP-U header.

6.	Based on the PDU Set in GTP-U header and per-PDU set information contained in the QoS Profile received from the SMF, the RAN performs per-PDU Set based QoS handling.

NOTE: The details of RAN behaviour is up to RAN WG.

### 6.x.4	Impacts on services, entities and interfaces

-	PCF:

-	 Support per-PDU Set policy

-	UPF:

- Support Media Unit Identification and packet marking in GTP-U extension header.

-	SMF:

-	Configure per-PDU Set policy (via QoS Profile)

-	NG-RAN:

-	Support application of Per-PDU Set Policy to packets within a QoS Flow

*** End of changes ***