SA WG2 Meeting #151E	S2-2204653

# 16 – 20 May 2022, e-meeting

Source:	Apple

Title:	KI#4, Sol #26: Update to remove an editor's note

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution:

# 1		Discussion

The following Editor's note is addressed:

"Traffic categories can also be defined directly based on traffic characteristics and the specific QoS attributes could have a relative priority level. As an example, for a certain class of traffic, there could be relative priority among the QoS attributes latency and bandwidth (e.g., low latency and high bandwidth with latency as higher priority). Priority can be given to single or multiple QoS attributes (e.g., prioritize latency and packet error rate over bandwidth for voice traffic, prioritize latency and bandwidth over packet error rate for gaming or video streaming traffic, etc.).

Editor’s note: The different attributes to characterize traffic are for further discussion."

Solution #26 provides two alternatives to define Traffic categories:

- Traffic categories based on service type

In this option, traffic categories are specified for service types, similarly to the solutions defined in Solution #22 or Solution #23. It is assumed that each service type can be mapped to corresponding 5QI value.

- Traffic categories based on traffic characteristics

In this option, traffic categories are directly based on three distinct traffic characteristics: latency, loss and bandwidth, resulting in the following example traffic category values:

Traffic category 1:  Low latency, Normal loss, Low bandwidth traffic category

Traffic category 2:  High latency, Low loss, High bandwidth traffic category

etc.

This option is more appropriate to associate traffic categories to different flows e.g. to meet the requirements set by super-apps that can provide multiple services simultaneously with multiple flows active at any given time.

Additionally, the traffic category values become more "abstract" compared to option 1) helping to alleviate privacy concerns on UE revealing information about user activities. For example, if the user has started an app that creates a traffic flow with a traffic category "Low latency, Normal loss, Low bandwidth", revealing such information about user activities does not violate user privacy. However, if the user has started an app that creates a traffic flow with a traffic category e.g. "Augmented Reality" (per option 1), this would reveal more information about user’s activities and potentially needs user consent.

# 2		Proposal

It is proposed to delete the Editor's note from the solution description:

******************************** First change  *******************************

#### 6.26.1.2	Traffic categories based on Traffic characteristics

Traffic categories can also be defined directly based on traffic characteristics and the specific QoS attributes could have a relative priority level. As an example, for a certain class of traffic, there could be relative priority among the QoS attributes latency and bandwidth (e.g., low latency and high bandwidth with latency as higher priority). Priority can be given to single or multiple QoS attributes (e.g., prioritize latency and packet error rate over bandwidth for voice traffic, prioritize latency and bandwidth over packet error rate for gaming or video streaming traffic, etc.).

The traffic categories could be defined as follows based on different traffic characteristics.

Table 6.26.1.2-1: Traffic categories based on Traffic characteristics of different applications

| Traffic Category |  |  | Service Type | Usage examples |
| --- | --- | --- | --- | --- |
| Latency | Loss | Bandwidth |  |  |
| Low | Normal | Low | Real-time online interaction | Games |
| Normal | Normal | Normal | Default bearer | Best Effort. Default service class; pretty much a mix of everything. |
| High | Low | High | Video Streaming | Youtube, Netflix, Hulu |
| High | Normal | High | AR/VR |  |
| High | Normal | Normal | Background | Bulk data upload |
| High | High | Normal | Background (alt) | "Background", user-initiated, high delay tolerant, high loss tolerant, elastic flow, variable size. E.g., user-initiated iCloud synching or Time Capsule backup; or traffics of background applications, for which there is some progress feedback. |
| Any | Any | Any | Off Peak Data/Cost | Push traffic |

NOTE: The above traffic categories are just illustrative examples, other traffic categories may be added as needed.

******************************** End of change *******************************