3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204770

Elbonia, May 16th –20th, 2022	(revision of S2-2204182r02)

Source:	Samsung

Title:	KI #1, Sol #2: Update for clarifications on the PDU Session with re-mapped S-NSSAI

Document for:	Approval

Agenda Item:		9.14

Work Item / Release: FS_eNS_Ph3 / Rel-18

Abstract: This paper tries to resolve the EN in KI#1, SOL#2

1. Introduction

This paper tries to resolve the EN in KI#1, SOL#2 by providing a procedure on the establishment of the PDU session with re-mapped S-NSSAI.

2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-41.

# * * * * First change * * * *

6.2.3.3	Change of S-NSSAI of a PDU Session

PCF is configured with information that pairs of network slices can be used as an alternative for each other (both slices support the same services with the same DNNs, and same DNAIs pointing to the same DNs, hosting the same services, etc.).

Figure 6.2.3.3-1: Change of S-NSSAI of a PDU Session

1.	The AMF determines that a network slice that requires slice re-mapping occurs.

2.	From AMF to PCF: AM Policy Association ID, S-NSSAI that requires slice re-mapping.

If the AMF has received a PCR trigger that instructs the AMF to interact with PCF when a network slice that requires slice re-mapping occurs and the AMF determined in step 1 that a network slice that requires slice re-mapping occurs, the AMF triggers AM Policy Association modification procedure to PCF.

3.	From PCF to AMF: AM Policy Association ID, selected S-NSSAI.

The PCF selects re-mapped S-NSSAI and provides it to the AMF based on the S-NSSAI that requires slice re-mapping received from AMF in step 2. The re-mapped S-NSSAI is selected within Allowed NSSAI.

The PCF may update URSP rules and triggers UE Configuration Update procedure based on the S-NSSAI that requires slice re-mapping and the selected re-mapped S-NSSAI.

Editor's note:	It is FFS whether and how the change of S-NSSAI of a PDU session applies to roaming scenario.

4.	For each old PDU session with SSC mode 2 or SSC mode 3 and associated with the network slice that requires slice re-mapping, AMF may trigger a change of the slice of the old PDU session to the selected re-mapped S-NSSAI received in step 3 according to the procedure shown in Clause 6.2.3.4.

6.2.3.4	AMF-triggered PDU Session Modification to change of S-NSSAI of PDU Session

Figure 6.2.3.4-1: AMF-triggered PDU session modification

- From AMF to SMF: SM Context ID, selected S-NSSAI

The AMF triggers AMF-initiated PDU session modification procedure to change the S-NSSAI of the PDU session with the selected S-NSSAI.

- From SMF to AMF: Result Indication

- Steps 1 to 6 as in Figure 4.3.5.2-1 of Cause 4.3.5.2 of TS 23.502 [5] are performed with the following differences:

- In step 1, if the SMF received the Nsmf_PDUSession_SMContextUpdate request message (SM Context ID, selected S-NSSAI) from the AMF and the selected S-NSSAI is different from the S-NSSAI of the PDU session for the SM Context ID, the SMF decides to change the S-NSSAI of the PDU session to the selected S-NSSAI.

- In step 2, if the SMF invokes the Namf_Communication_N1N2MessageTransfer due to the change of the S-NSSAI, Cause indicates that a PDU Session re-establishment to the same DN is required and the selected S-NSSAI is included in the PDU Session Modification Command.

- In step 4, if the UE receives PDU Session Modification Command that includes the Cause indicating the same DN and the S-NSSAI, the UE may decide to initiate the PDU Session Establishment procedure described in clause 4.3.2.2 in TS 23.502 [5], to the same DN with the new S-NSSAI the following differences:

  - In Step 1 of clause 4.3.2.2.1, the new PDU Session ID is included as PDU Session ID in the NAS request message, and the Old PDU Session ID which indicates the existing PDU Session to be released is also provided to AMF in the NAS request message.

- * * * * Second change * * * *

6.2.4	Impacts on services, entities and interfaces

AMF:

-	Support slice re-mapping

- send a message to SMF to trigger a change of S-NSSAI

SMF:

-	trigger SSC mode operation with a S-NSSAI received from AMF as a new PDU session

NRF:

-	Support discovery of PCF that support slice re-mapping

PCF:

-	Support slice re-mapping

UE:

-  Handle the PDU session modification command message that includes S-NSSAI

# * * * * End of changes * * * *