SA WG2 Meeting #S2-151E	S2-2204790

16 - 20 May, 2022, E-Meeting	(revision of S2-2204064r01)

Source: 	NTT DOCOMO

Title: 	New Solution: KI#5: Improved support of RAs including TAs supporting Rejected S-NSSAIs

Document for: 	Agreement

Agenda Item: 	9.14

Work Item / Release:	FS_eNS_ph3 / Rel-18

Abstract of the contribution: This paper proposes a solution to the key issue Key Issue #5: Improved support of RAs including TAs supporting Rejected S-NSSAIs.

Discussion

This paper addresses Key Issue #5: Improved support of RAs including TAs supporting Rejected S-NSSAIs.

The proposed method proposes the "exception" to the existing rejected NSSAI handling.

The proposed method does

- not propose any new type of NSSAI IE;

- not propose any new cause value associated to the rejected NSSAI;

- not cause impact to the basic NSSAI storage concept for rejected NSSAI i.e., UE stores the S-NSSAI in rejected NSSAI IE per cause value

Proposal

It is proposed to capture the following changes into the TR 23.700-41.

* * * First Change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions |  |  |  |  | KI#5 |  |
| X |  |  |  |  | X |  |

* * * Next Change * * * *

6.X	Solution #X: Exception to the rejected NSSAI handling

6.X.1	Introduction

This solution applies to KI#5 i.e., how to allow the UE to initiate a registration for an S-NSSAI which was rejected for the RA when the UE enters a TA that is part of the RA and the TA supports this S-NSSAI.

The issue scenario is that initially UE receives a rejected S-NSSAI for the current Registration Area. Then UE enters another TA that is part of the RA and the TA "may" support this S-NSSAI.

6.X.2	Functional Description

This solution outlines are as follows:

The AMF

-	provides the rejected S-NSSAIs for the current RA per current mechanism i.e., no new cause value.

-	associates an indication to each rejected S-SNSAI for an exceptional handling that UE can use the S-NSSAI even if UE stores the S-NSSAI in the rejected NSSAI storage for the current RA when the UE enters another TA in the current RA and provides the associated indication to the UE.

-	may associate a list of TA (included in the current RA) where the rejected S-NSSAI is supported and provide it to the UE.

The UE

-	stores the received rejected NSSAI per current mechanism.

-	sends a 5GMM capability of support of new slice granularity i.e., per tracking area in Registration Request message.

-	can use the S-NSSAI stored in rejected NSSAI storage if the received exception indication indicates that it is allowed to use the S-NSSAI when the UE enters another TA in the current RA.

-	shall not use the S-NSSAI stored in rejected NSSAI storage if the received indication indicates that it is not allowed to use the S-NSSAI when the UE enters another TA in the current RA.

6.X.3	Procedures

Figure 6.x.3-1: Exceptional handling of Rejected NSSAI

1.	The UE initiates Registration procedure. The Registration Request message includes a 5GMM capability of support of new slice granularity i.e., per tracking area.

2.	The AMF continues with Registration procedure.

3.	The AMF sends Rejected S-NSSAI(s) with the cause value "S-NSSAI not available in the current registration area" and an associated exception indication. The exception indication indicates the exception to the Rejected S-NSSAI handling. If operator and implementation policy require, the AMF may also provide a list of TA (included in the current RA) where the rejected S-NSSAI(s) is supported and associated the list of TA to each rejected S-NSSAI.

4.	The UE continues with Registration procedure.

When the UE moves to another TA in current RA where the UE received the Rejected S-NSSAI and the cause value indicates "S-NSSAI not available in the current registration area", if the exception indication indicates that the Rejected S-NSSAI is TA granularity so that it is allowed to use the Rejected S-NSSAI in anther TA, the UE is allowed to use the Rejected S-NSSAI in the Registration procedure. If the previously Rejected S-NSSAI is included in the Allowed NSSAI, UE can request the service for the S-NSSAI. If the AMF provides a list of TA associated with the rejected S-NSSAI(s) to the UE, the UE will send Registration Request for the rejected S-NSSAI when the UE moves into a new TA that is in the list of TA.

6.X.4	Impacts on services, entities and interfaces

The following impacts have been identified:

- UE:

-	sending a 5GMM capability of support of new slice granularity i.e., per tracking area in Registration Request message

-	performing the exceptional handling of Rejected S-NSSAI

- AMF:

-	providing an exception indication indicates that the Rejected S-NSSAI is TA granularity so that it is allowed to use the Rejected S-NSSAI in anther TA

-	providing a list of TA associated to the rejected S-NSSAI