3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204792

Elbonia, 16th – 20th May, 2022	(revision of S2-2204526r03)

Source:	ZTE

Title:	FS_eNS_Ph3 KI#5 New solution Rejected S-NSSAI with new cause value

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This paper propose a new solution for KI#5

1. Introduction/Discussion

This paper propose a new solution for KI#5.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41 V0.2.0.

# * * * * First change * * * *

6.14	Solution #x: Rejected S-NSSAI with new cause value

6.14.1	Introduction

This solution aims to address the KI#5: Improved support of RAs including TAs supporting Rejected S-NSSAIs. In particular, this solution proposes new cause value for the Rejected NSSAI so the UE can differentiate the rejection cause and initiate a registration for an S-NSSAI when the UE enters a TA that is part of the RA and the TA supports this S-NSSAI.

6.14.2	Functional Description

During UE registration, the UE sends Requested NSSAI in RRC message to the RAN. The RAN selects the AMF based on the Requested NSSAI. The NSSF may further reselect a different AMF. The selected AMF may not support all S-NSSAIs in the Requested NSSAI. In previous releases, the AMF rejects the unsupported S-NSSAI with cause value “rejected for the RA” and the UE shall not request this S-NSSAI again before the UE moves outside of the RA. This solution introduces a new cause value “rejected due to AMF not support”, so the UE can still request the rejected S-NSSAI in UE registration procedure without moving outside of the RA, and the RAN may select a new AMF, which supports this S-NSSAI.UE may provide new UE capability indication for this feature to AMF

NOTE:	If the AMF detects that the UE continuously triggers registration for the S-NSSAI and the AMF rejects it with cause value “rejected due to AMF not support”, the AMF may use different cause value “rejected due to AMF not support” for the rejected S-NSSAI so the UE will not try again before it moves outside of the RA.

6.14.3	Procedure

During UE registration procedure, the AMF provides new cause values for the rejected S-NSSAI in the Registration Accept/Reject message to UE

6.14.4	Impacts on services, entities and interfaces

AMF:

-	providing one new cause value “rejected due to AMF not support” for the rejected S-NSSAI

UE:

-	send new UE capability indication to AMF

-	support handling of new cause value

# * * * * End of changes * * * *