3GPP SA WG2 Meeting #151E (e-meeting)	S2-2204822

16 - 20 May, 2022, Elbonia

Source: 	Ericsson, LG Electronics

Title: 	KI#1, Sol #5: Update to clarify the roaming procedure

Document for: 	Approval

Agenda Item: 	9.15

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: This paper proposes some updates to solution#5 for KI#1 for URSP rule provisioning and updating procedures in roaming.

# 1 Introduction

This paper proposes some updates to solution#5 for KI#1 in TR 23.700-85 to study URSP rule provisioning and updating procedures in roaming scenario. It can also be a new solution instead.

The proposed updates maintains the proposal that the AF provides application guidance for URSP determination to all PLMNs whose subscribers will be probably roaming into the VPLMN, and the AF has roaming agreements with. Alternatively, the AF provides application guidance for URSP determination to the PLMN where the user is currently registered.

This proposes updates are:

-  Reuse the existing Npcf_UEPolicyControl Service for the H-PCF to subscribe to reception of new/updated Service Parameters for the UE, using a new PCRT and for the V-PCF to notify to the H-PCF and provide the Service Parameters for the UE to the H-PCF.

-	The H-PCF generates a new URSP Rule or updates an existing URSP Rule with the same traffic descriptor as provided in the Service Parameters, if the S-NSSAI subscription information indicates that this DNN, S-NSSAI is subscribed, and LBO roaming is allowed.

-	removal of the EN:

Editor's note:	It is FFS how the H-PCF provides the URSP rules of the HPLMN (i.e. the original one) to the UE when the UE comes back and registers to the HPLMN.

The H-PCF subscribes to “change of PLMN” to the V-PCF, then when the UE moves to a different PLMN, the H-PCF removes or updates the URSP Rule sent to the UE. When the H-PCF updates the URSP Rule is to remove the RSD that routed the traffic thought the VPLMN were the UE was registered.

# 2 Proposal

It is proposed to include the following change in TR 23.700-85.

*** START OF CHANGES ***

## 6.5	Solution #5: URSP determination based on service parameter obtained from VPLMN

### 6.5.1	Description

This is a solution for key issue #1 "URSP in VPLMN".

Currently, URSP is only controlled and determined by HPLMN. Therefore, to give influence on URSP determination, e.g. application guidance for URSP determination specified in clause 4.15.6.10 of TS 23.502 [3], the AF needs to provide inputs to HPLMN. The application guidance for URSP determination may be for Edge services or for location-based services which means there may be the case that the URSP rules determined based on the application guidance can be applied and valid only when the UEs are in any specific VPLMN in roaming case. Anyhow, in this case, the AF may need to provide application guidance for URSP determination to all PLMNs whose subscribers will be probably roaming into the VPLMN, and the AF has roaming agreements with. In addition, if the AF includes a geographical area as spatial validity condition in the application guidance for URSP determination, the NEF may need to transform this information into 3GPP identifiers (e.g. TAI(s)). For this, the NEF needs to have knowledge on VPLMN's 3GPP identifiers (e.g. TAI(s)).

In this solution, the AF provides application guidance for URSP determination to the PLMN (i.e. to UDR via NEF of this PLMN) where the related services are provided or the related traffic is routed. H-PCF of the roaming UE obtains the application guidance for URSP determination from VPLMN by requesting V-PCF to subscribe to V-UDR to be notified about the data creation/modification for the UE. V-PCF subscribes to V-UDR on behalf of H-PCF, by adding a new PCRT that request notification when Service Parameters are available for a UE Policy Association. If the AF provides inputs to V-UDR via V-NEF, V-PCF provides the data notified by V-UDR to H-PCF.

This solution reuses the existing Npcf_UEPolicyControl Service for the H-PCF to subscribe to reception of new/updated Service Parameters for the UE, using a new PCRT and for the V-PCF to notify to the H-PCF and provide the Service Parameters for the UE to the H-PCF.

The H-PCF generates a new URSP Rule or updates an existing URSP Rule with the same traffic descriptor as provided in the Service Parameters, if the S-NSSAI subscription information indicates that this DNN, S-NSSAI is subscribed, and LBO roaming is allowed.

The H-PCF sends the URSP Rule to the UE using existing procedures. In addition, the H-PCF subscribes to “change of PLMN”, then when the UE moves to a different PLMN, the H-PCF removes or updates the URSP Rule sent to the UE to remove the RSD that routed the traffic thought the VPLMN were the UE was registered. The H-PCF also removes or updates the URSP Rule sent to remove the RSD that routed the traffic though the VPLMN when the V-PCF requests to terminate the UE Policy Association.

### 6.5.2	Procedures

#### 6.5.2.1	Procedure for URSP determination based on service parameter obtained from VPLMN

Figure 6.5.2.1-1 shows the procedure for URSP determination based on service parameter obtained from VPLMN.

Figure 6.5.2.1-1: Procedure for URSP determination based on service parameter obtained from VPLMN

1.	The UE is registered to 5GC and the UE Policy Association is performed between the AMF and the H-PCF via V-PCF. In addition, the H-PCF may subscribe using a new PCRT “Service Parameters received” to the V-PCF. The V-PCF provides the Service Parameters if available.

2.	Based on the reception of a new PCRT from the H-PCF, the V-PCF subscribes to the reception of Service Parameters from the AF for the determination of URSP Rules for any UE registered in this PLMN, this is done based on the agreements between the AF and the VPLMN. The V-PCF includes VPLMN DNN and S-NSSAI in the subscription request to the UDR given that the AF provides Application guidance for URSP determination to the VPLMN.

3.	To provide Application guidance for URSP determination as specified in clause 4.15.6.10 of TS 23.502 [3], the AF creates a new request.

4-6	As specified in steps 2 to 4 in clause 4.15.6.7 of TS 23.502 [3], the AF sends its request to the NEF, and the NEF stores the AF request information in the UDR and responds to the AF. In this figure, the NEF is V-NEF and the UDR is V-UDR.

The AF request is targeting any UE (all UEs) registered in this VPLMN. Therefore, the V-NEF authorizes the request based on local policy based on roaming agreements between the AF and the VPLMN (e.g. based on AF Id) without requesting for any service specific authorization from UDM as specified in clause 4.15.6.10 of TS 23.502 [3].

NOTE 1:	The AF request targeting a specific UE or a group of UE(s) does not apply to this solution because Service Specific Authorization for an individual UE or group of UEs by NEF to UDM has to be performed for the AF request as specified in clause 4.15.6.10 of TS 23.502 [3].

7.	The V-PCF receives a Nudr_DM_Notify notification of data change from the V-UDR.

8.	The V-PCF sends the invoking  Npcf_UEPolicyControl_Update including the Service Parameters that have been received from the UDR to the H-PCF. Before sending the Npcf_UEPolicyControl_Update, the V-PCF replaces the DNN and S-NSSAI in the Service Parameters with the DNN and S-NSSAI of the HPLMN.

9.	The H-PCF determines the URSP rules that is to be sent to the UE based on the Service Parameters received from the V-PCF and the S-NSSAI subscription information for this SUPI, DNN, S-NSSAI retrieved from the UE context policy control subscription information from the UDR  The H-PCF generates a new URSP Rule or update an existing URSP Rule that was sent to the UE for the same traffic descriptor to include a new RSD with the DNN, S-NSSAI provided by the V-PCF then sets the RSD precedence and validity conditions taking the information provided by the V-PCF into account.

That is, the H-PCF replaces any URSP rules related to Route Selection Descriptor(s) including the DNN and S-NSSAI.

10.	The H-PCF provisions the URSP rules to the UE via the V-PCF by using the UE Configuration Update procedure for transparent UE Policy delivery as specified in clause 4.2.4.3 of TS 23.502 [3].

11	The UE needs to send traffic, e.g. destined to an Edge service. Therefore, the UE determines a URSP rule applicable for the traffic.

12.	If there is no PDU Session that matches all components in the selected RSD within the URSP rule, the UE establishes a PDU Session.

13.	The UE communicates with the Application Server, e.g. Edge Application Server.

### 6.5.3	Impacts on services, entities and interfaces

H-PCF:

-	Subscribes to a new PCRT “Service Parameters received”.

-	Reception of Service Parameters for a UE Policy Control Association established with the V-PCF.

-	Determines URSP rules based on the data provided by AF and received via V-UDR and V-PCF, taking S-NSSAI subscription information into account.

V-PCF:

-	Subscribes to V-UDR to be notified about the Service Parameters creation/modification for roaming UEs if the H-PCF provided a PCRT.

-	Provides the Service Parameters notified by V-UDR to H-PCF.

H-UDR:

-	S-NSSAI Subscription information needs to include an indication of whether LBO roaming is allowed for a DNN, S-NSSAI.

*** END CHANGES ***