3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204973

Elbonia, May 16th – 20th, 2022	(revision of S2-2204443)

Source:	China Mobile, China Southern Power Grid

Title:	Solution to enable SMF awareness of Multiple SMF management

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing a solution to Key Issue#4 for FS_GMEC on enhancements of 5G VN group communication.

1. Introduction/Discussion

There is a restriction on Rel-16 5G VN group communication session management, "A dedicated SMF is responsible for all the PDU Sessions for communication of a certain 5G VN group". FS_GMEC in Rel-18 work aims at supporting the VN group communication by multiple SMFs.

Observation1: In order to support multiple SMFs for VN group communication, the SMF may interact with NRF(introduced in solution #4) or GSMF(Group Session Management Function defined in solution #5) . But before the interaction, whether and how does the SMF aware of multiple SMFs shall be involved in serving a certain VN group?

Observation2: It seems not all scenarios require multiple SMFs networking for 5G VN group. Different 5G VN group requires different area size. For example, the Ethernet based 5G LAN-type service might be required by the industry field where service coverage is limited to a fixed and small area. In such case, single SMF is enough to provide the required 5G LAN-type service. The IP based 5G LAN-type service might be required by large enterprises which may located in different workstation. In such case, multiple SMFs are required to support the required 5G LAN-type service.

Observation3: For VN groups that do not need to be served by multiple SMFs, the procedures addressed by KI#4 may not be necessary on network side. From deployment perspective, If the 5G LAN-type service is required within a small area that can be covered by a single SMF, then there is no need to involve the network function like GSMF(Group Session Management Function defined in solution #5). It will simplify the deployment and reduce the unnecessary signaling procedure. For example, the signaling procedure between SMF and NRF(introduced in solution #4) or GSMF(Group Session Management Function defined in solution #5).

So, it might be benefit to distinguish the different 5G LAN-type service requirement which can be served by single SMF or multiple SMFs.

So, this pCR is intend to introduce a solution to enable the SMF be aware of multiple SMFs shall be involved in serving a certain VN group.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.2.0.

# * * * * First change * * * *

6.x	Solution #x: Solution of Multiple SMFs for VN group communication

6.x.1	Introduction

This solution is to address the Key Issue #4 on enhancements of 5G VN group communication. It introduces a new indicator in 5G VN group data which is aiming to inform the SMF whether the procedures used for multiple SMFs is applicable for this 5G VN group.

For example, if one dedicated SMF is enough to support a certain 5G VN group, then it’s not necessary for the SMF to continue the procedures with NRF or GSMF. And also, if the 5G VN group will use N6 based switch, then it’s also not necessary for the SMF to continue the procedures with NRF or GSMF. So, this solution can be regard as the trigger point for the KI#4 and can be collaborate with other solutions on the same KI topic.

6.x.2	Functional Description

The main idea of this solution is as below:

-	Enhance the parameter provisioning service by introducing the Single-SMF indicator information as a new optional input parameter, and this Single-SMF indicator is stored as part of the group data in group subscription data.

-	SMF can retrieve such indicator from UDM or be notified by UDM via the group subscription data.

-	In case the indicator exist, the SMF can be informed that this 5G VN group does not require multiple SMF to serve and will not continue the other procedures which may lead to additional interaction with other nodes. For example registration or updating its profile to NRF(Solution #4) or Group Session Management Function(Solution #5).

-	In case the indicator doesn’t exist, the SMF can continue the procedure which will be addressed by other solutions for KI#4.

Editor’s Note: it is FFS whether the existing group data information can serve as the Single-SMF indicator.

6.x.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

6.x.4	Impacts on existing entities and interfaces

Editor's Note: This clause lists impacts to existing entities and interfaces.

# * * * * End of changes * * * *