SA WG2 Meeting #S2-151E	S2-2203642

16 - 20 May, 2022, Electronic meeting

3GPP TSG-CT3 Meeting #121e	C3-222419

# E-Meeting, 6th – 12th April 2022

# Title:	LS on handling of FQDN ranges in AF influence on traffic routing procedures

# Response to:	-

# Release:	Release 17

# Work Item:	eEDGE_5GC

Source:	CT3

To:	SA2

Cc:	-

Contact Person:

Name:	Susana Fernández

E-mail Address:	susana.fernandez@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

CT3 is discussing the support of FQDN ranges as defined in TS 23.501 for AF influence on traffic routing procedures.

According to that TS the FQDN range is considered one example of traffic filtering information and when provided, “it may be used by SMF as one of the triggers for retrieving EAS Deployment Information from NEF. Retrieval of EAS Deployment Information is defined in TS 23.548 [130]”.

CT3 has the following questions:

Question 1: Is the FQDN range only used as a trigger to initiate the retrieval of EAS Deployment Information from NEF or is it also used to identify the traffic for influencing SMF routing decisions?

Question 2: If it is used to identify the traffic for influencing SMF routing decisions, can SA2 clarify how it works? Is the FQDN range(s) used to identify the sessions/users for which the EDIs apply, and if yes, how is this applicability exactly determined?

Question 3: If it is only used as a trigger for the retrieval of EAS Deployment Information, why an FQDN range is required? Could a specific indicator or the application identifier included to identify the traffic satisfy the same purpose?

2. Actions:

To SA2 group.

ACTION: 	CT3 kindly requests SA2 to answer the questions above and update their Specifications if considered necessary.

3. Date of Next CT3 Meetings:

3GPP TSG CT3#122e			05/2022	E-Meeting

3GPP TSG CT3#122-bise		06/2022	E-Meeting (TBD)