3GPP SA WG2 Meeting #151E	S2-2203906

16-20 May 2022, Electronic Meeting

Source:	Ericsson

Title:	KI#4 & KI#5, Sol #26: Update with Editor’s notes for aspects that are FFS

Document for:	Agreement

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This paper adds some Editor’s notes for the key issue “PDU Set integrated packet handling” for inclusion in TR 23.700-60.

# Discussion

The FS_XRM SID has a KI “PDU Set integrated packet handling” as agreed in S2-2201802. As per the KI description,

This key issue proposes to study PDU Set integrated packet handling in 5G network, in which the group of packets belongs to a same PDU Set will be handled in an integrated manner.

Based on the related solution #26, the following changes are proposed for TR 23.700-60.

# Proposal

It is proposed to add the following changes to TR 23.700-60.

*** Start of changes ***

### 6.26.3	Procedures

#### 6.26.3.1	PDU session modification

Figure 6.26.3.1-1: PDU session modification procedure

0.	AF sends AF request with flow description and the associated XR traffic configuration information, where the XR traffic configuration information includes time assistance information (e.g. burst arrival time, periodicity), PDU set based QoS requirement (e.g. PDU set delay budget, PDU set error rate) and frame type/importance.

1.	PCF initiates SM Policy Association Modification procedure as defined in clause 4.16.5.2 of TS 23.502 [3] to notify SMF about the modification of policies, which further triggers PDU session modification procedure. PCF includes service data flow filter with the associated XR traffic configuration information in the PCC rules provided to SMF.

2a.	SMF sends N4 Session Modification Request to UPF, which contains the mapping of packet filter set and the associated QFI, frame type/importance and time assistance information.

2b.	UPF reponds SMF with N4 Session Modification Response.

3.	SMF invokes Namf_Communication_N1N2MessageTransfer to transfer N2 SM information.

4.	AMF transfers N2 SM information to RAN node, which contains the mapping of QFI, frame type/importance, time assistance information and PDU set based QoS requirement.

5-12	PDU session Modification procedure are performed as in clause 4.3.3.2 of TS 23.502 [3].

Editor's note: Whether and how UE maps uplink packets to QoS Flows is FFS.

### 6.26.4	Impacts on services, entities and interfaces

The proposed solution is based on Rel-17 procedures but some enhancements may be needed to make it possible for the 5GS to configure UPF/RAN node with the XR traffic configuration information, such as:

AF:

-	Include XR traffic configuration information associated with flow description in AF request.

PCF:

-	Include XR traffic configuration information associated with service data flow filter in PCC rules.

SMF:

-	Include XR traffic configuration information associated with packet filter set in N4 rules for UPF.

-	Include XR traffic configuration information associated with QFI in N2 SM information for RAN node.

Editor's note: UPF and UE impacts to be added.

*** End of changes ***