3GPP TSG-SA WG2#151E	S2-2204052

Elbonia, 16 - 20 May 2022	(was S2-220xxxx)

Source:	CATT

Title:	KI #1, Sol #12: Update to combine sol#10 and #11

Document for:	Discussion/Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2/Rel-18

Abstract of the contribution: This contribution proposes some update for solution#12.

- Introduction

1) Resolve EN of solution#12

There is one EN unresolved in solution#12

Editor's note:	Other triggers for removal of these parameters are FFS.

In order to resolve this EN, it is proposed to introduce a timer for the buffered parameters, and when the timer expired the buffered parameters will be removed.

2) Combine solution#10 & #11 with solution#12

During the SA2#150E discussion, the solution#10&#11 were also agreed to be captured in TR 23.700-33. The main difference between solution#10 and solution#12 is whether the Target UE information is stored by UE-to-UE Relay during discovery procedure and used for subsequent communication establishment procedure. The main difference between solution#11 and solultion#12 is whether DCA message to Source UE is sent before or after communication is established with the Target UE.

There was also some initial discussion these solutions can be combined together to accommodate more cases. In this paper, the solution#12 is updated to combine solution#10&#11 together.

- Proposal

It is proposed to capture the following changes into TR 23.700-33.

>>>>Start Changes <<<<

6.12	Solution #12: Layer-3 UE-to-UE Relay discovery and communication

6.12.1	Description

The solution applies to Key Issue #1 "Support of UE-to-UE Relay".

In this solution, the Layer-3 UE-to-UE Relay operation is supported with the following principles:

-	Authorization and configuration:

-	Only the UE authorized by the service authorization configuration can act as a UE-to-UE Relay. These UEs will be configured according to the service authorization and provisioning solutions for Key Issue #6.

-	UE-to-UE Relay discovery:

-	Model A and Model B direct discovery as defined in clause 6.3.2 of TS 23.304 [3] are supported for UE-to-UE Relay discovery.

-	Relay Service Code (RSC) is used to indicate the connectivity service that UE-to-UE Relay can support.

-	In order to serve as a UE-to-UE Relay, it is required to discover other UEs in proximity in advance or on demand.

-	The Source and Destination Layer-2 IDs used for discovery message are determined based on the principles as defined in clause 5.8 of TS 23.304 [3].

-	UE-to-UE Relay communication:

-	Source UE initiates the Layer-2 link establishment procedure as defined in clause 6.4.3.1 of TS 23.304 [3] with UE-to-UE Relay, and the QoS Info and IP address are negotiated during this procedure.

-	The UE-to-UE Relay initiates the Layer-2 link establishment procedure as defined in clause 6.4.3.1 of TS 23.304 [3] with Target UE, and the QoS Info and IP address are negotiated during this procedure.

-	IP, Ethernet and Unstructured traffic type are supported.

-	Both UE-to-UE Relay allocated IP address and Link-Local IPv6 address are supported.

-	QoS handling:

-	When the Source UE establishes the unicast Layer-2 link with the UE-to-UE Relay, it can establish corresponding PC5 QoS Flows according to procedure defined in clause 6.4.3.1 of TS 23.304 [3]. The UE-to-UE Relay determines the PC5 QoS parameters used on the PC5 QoS Flow between Source UE and UE-to-UE Relay, as well as the PC5 QoS parameters used on the PC5 QoS Flow between UE-to-UE Relay and Target UE.

-	The PC5 QoS Flows are also set up between UE-to-UE Relay and Target UE using the PC5 QoS parameters determined by UE-to-UE Relay.

The Control Plane and User Plane protocol stacks of Layer-3 UE-to-UE Relay are as followings.

Figure 6.12.1-1: Control plane for Layer-3 UE-to-UE Relay

Figure 6.12.1-2: User plane for Layer-3 UE-to-UE Relay

6.12.2	Procedures

6.12.2.1	Procedures for UE-to-UE Relay discovery

6.12.2.1.1	Procedure for UE-to-UE Relay discovery with Model A

Depicted in figure 6.12.2.1.1-1 is the procedure for UE-to-UE Relay discovery with Model A.

Figure 6.12.2.1.1-1: UE-to-UE Relay discovery with Model A

1.	The UE-to-UE Relay has discovered other UEs in proximity via the previous direct discovery or direct communication procedures. The UE-to-UE Relay obtains the User Info ID, Relay Service Code (RSC) and Layer-2 ID of other UEs in proximity. The UE-to-UE Relay may buffer these parameters for subsequent UE-to-UE Relay communication as described in clause 6.12.2.2. These parameters will be removed by UE-to-UE Relay in case the buffer timer expired.

2.	The UE-to-UE Relay sends an Announcement message. The Announcement message may include the Type of Discovery Message, User Info ID of the UE-to-UE Relay, RSC, and a list of User Info ID of the Target UEs (UEs discovered in step 1). The Layer-2 ID of Target UE may be also included in the Announcement message if it is not buffered by the UE-to-UE Relay, as described in step 2 of clause 6.10.2.1.

The Source Layer-2 ID of the Announcement message is self-assigned by the UE-to-UE Relay, and the Destination Layer-2 ID is selected based on the ProSe policy.

6.12.2.1.2	Procedure for UE-to-UE Relay discovery with Model B

Depicted in figure 6.12.2.1.2-1 is the procedure for UE-to-UE Relay discovery with Model B.

Figure 6.12.2.1.2-1: UE-to-UE Relay discovery with Model B

1.	The UE-to-UE Relay may have discovered other UEs in proximity via the previous direct discovery or direct communication procedures. The UE-to-UE Relay obtains the User Info ID, Relay Service Code (RSC) and Layer-2 ID of other UEs in proximity.

2.	The Source UE sends a Solicitation message. The Solicitation message may include the Type of Discovery Message, User Info ID of Source UE, RSC, and User Info ID of Target UE.

The Source Layer-2 ID of the Announcement message is self-assigned by the Source UE, and the Destination Layer-2 ID is selected based on the ProSe policy.

3.	The UE-to-UE Relay may perform direct discovery procedure to discover the Target UE if it has not been discovered in step 1, e.g. the UE-to-UE Relay may send the Solicitation message and receive the Response message as described in steps 2-3 of clause 6.10.2.2. The UE-to-UE Relay obtains the User Info ID, Relay Service Code (RSC) and Layer-2 ID of Target UEs in proximity. The UE-to-UE Relay may buffer these parameters for subsequent UE-to-UE Relay communication as described in clause 6.12.2.2. These parameters will be removed by UE-to-UE Relay in case the buffer timer expired.

4.	The UE-to-UE Relay that match the values of the User Info ID of Target UE and RSC contained in the Solicitation message respond to the Source UE with a Response message. The Response message contains the Type of Discovery Message, User Info ID of UE-to-UE Relay, RSC, and User Info ID of Target UE. The Layer-2 ID of Target UE may be also included in the Response message if it is not buffered by the UE-to-UE Relay, as described in step 4 of clause 6.10.2.2.

The Source Layer-2 ID of the Response message is self-assigned by the UE-to-UE Relay, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Solicitation message.

6.12.2.2	Procedures for UE-to-UE Relay communication

Depicted in figure 6.12.2.2-1 is the procedure for Layer-3 UE-to-UE Relay communication.

Figure 6.12.2.2-1: Layer-3 UE-to-UE Relay communication

1.	Service authorization and provisioning are performed for Source UE, Target UE and UE-to-UE Relay as described for solutions for Key Issue #6.

2.	The Source UE performs discovery of a UE-to-UE Relay as described in clause 6.12.2.1.

3.	The Source UE sends a Direct Communication Request message to initiate the unicast Layer-2 link establishment procedure with the UE-to-UE Relay. The Direct Communication Request message includes User Info ID of Source UE, User Info ID of UE-to-UE Relay, User Info ID of Target UE and Relay Service Code (RSC). The Layer-2 ID of Target UE may be also included in the Direct Communication Request message if it is obtained by Source UE during the discovery procedure, as described in step 1a of clause 6.11.2.

The Source Layer-2 ID of the DCR message is self-assigned by the Source UE, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the discovery message of UE-to-UE Relay.

4.	If the User Info ID of Target UE, User Info ID of UE-to-UE Relay and RSC are included in the Direct Communication Request, the UE-to-UE Relay responds by establishing the security with Source UE. When the security protection is enabled, the Source UE sends IP Address Configuration or Link-Local IPv6 address (if IP communication is used), QoS Info (PFI and PC5 QoS parameters).

The Source Layer-2 ID used for the security establishment procedure is self-assigned by the UE-to-UE Relay, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the security establishment procedure messages, the Source UE obtains the UE-to-UE Relay's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

5.	The UE-to-UE Relay sends a Direct Communication Accept message to the Source UE that has successfully established security with UE-to-UE Relay. The Direct Communication Accept message includes User Info ID of Source UE, User Info ID of UE-to-UE Relay, User Info ID of Target UE, QoS Info (PFI and split PC5 QoS parameters), RSC, IP Address Configuration or Link-Local IPv6 address (if IP communication is used).

The UE-to-UE Relay may also send a Direct Communication Accept message to the Source UE after the communication is established between UE-to-UE Relay and Target UE, as described in step 8a of clause 6.11.2.

6.	Triggered by step 3, the UE-to-UE Relay sends a Direct Communication Request message to initiate the unicast Layer-2 link establishment procedure with the Target UE. The Direct Communication Request message includes User Info ID of Source UE, User Info ID of UE-to-UE Relay, User Info ID of Target UE and Relay Service Code (RSC).

The Source Layer-2 ID of the DCR message is self-assigned by the UE-to-UE Relay, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the discovery message of Target UE. The Destination Layer-2 ID may also be set to the Layer-2 ID of Target UE received in step 3 if the Layer-2 ID of Target UE is not buffered in the UE-to-UE Relay during the discovery procedure.

7.	If the User Info ID of Target UE and RSC are included in the Direct Communication Request, the Target UE responds by establishing the security with UE-to-UE Relay. When the security protection is enabled, the UE-to-UE Relay sends IP Address Configuration or Link-Local IPv6 address (if IP communication is used), QoS Info (PFI and split PC5 QoS parameters).

The Source Layer-2 ID used for the security establishment procedure is self-assigned by the Target UE, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the security establishment procedure messages, the UE-to-UE Relay obtains the Target UE's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

8.	The Target UE sends a Direct Communication Accept message to the UE-to-UE Relay that has successfully established security with Target UE. The Direct Communication Accept message includes User Info ID of Source UE, User Info ID of UE-to-UE Relay, User Info ID of Target UE, QoS Info (PFI and split PC5 QoS parameters), RSC, IP Address Configuration or Link-Local IPv6 address (if IP communication is used).

The UE-to-UE Relay may further update the PC5 link with the Source UE based on the outcome of PC5 link establishment with the Target UE.

9.	For IP communication, the UE-to-UE Relay stores an association of User Info ID and the IP address of Target UE into its DNS entries, and the UE-to-UE Relay acts as a DNS server to other UEs. The Source UE sends a DNS query to the UE-to-UE Relay to request IP address of Target UE after step 8, and the UE-to-UE Relay returns the IP address of the Target UE to the Source UE.

The Source UE may also obtain the IP address of Target UE via Direct Communication Accept message, as described in step 8a of clause 6.11.2.

NOTE:	The Source UE may send DNS query multiple times to obtain the IP address of Target UE.

10.	The Source UE communicates with the Target UE via the UE-to-UE Relay. For IP communication, the UE-to-UE Relay acts as an IP router. For Unstructured communication, the UE-to-UE Relay performs traffic relaying based on the mapping between the two PC5 Layer-2 links and PC5 QoS Flows. For Ethernet communication, the UE-to-UE Relay acts as an Ethernet switch.

In the case of one Source UE communicates with multiple Target UEs, the PC5 link between Source UE and UE-to-UE Relay can be shared for multiple Target UEs while the PC5 link is established individually between UE-to-UE Relay and Target UEs. For the shared PC5 link, the Layer-2 link modification procedure can be used.

In the case of multiple Source UEs communicate with one Target UE, the PC5 link is established individually between Source UEs and UE-to-UE Relay while the PC5 link between UE-to-UE Relay and Target UE can be shared. For the shared PC5 link, the Layer-2 link modification procedure can be used.

6.12.3	Impacts on services, entities and interfaces

UE:

-	New functionality related to UE-to-UE Relay discovery, as well as communication via UE-to-UE Relay.

>>>>End Changes<<<<