SA WG2 Meeting #151e	S2-2204658

# May 16th – 20th, 2022; Elbonia                  	       (revision of S2-22xxxxx)

Source: 	Philips International B.V.

Title: 	KI#2 clarifications on multiple UEs

Document for: 	Approval

Agenda Item: 	9.20

Work Item / Release:	FS_Ranging / Rel-18

Abstract of the contribution: This contribution proposes to add some clarifications on multiple UEs being involved in ranging..

# 1	Discussion

Currently KI#2 mentions the use of an Assistant UE to assist in ranging between two devices. However, there are also other cases to be considered when making use of multiple devices for ranging, such using multiple anchor UEs to improve accuracy of position estimation.

This pCR proposes to update KI#2 to address some of the other cases involving multiple UEs as well.

# 2 Proposal

It is proposed to update TR 23.700-86 by updating Key Issue #2 as follows.

************************** START OF CHANGE ***********************

## 5.2	Key Issue #2: Ranging service operation procedure with the assistance of another UE

### 5.2.1	General description

The ranging service operation between two UEs may need the assistance of  one or more other UEs (e.g., assistant UE) if the direct ranging/sidelink positioning between a reference UE and a target UE cannot be supported, or if the ranging/sidelink positioning is not sufficiently accurate. The direct communication between a reference UE and an assistant UE or another reference UE, and between a target UE and an assistant UE or another reference UE are respectively supported, so that the assistant UE or other reference UE can help on measurement and data transmission.

This key issue addresses how to enhance the 5GS to support ranging service operation procedures with the assistance of other UEs (e.g., assistant UE, additional reference UEs).

At least the following aspects need to be considered in potential solutions:

-	How to discover and determine an assistant UE or additional reference UE for ranging service?

-	What parameters need to be exchanged between different UEs (reference UE, assistant UE, target UE) during ranging service operation procedures?

-	How the reference UE or target UE gets the ranging result with the assistance of an assistant UE or other reference UE during the ranging service operation procedures?

NOTE:	The direct discovery procedure between reference UE and assistant UE, between reference UEs, and between target UE and assistant UE will be developed under the key issue related to ranging direct discovery.

************************** END OF CHANGE ***********************